3GPP TSG-SA2 Meeting #154AHe eMeeting	S2-2300150

# Elbonia, January 16 – 20, 2023													 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0673 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#5 – Planned Data Transfer with QoS Policy (PDTQ) Negotiation and Activation for future PDU session to support Application AI/ML data transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Oracle, Toyota, NTT DoCoMo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to enable 5GC to assist the Application AI/ML group data transfer, this CR defines the PDTQ data transfer mechanism to support the Application AI/ML data transfer. The solution describes the negotiation of the PDTQ policy which includes a list of QoS profiles corresponding to their respective Desired Time Window. The solution includes the ability to negotiate a list of time windows between the AF and the 5GC. The negotiated time windows will have their corresponding QoS profiles and possible a set of prioritized alternative service requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enables and extending the NEF and PCF services to support PDTQ data transfer windows negotiation - Defines PDTQ warning notification procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a 5GC mechanism to support the group data transfer for at the target planned window for the specific Application AI/ML operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.X, 4.16.X.1, 4.16.X.2, 4.16.X.2.1, 4.16.X.2.2, 5.5.9.2, 5.2.6.30.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *  (All New)

### 4.16.X	PDTQ Negotiations for time windows for planned application data transfer

#### 4.16.X.1	General

The intent of this clause is to specify generic service procedures to enable the AF to negotiate viable time window for the planned application data transfer with specific QoS requirements and operational conditions via the support of the NEF.

The PDTQ policies are defined for a specific ASP which include the one or more QoS profiles corresponding to the respective Desired Time Windows, the maximum number of UEs to be supported, the optional Network Area Information and the Request for Notification if the AF accepts that the PDTQ policy can be re-negotiated using the PDTQ warning notification procedure.

The Network Performance analytics or DN Performance analytics for NWDAF as described in TS 23.288 [50] will be subscribed by the PCF to assist its decision to derive the PDTQ policies for the selected time window(s).

One or more negotiated PDTQ policies could be provided by PCF to AF via NEF together with the PDTQ Reference ID.  If the AF receives more than one PDTQ policies from the PCF, the AF will select one of them and inform the PCF about the selected PDTQ policy which will then be stored in the UDR to be applied to the AF session when requested by the AF. Prior to the start of the Desired Time Window for the planned application data transfer, the AF requests the PCF to apply the selected PDTQ policy to the selected target UEs' PDU sessions. The PCF will then determine the appropriate PCC rules according to the negotiated PDTQ policy for the corresponding PDU session.

#### 4.16.X.2	Procedures

##### 4.16.X.2.1	PDTQ Transfer Policies Negotiation procedure for planned application data transfer

This clause describes the PDTQ procedures to negotiate viable time window for the planned application data transfer with specific QoS requirements and operational conditions via the support of the NEF.

Prior to identifying the set of target UEs to support the planned application data transfer during the specified time-window, AF initiates requests to the NEF with the required set of parameters (e.g. QoS parameters or target AOI etc.) to (re)negotiate PDTQ policies that should have been pre-provisioned or negotiated previously and are stored in the UDR.

Figure 4.16.X.2.1-1: PDTQ Transfer Policies Negotiation procedure for planned application data transfer

Prior to the transport of the Application AI/ML data, the AF negotiates with the 5G Core for the PDTQ policies to provide assistance for its application data transfer.  The AF discovers its serving NEF, if it has not done so, using the mechanism as described in clause 6.3.14 of TS 23.501 [2].

1a.  The AF invokes the Nnef_PDTQNegotiation_Create Request (ASP Identifier, Number of UEs, the list of Desired time windows, QoS reference or one or more QoS parameter, and optionally the Network Area Information, GPSI(s), IP Address(es), Request for notification, Alternative Service Requirements).  The Request for notification is an indication that PDTQ warning notification should be sent to the AF.  Note that, only either Network Area Information or GPSI(s)/IP Address (es) information and not both are to be used for the PDTQ negotiation.

NOTE 1:	Based on AF's internal logic (policy), the AF may determine the Minimum Guaranteed QoS requirements based on the UEs who are to be participated in the PDTQ transfer window and the network input data and the group application data transfer trigger conditions.

1b & 1c.  The NEF may authenticate the AF and authorize the PDTQ request from the AF.  If the authentication/authorization of the AF’s request has failed, the NEF will respond to the AF's request through the Nnef_PDTQNegotiation_Create Response with the failure result, and the following steps are skipped.

2.      Based on an AF request, the NEF may translate the information provided by the AF (e.g. QoS reference or geographical information etc.) based on the local policy and invokes the Npcf_PDTQPolicyControl_Create (ASP Identifier, Number of UEs, the list of Desired time windows, QoS reference or one or more QoS parameters, and optionally the alternative Service Requirements, Network Area Information, GPSI(s), IP Addresses(es), Request for notification) with the H-PCF to authorize the creation of the policy regarding the PDTQ. If the PCF was provided with Request for notification, then PCF will send PDTQ warning notification to the AF as specified in clause 4.16.X.2.3 to notify the AF that the network performance in the area of interest goes below the criteria set by the operator as described in clause 6.1.2.7 of TS 23.503 [20].

3.      H-PCF queries the UDR to retrieve all existing PDTQ polices for the corresponding ASP using Nudr_DM_Query (Policy Data, Planned Data Transfer with QoS) service operation.

4.      The UDR provides all the stored PDTQ policies and corresponding related information (e.g. the Number of UEs, the list of Desired time windows, QoS reference or one or more QoS parameters, and optionally the alternative Service Requirements) to the H-PCF.

5. 	Based on information provided by the AF and other available information, the H-PCF quries or/and subscribes to the NWDAF as defined in clause 6.6.4 or clause 6.14.4 of TS 23.288 [50] to request the Network Performance analytics and in case of non-GBR traffic type, the DN Performance analytics can be requested as well.

When requesting the network performance or DN performance analytics, if “any UE” is used, then the AoI information is used to identify the target gNB(s) for the prediction of the availability of the network resource. If the UE information is used, then the prediction of the availability of the network resource is target towards the UE(s)’ serving gNB(s).

6.  	By referring to the outcome of the analytics report as described in clause 6.1.2.7 of TS 23.503 [20], H-PCF determines one or more PDTQ policies.  The PDTQ policies include the QoS parameters' (e.g. PDB, PLR etc.) for each Desired time window and the Network Area Information as requested by the AF.

7.	The PCF sends one or more PDTQ policies to NEF in Npcf_PDTQPolicyControl_Create Response including the PDTQ Reference ID.

8.	The NEF sends a Nnef_PDTQNegotiation_Create response to the AF to provide one or more PDTQ policies together with the PDTQ Reference ID.

9.	If more than one PDTQ policies were provided to the AF, the AF selects one of the PDTQ policies and notifies NEF for the selected PDTQ policy via Nnef_PDTQNegotiated_Update together with the PDTQ Reference ID.   The AF stores the PDTQ Reference ID and the selected PDTQ policy for the future interaction with the PCF. If the NEF received only one PDTQ policy from the PCF, steps 10-11 are not executed and the flow proceeds to step 12. Otherwise, the flow proceeds to step 10.

10-12. The NEF notifies H-PCF for the selected PDTQ policy by the AF.  The H-PCF acknowledges NEF.  The NEF responds to the AF's request through the Nnef_PDTQNegotiation_Update Response.

13.-14.  The H-PCF stores the PDTQ Reference ID together with the new PDTQ policy in the UDR by invoking Nudr_DM_Update (PDTQ Reference id, Policy Data, Planned Data Transfer with QoS). The UDR sends a response to the H-PCF as its acknowledgement.

After the PDTQ policy negotiation, if the AF decides to select an alternative PDTQ policy (e.g. data rate reduction, relaxing delay constraints of planned and event driven traffic etc.), the AF may trigger this procedure to request the corresponding H-PCF via the support of NEF to revise the PDTQ policy. In such a case, in step 1 instead of invoking Nnef_PDTQNegotiation_Create Request the AF will invoke the Nnef_PDTQNegotiation_Update Request together with the PDTQ Reference ID. Consequently, instead of invoking Npcf_PDTQPolicyControl_Create Request the NEF will invoke Npcf_PDTQPolicyControl_Update Request together with the PDTQ Reference ID.

##### 4.16.X.2.2	Procedure for PDTQ warning notification

Figure 4.16.X.2.2-1: The procedure for PDTQ warning notification

1.	The negotiation for PDTQ policy as described in clause 4.16.X.2.1 is completed. In addition, the PCF has subscribed to analytics on "Network Performance" and, if possible “DN Performance” in addition, from NWDAF for the area of interest and time window of a PDTQ policy following the procedure and services described in TS 23.288 [50], including a Reporting Threshold in the Analytics Reporting information. The value for Reporting Threshold is set by the PCF based on operator configuration.

2.	The PCF is notified with the network performance analytics and, if possible DN performance analytics in addition, in the area of interest from the NWDAF when the NWDAF determines that the network performance goes below the threshold as described for the Network Performance analytics in TS 23.288 [50] and, if possible the NWDAF determines that the DN performance goes below the threshold as described for the DN Performance analytics in TS 23.288 [50].

3.	The H-PCF may request from the UDR the stored PDTQ policies using Nudr_DM_Query (Policy Data, PDTQ) service operation.

4.	The UDR provides all the PDTQ Policies together with the relevant information received from the AF (as defined in clause 6.1.2.7 of TS 23.503 [20]) to the H-PCF.

5.	The H-PCF identifies the PDTQ policies affected by the notification received from NWDAF. For each of them, the H-PCF determines the ASP of which the PDTQ traffic will be influenced by the degradation of network performance and which requested the H-PCF to send the notification. The PCF then performs the following steps for each of the determined ASPs, i.e. Steps 6 - 16 can occur multiple times (i.e. once per ASP).

6.	The PCF decides based on operator policies, whether a new list of candidate PDTQ policies can be calculated for the ASP. If the PCF does not find any new candidate PDTQ policy, the previously negotiated PDTQ policy shall be kept, no interaction with that ASP shall occur and the procedure stops for that PDTQ policy.

NOTE 1:	The PDTQ policies of an ASP which did not request to be notified are kept and no interaction with this ASP occurs.

7.	The PCF sets the no longer valid PDTQ policy in the UDR as invalidated by invoking Nudr_DM_Update (PDTQ Reference ID, invalidation flag) service.

NOTE 2:	The PDTQ policies that are applicable for future sessions are checked by the PCF in step 6.

8.	The UDR sends a response to the H-PCF as acknowledgement.

9.	The PCF sends the notification to the NEF by invoking Npcf_PolicyControl_Notify (PDTQ Reference ID, list of candidate PDTQ policies) service operation.

10.	The NEF sends the PDTQ warning notification to the AF by invoking Nnef_PDTQNegotiation_Notify (PDTQ Reference ID, list of candidate PDTQ policies) service operation.

11.	The AF checks the new PDTQ policies included in the candidate list in the PDTQ warning notification.

12.	If the AF selects any of the new PDTQ policies, the steps 9-14 from clause 4.16.7.X.2.1 are executed.

13.	If the AF doesn't select any of the new PDTQ policies, the steps 9-12 from clause 4.16.7.X.2.1 are executed, with the AF indicating that none of the candidate PDTQ policies is acceptable.

14.-15.	If the step 13 is executed, the PCF removes the no longer valid PDTQ policy from UDR for the corresponding PDTQ Reference ID.

NOTE 3:	The PCF can also remove the no longer valid PDTQ policy after an operator configurable time for the case that the AF does not respond.

If there is a new PDTQ policy stored in the UDR or a PDTQ policy was removed from the UDR, the PCFs are notified by the UDR accordingly.

The AF can send a Stop notification by invoking Nnef_PDTQNegotiation_Update service, when the AF requests not to receive the PDTQ warning notification anymore. Then, the NEF invokes Npcf_PDTQPolicyControl_Update service in order to provide this information for the H-PCF.

* * * Next Changes * * *

##### 5.2.5.9.2	Npcf_PDTQPolicyControl_Create service operation

Service operation name: Npcf_PDTQPolicyControl_Create

Description: This service is to create the planned data transfer with QoS requirements policy.

Inputs, Required: ASP identifier, Number of UEs, list of Desired time windows, QoS reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: S-NSSAI, DNN, Network Area Information, GPSI(s), IP Addresses(es), Request for notification, Alternative Service Requirements.

NOTE: Only either Network Area Information or GPSI/IP Address information and not both to be used.

Outputs, Required: One or more planned data transfer with QoS requirements policies, PDTQ Reference ID.

Outputs, Optional: None.

* * * Next Changes * * *

##### 5.2.6.30.2	Nnef_PDTQPolicyNegotiation_Create service operation

Service operation name: Nnef_PDTQPolicyNegotiation_Create

Description: The consumer requests a planned data transfer with QoS requirements policy.

Inputs, Required: ASP Identifier, Number of UEs, list of Desired time windows, QoS reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: Network Area Information, GPSI(s), IP Addresses(es), Request for notification, Alternative Service Requirements.

NOTE: Only either Network Area Information or GPSI/IP Address information and not both to be used.

Outputs, Required: PDTQ Reference ID, one or more planned data transfer with QoS requirements policies.

Output, Optional: None.

* * * End of Changes * * *