3GPP TSG-SA2 Meeting #154AHe	S2-2300152r08

# Elbonia, January 16 – 20, 2023													 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3674 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – NEF services description for assistance to member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Sony, Toyota |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to enable 5GC to assist the Application AI/ML AF to preform candidate member selection, this CR defines a new NEF services for assistance to member selection. The solution describes the member selection assistance functionality is hosted in the NEF. The AF can request the NEF to provide a list of recommended UEs based on new services supported by the NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new Network Capabilities for external exposure. - Describe a new member selection assistance procedure. - Add a new service provided by the NEF and the corresponding service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a 5GC mechanism to support the member selection to assist the Application AI/ML operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.1, 4.15.2, 4.15.X, 5.2.6.1, 5.2.6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 4.15	Network Exposure

### 4.15.1	General

The network capability exposure comprises

-	Exposure of network events externally as well as internally towards core network NFs;

-	Exposure of provisioning capability towards external functions;

-	Exposure of policy and charging capabilities towards external functions;

-	Exposure of core network internal capabilities for analytics.

-	Exposure of analytics to external party.

-	Retrieval of data from external party by NWDAF.

-	Exposure of number of registered UEs and/or established PDU Sessions for a network slice that is subject to Network Slice Admission Control towards core network NFs or external party.

-	Exposure of UE member selection assistance information including list of candidate UEs that meet some filtering criteria provided  by the AF.

When subscribing to event reporting the NF consumer(s) provide:

-	One or multiple Event ID(s). An Event ID identifies the type of event being subscribed to (e.g. PDU Session Release, UE mobility out of an Area of Interest, etc.).

-	Event Filter Information: Provides Event Parameter Types and Event Parameter Value(s) to be matched against, in order to meet the condition for notifying the subscribed Event ID e.g. the Event Parameter Type could be "Area of interest" and Event Parameter Value list could be list of TAs; The Event Filter depends on the Event ID. The Event Filter Information is provided per Event ID(s) being subscribed to: within a subscription different Event ID(s) may be associated with different Event Filter Information.

-	Event Reporting Information described in the Table 4.15.1-1 below. Within a subscription all Event ID(s) are associated with a unique Event Reporting Information.

-	Target of Event Reporting: this may indicate a specific UE or PDU Session, a group of UE(s) or any UE (i.e. all UEs), Within a subscription all Event ID (s) are associated with the same Target of Event Reporting (possibly corresponding to multiple UE or multiple PDU Sessions).

-	A Notification Target Address (+ Notification Correlation ID) allowing the Event Receiving NF to correlate notifications received from the Event provider with this subscription. A subscription is associated with an unique Notification Target Address (+ Notification Correlation ID). In the case that the NF consumer subscribes to the NF producer on behalf of other NF, the NF consumer includes the Notification Target Address(+Notification Correlation ID) of other NF for the Event ID which is to be notified to other NF directly and the Notification Target Address(+Notification Correlation ID) of itself for the Subscription change related event notification. Each Notification Target Address(+ Notification Correlation ID) is associated with related (set of) Event ID(s).

-	An Expiry time represents the time upto which the subscription is desired to be kept as active. The NF service consumer may suggest an Expiry time and provide to the NF service producer. Based on the operator's policy, the NF service producer decides whether the subscription can be expired. If the subscription can be expired, the NF service producer determines the Expiry time and provide it in the response to the NF service consumer. If the event subscription is about to expire based on the received Expiry time and the NF service consumer wants to keep receiving notifications, the NF service consumer update the subscription with the NF service producer in order to extend the Expiry time. Once the Expiry time associated with the subscription is reached, the subscription becomes invalid at the NF service producer. If the NF service consumer wants to keep receiving notifications, it shall create a new subscription with the NF service producer.

When the subscription is accepted by the Event provider NF, the consumer NF receives from the event provider NF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

NOTE 1:	The Notification Correlation ID is allocated by the consumer NF that subscribes to event reporting and the Subscription Correlation ID is allocated by the NF that notifies when the event is met. Both correlation identifiers can be assigned the same value, although in principle they are supposed to be different, as they are optimized for finding the subscription related context within each NF.

The consumer NF may use an operation dedicated to subscription modification to add or remove Event ID(s) to this subscription or to modify Event Filter Information.

Events are subscribed by the consumer NF(s) by providing Event Filters. The contents of the Event Reporting Information along with the presence requirement of each information element is described in Table 4.15.1-1.

Table 4.15.1-1: Event Reporting Information

| Event Reporting Information Parameter | Description | Presence requirement |
| --- | --- | --- |
| Event reporting mode | Mode of reporting - e.g. reporting up to a maximum number of reports, periodic reporting along with periodicity, reporting up to a maximum duration, reporting when threshold is reached | mandatory |
| Maximum number of reports | Maximum number of reports after which the event subscription ceases to exist | (see NOTE 1) |
| Maximum duration of reporting | Maximum duration after which the event subscription ceases to exist | (see NOTE 1) |
| Immediate reporting flag | The Event provider NF notifies the current status of the subscribed event, if available, immediately to the consumer NF. |  |
| Sampling ratio | Percentage of sampling (1%..100%) among impacted UEs. | optional (see NOTE 2) |
| Partitioning criteria | Criteria for partitioning UEs before applying sampling ratio. | optional (see NOTE 2) |
| Group Reporting Guard Time | Parameter for group-based monitoring configuration to indicate the time for which the Monitoring Event Reporting(s) related with the UEs in a group can be aggregated before sending them to the consumer NF. | optional |
| Deactivate notification flag | Indicates to the Event provider NF that the notification of the available events shall be muted until the Event consumer NF provides the retrieval notification flag to retrieve the events stored. | optional |
| Retrieval notification flag | Indicates to the Event provider NF to send the notification to the Event consumer NF with the stored events and mutes again the notification of future events. | optional |
| Granularity of dynamics | The maximum amount of dynamics in the event which allows to skip an event notification. | optional (see NOTE 3) |
| Reporting type | Event provider NF reports only when the events differs from the previously notified event. | optional (see NOTE 4) |
| Reporting Threshold | Threshold values indicate conditions on the level to be reached for the reporting | Optional (see NOTE 5) |
| NOTE 1: The requester shall include 2) Maximum number of reports or 3) Maximum duration of reporting, or both, depending on 1) Event reporting mode. NOTE 2: Parameter only applicable to certain event IDs reporting metrics (e.g. Number of UEs present in a geographical area) used and used e.g. by the NWDAF for data collection. NOTE 3: The Granularity of dynamics includes 1) the range of scalar value, 2) the list of events identification, or 3) the previous notification. The range of scalar value is only applicable to the event depicted as number (e.g. the number of UE), the others is applicable to the event depicted as identification (e.g. UE location, UE identification). NOTE 4: The differences in event includes the events have been newly appeared, disappeared and changed from the previous notification. NOTE 5: This parameter is included only if the Event reporting mode indicates reporting when threshold is reached. |  |  |

NOTE 2:	Explicit unsubscribe by the NF consumer is still possible.

Maximum number of reports is applicable to the subscription to one UE or a group of UE(s). When the subscription is applied to a group of UE(s), the initial value of the parameter is applied to each individual member UE. The count of number of reports is per UE and per Event Type granularity also for group member UE.

Maximum duration of reporting is applicable to the subscription to one UE, a group of UE(s) or any UE. When the subscription is applied to a group of UE(s), this parameter applies to each group member UE. When the subscription is applied to any UE, this parameter applies to all the impacted UEs.

If for a given subscription Maximum duration of reporting is included then the subscription is cancelled locally in the NF as soon as Maximum duration of reporting is reached. If the Maximum number of reports is reached for a given subscription, the NEF cancels the subscription in the affected NFs. If both Maximum Number of reports and Maximum duration of reporting are included then the subscription expires or is cancelled as soon as one of the conditions is met.

Sampling ratio is applicable to subscription targeting a group of UEs or any UE. When a sampling ratio is provided, a random subset is selected among the target UEs according to the sampling ratio and only the events related to this subset are reported. A UE remains selected until it is no longer managed by the event provider NF. A UE newly managed by the NF may be selected.

Partitioning criteria are applicable to subscription targeting a group of UEs or any UE and may be used when sampling ratio is used. These criteria are used to instruct the NF on how to group the UEs before applying the sampling ratio. When partitioning criteria are provided, the NF groups the targeted UEs (i.e. creates sub-populations/strata) based on the partition criteria parameter. Then, from each sub-population/stratum, the NF selects a subset of UEs by sampling randomly from each sub-population according to the sampling ratio. For a given type of partitioning criteria, the UE belongs to a single sub-population/stratum as long as it is served by the NF. The types of Partitioning Criteria are described in Table 4.15.1-2:

Table 4.15.1-2: Types of Partitioning Criteria

Types of Partitioning Criteria

Type Allocation Code

Subscriber PLMN ID

Geographical area, i.e. list(s) of TAI(s)

S-NSSAI

DNN

Group Reporting Guard Time is an optional parameter for group-based monitoring configuration to indicate the time for which the Monitoring Event Reporting(s) related with the UEs in a group can be aggregated before sending them to the consumer NF. The value of the Group Reporting Guard time should be set less than the Maximum duration of reporting. For the continuous monitoring reporting, unless the Maximum duration of reporting has been reached, the Group Reporting Guard timer is restarted when it expires. If the time left until the Maximum duration of reporting is less than the Group Reporting Guard Time, then the Group Reporting Guard timer shall be set to expire when the Maximum duration of reporting expires. If the Maximum duration of reporting is expired, the Group Reporting Guard Time, if running, shall be considered to expire and aggregated Monitoring Event Reporting(s) is sent to destination immediately.

Deactivation notification flag and retrieval notification flag enable the mute storage of events for a limited size of events at the Event producer NF, thus reducing the number of notifications and the overall signalling between the Event producer NF and the Event consumer NF. Usage of these parameters is further specified in clause 6.2.7 of TS 23.288.

Granularity of dynamics indicates negligible changes in the target event of the subscription for which notification is not required. The changes in the Granularity of dynamics are depicted as the range of scalar value (x, y) where it means [the previously notified scalar value - x, the previously notified scalar value + y), the list of event identification(s) (e.g. list of SUPI(s)), or the previous notification. If the Granularity of dynamics is provided, the provider NF shall send the notification only when the changes in the target event are not described in the Granularity of dynamics, except for the first notification.

Reporting type is a parameter to reduce the data volume of notification. If the Reporting type is provided, the provider NF shall notify the events that have been newly appeared, disappeared and changed compared to the previous notification, except for the first notification.

Table 4.15.1-1 indicates the presence requirements for the Event Reporting Information.

Corresponding notifications contain at least the Notification Correlation ID together with the Event ID and the individual target (e.g. UE or PDU Session ID) associated with the notification.

If the NF service consumer decides to terminate the event subscription, it unsubscribes the event subscription by sending unsubscription request to the event provider NF. After receiving unsubscription request from the NF service consumer, the event provider NF terminates the event subscription.

If the NF service consumer has subscribed to group-based monitoring, then the NF service consumer may subsequently unsubscribe to the whole group or one or more identified group member UEs. Such partial cancellation of group-based monitoring does not affect the other group member UEs, but it only cancels the monitoring event subscription for the identified group member UEs. Partial cancellation of group-based monitoring can be caused by the following reasons:

-	UE's subscription is discontinued in the UDM;

-	UE's authorisation to the subscribed event type is revoked;

-	The subscribing NF explicitly unsubscribes to monitoring of selected UE(s); or

-	UE is removed from the monitoring target group that was identified in monitoring subscription.

If the NF service consumer has subscribed to group-based monitoring and later new group member UEs are added to the group, then the NF service consumer may also subsequently add monitoring event subscription for the new group member UEs.

The following clauses describe the external exposure of network capabilities and core network internal event and capability exposure.

When the immediate reporting flag is set, the first corresponding event report is included in the subscription response message, if corresponding information is available at the reception of the subscription request of the event.

During Monitoring Event Subscription as in clause 4.15.3, Parameter Provisioning as in clause 4.15.6, NIDD configuration as in clause 4.25.3 and Enhanced Coverage Restriction Control as in clause 4.27.1, the optional parameter MTC Provider Information is a reference parameter that may be provided by AF or determined by NEF based on which AF it communicates with. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application.

NOTE 3:	The MTC Provider Information can be used by Service Providers for, e.g. to distinguish their different customers.

NOTE 4:	The MTC Provider Information is an optional parameter. The NEF can validate the provided MTC Provider Information and override it to a NEF selected MTC Provider Information based on configuration. How the NEF determines the MTC Provider Information, if not present, is left to implementation (e.g. based on the requesting AF).

* * * Next Changes * * *

### 4.15.2	External Exposure of Network Capabilities

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability, network status reporting capability, Analytics reporting capability and UE member selection assistance capability. The Monitoring capability is for monitoring of specific event for UE in 5GS and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5GS. The Policy/Charging capability is for handling QoS and charging policy for the UE based on the request from external party. The Analytics capability is for allowing external party to acquire analytics information generated by 5G System. The UE member selection assistance capability allows an external party to acquire a list of candidate UEs recommended by the 5GS based on the parameters contained in the request from the AF. The list of candidate UEs assists the AF to select UEs as members for an application layer operation e.g., federated learning operation in the application layer.

The details for the External Exposure of Analytics capabilities as well as interactions between NEF, external party and NWDAF are described in TS 23.288 [50].

* * * Next Changes * * *

### 4.15.X	UE member selection assistance

The UE member selection assistance capability hosted by the NEF provides a list of candidate UE(s) based on the request receiving from the AF. AF provides a UE list and a set of filtering criteria as part of the service operation parameters that the candidate UEs should satisfy. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve the information from 5GC NFs, separately. NEF consolidates all the information collected from other 5GC NFs to derive a list of candidate UEs, which meets the filtering criteria requested by the AF and sends the list of candidate UE(s)that fulfilled the filtering criteria  to the AF. The member selection assistance capability can be used to assist third party to select the group of UEs to support application service (e.g. FL operation).

The AF may also select the list of  UE that  e.g. form a federated learning operation in the application layer based on network exposure information as described in clause 4.15 of TS 23.502 [4] and in clause ,collected directly from 5GC, e.g., UE location reporting from the AMF, user plane information from the UPF and data analytics from NWDAF

#### 4.15.X.1	5GS assistance for UE member selection.

Figure 4.15.X-1: 5GC assistane to Member selection

1-2.	If the AF has not discovered its serving NEF, it queries NRF including the AF instance ID, Application Identity and member selection service indication to retrieve its serving NEF that supports the member selection assistance functionality.

3.	A PDU Session between the UE and the AF may have been established.

4.	AF subscribes the member selection assistance functionality by sending Nnef_ UESelectionAssistance_subscribe including the Application Identity, a. list of target UE(s), one or more filters listed in the table 5.2.6.X.1-1, optionally time window(s).

Editor’s Note:	Whether the  list of filters in table 5.2.6.X.1-1 needs to be expanded is FFS.

5.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and proceeds with the corresponding procedures which may involve a composite set of 5GC service procedures corresponding to a set of 5GC events, analytics and/or notifications (e.g. UPF, NWDAF, AMF) listed in the table 5.2.6.X.1-1.

6.	NEF triggers the QoS monitoring event to the UPF via PCF and SMF as described in clause 5.33.3 of TS 23.501[2] and in clause 5.2.26.2 including the different event ID to collect the QoS monitoring,

7.	NEF may subscribe to NWDAF via Nnwdaf_AnalyticsSubscription_Subscribe including the different analytics ID to collect the analytics related information

8.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list of candidate UE(s) which fulfill the filtering criteria in the AF request.

9.	NEF sends the response to the AF using the Nnef_UESelectionAssistance_Notify including the Application Identity and the list of candidate UE(s).

* * * Next Changes * * *

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_UEMemberSelectionAssistance | Get | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |

* * * Next Changes * * *

#### 5.2.6.X	Nnef_ UEMemberSelectionAssistance service

##### 5.2.6.X.1	General

The service provides the capability to subscribe, unsubscribe or notify the member selection assistance information from the NEF. See clause 4.15.x for the detailed procedures.

Table 5.2.6.X.1-1: Description of UE Member filtering criteria

| UE Member filter | Description of filtering criterion | Editor’s Note: Whether and how to specify NEF actions based on the provided filters is FFS |  |
| --- | --- | --- | --- |
| QoS and performance related information |  |  |
| QoS monitoring | Indicates monitored QoS performance for the selected UE should meet the threshold |  |  |
| Expected QoS estimate in the area | Indicate the selected UE in a certain area regarding the QoS change statistics for an Analytics target period in the past or the likelihood of a QoS change for an Analytics target period in the future. |  |  |
| UE Transmission Latency Performance | Indicate the UE transmission latency performance that refers to a time delay for completing the transmission of a specific data volume between UE to AF |  |  |
| Location related information |  |  |
| UE current location | Indicate the certain area that the selected UE appears |  |  |
| UE trajectory | Indicate the certain area trajectory that the selected UEs move in or out in the past or in the future |  |  |
| UE status |  |  |
| CM state | Indicate the selected UE should in the CM-CONNECTED state |  |  |
| UE Reachability | Indicate the selected UE should reachable |  |  |
| PDU session attributes |  |  |
| S-NSSAI, DNN | Indicate the S-NSSAI and DNN the selected UE should support |  |  |

##### 5.2.6.X.2	Nnef_UEMemberSelectionAssistance_Subscribe service operation

Service operation name: Nnef_UEMemberSelectionAssistance_Subscribe

Description: the NF consumer subscribes to receive the UE member selection assistance information, or the subscription is updated if the same subscription is already defined in NEF.

Inputs, Required: Target of Member Selection Assistance Reporting (GPSI, SUPI, a list of GPSIs or a list of SUPIs), Notification Target Address (+ Notification Correlation ID), time window(s) for selecting the candidate UEs, one or more filtering criteria.

Inputs, Optional: Subscription Correlation ID (in the case of modification of the existing subscription), a list of group member UE(s) whose subscription to the Member Selection Assistance notification(s) are removed or added for a group-based subscription, Expiry time, a set of filtering criteria shown in the clause 5.2.6.X.1 Table 5.2.6.X.1 -1.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator’s policy).

##### 5.2.6.X.3	Nnef_UEMemberSelectionAssistance_Unsubscribe service operation

Service operation name: Nnef_UEMemberSelectionAssistance_Unsubscribe

Description: the NF consumer deletes a subscription for UE member selection assistance information that is already created in NEF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication

##### 5.2.6.X.4	Nnef_ UEMemberSelectionAssistance_Notify service operation

Service operation name: Nnef_UEMemberSelectionAssistance_Notify

Description: NEF reports the UE member selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information, a list of recommended UE(s).

Outputs, Required: Operation execution result indication.

* * * Next Changes * * *

#### 4.15.X

This clause describes a  procedure that AF selects the AI/ML members by collecting network exposure information as defined in clause 4.15 of TS 23.502 [4] directly from 5GC, e.g. in case that member selection assistance functionality within NEF is not used by the AF (e.g. trusted AF by an Operator).

Figure X-1: Procedure for direct member selection assistance to AF

1.	[Optional] The AF requests the location reporting of the UEs from the AMF by invoking existing Namf_EventExposure_Subscribe (Location Reporting).

2.	[Optional] The AF initiates direct notification of QoS Monitoring procedure for delay information for the UEs in the candidate list, as defined in steps 1a-5 of clause 6.4.2.1 of TS 23.548 [10].

3.	[Optional] The AF requests user plane information, e.g. Throughput UL/DL, Packet transmission, Packet retransmission, for the UEs in the candidate list from UPF.

4.  [Optional] The AF requests analytics from NWDAF by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation, such as UE Communication, User Data Congestion Analytics, WLAN performance analytics per UE, etc, as defined in TS 23.288 [50].

NOTE X1:	In Steps 1-4, the AF maintains the candidate list itself and collects the member selection information directly from the 5GC, and includes the Target UE Identifier(s) for the UEs in the candidate list in the request.

5.	The AF selects members based on the information collected in Steps 1-4.

NOTE X2:	What information needs to be collected by the AF to perform member selection is decided by the AF itself.

NOTE X3:	AF may keep monitoring the information from AMF/UPF/SMF/NWDAF to update the members.

* * * Next Changes * * *

#### Annex Y (Informative):

#### This Annex describes an example of the procedure that 5GS assistance for FL member selection based on UE's travel history, direction and separation distance.

Figure Y-1: 5GC assistane to Member selection based on the UE’s travel history, direction and separation distance

- AF requests 5GS assistance to support the UE selection by considering the UE's historical location, UE's current location, separation distance and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

    - UE historical location: The Target AOI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

    - FL Coverage Area: The FL training server's coverage area, i.e. the coverage area of the FL training server where the selected UEs participate in the FL operation

    - Separation distance: Select those UEs that are geographically separated, fulfilling the separation distance no smaller than the certain separation distance

    - Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the criteria into the corresponding UE filtering information, NEF identifies which information need to be collected and proceeds with the corresponding procedures which may involve a composite set of 5GC service procedures corresponding to a set of 5GC events, analytics and/or notifications (e.g. UPF, PCF UDM, NWDAF, AMF) listed in the Table 5.2.6.X.1-1.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, event ID = Number of UEs present in a geographical area. AMF will provide a list of UEs that are within the FL coverage area to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the FL training server's coverage.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list.

6-7. The NEF invokes NWDAF Analytics Info request (UE list, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. The NEF compares the UE list provided by the AMF in steps 3-4 against the UE list provided by the NWDAF in steps 6-7. The NEF will then determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction and in the different sub-area within the FL coverage Area as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI (PRA) to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage.

* * * End of Changes * * *