3GPP TSG-SA WG2 Meeting #154-AH-e (e-meeting)	S2-2300231

# Elbonia, 16 – 20 January, 2023												 (revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3854 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN support in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion of the 5G_PIN study in TR 23.700-88, 5GC is enhanced to support the PIN service as described in TS 22.261 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new clause for high level description of PIN service. Introduce a new Annex to describe the relation between PIN and 5GS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support of PIN service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new), Annex N (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

## 5.X	Support of Personal IoT network service

### 5.X.1	General

Personal IoT Network (PIN) provides local connectivity between PIN elements i.e. UEs and/or non-3GPP devices. PIN elements communicate using direct connection (the connection between two PIN elements) or network-based connection (the connection between two PIN elements is relayed via one specific PIN element and/or 5GS). The management of the direct connection is out of the scope of 3GPP. For the network-based connection, a UE acts as the specific PIN element with Gateway Capability (PEGC). With the support of the PEGC registered to 5G network, the PIN Elements have access to the 5G network services and can communicate with other PIN Elements via 5GC. A PEGC may supports multiple PINs.

PIN and PIN elements are managed by specific PIN element with Management Capability (PEMC) and the support from an AF if AF deployed. A PIN includes at least one UE with PEGC and one or more UE with PEMC. The management of the PIN network and PIN Element is out of the scope of this specification. The PEGC has subscription data related to PIN deployed in the 5GS and can register to 5GS, the PEMC does not have subscription data related to PIN deployed in the 5GS and behaves as normal UE if it is registered in 5GS. See information in Annex X for the relation between PIN and 5GS.

Editor’s Note: the management role between PEMC/AF is FFS.

An AF for PIN may be deployed to support the PIN service. The AF for PIN may communicate with PEMC and PEGC via application layer for management of the PIN. The 5GC is enhanced to support the delivery of the session policy control for PIN service.

The support of 5G-RG as a PEGC is not considered in this release.

****************** END CHANGES ***************