3GPP TSG-SA WG2 Meeting #154AH (e-meeting)	S2-2300259

# Elbonia, January 16 – 20, 2023																	 (revison of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0810 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IPv6 prefix delegation in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, LG Electronics, Deutsche Telekom, ZTE, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_IPv6PD |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 5GS IPv6 prefix delegation is not generally available for UEs that do not support 5WWC and/or ProSe, consequently there is no feature parity between EPS and 5GS. This implies that different IP address allocation mechanisms may have to be used when UE establishes IP connectivity in EPS or 5GS as follows: - if UE establishes PDN connection in EPS, “IPv6 prefix delegation” can be used, - while if UE establishes PDU Session in 5GS, “IPv6 prefix delegation” cannot be used. To make IPv6 PD a generic feature, WID SP-220797 has been agreed. DISC paper S2-2205601 may be referenced for more discussion. With IPv6 prefix delegation made generally available for 5GS in TS 23.501, in session binding, a reference to TS 23.501 needs to be added and the reference to TS 23.304 for 5G ProSe Layer-3 UE-to-Network Relay can be removed as TS 23.304 simply refer to TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Session binding, clarify the reference for allocation of UE IPv4 address and/or IPv6 prefix(es) is as specified in TS 23.501 and TS 23.316. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IPv6 prefix delegation is not generally available for UEs that do not support WWC and/or ProSe, therefore there is no feature parity between EPS and 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR3859, TS 23.502 CR3690, TS 23.304 CR0164, TS 23.316 CR2075 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

##### 6.1.3.2.2	Session binding

Session binding is the association of the AF session information to one and only one PDU Session. The PCF shall perform the session binding, which may take the following PDU Session parameters into account:

a)	For an IP type PDU Session, the UE IPv4 address and/or IPv6 prefix as specified in TS 23.501 [2], in addition when using W-5GAN the specification in TS 23.316 [27] applies. For IPv6 prefix delegation, the assigned IPv6 network prefix is shorter than /64;

For an Ethernet type PDU Session, the UE MAC address(es);

b)	The UE identity (e.g. SUPI), if present;

c)	The information about the Data Network (DN) the user is accessing, i.e. DNN, if present.

Once it has determined the impacted PDU Session, the PCF shall identify the PCC rules affected by the AF session information, including new PCC rules to be installed and existing PCC rules to be modified or removed.

Session Binding applies for PDU Sessions of IP type. It may also apply to Ethernet PDU Session type but only when especially allowed by PCC related Policy Control Request triggers.

* * * * End of Changes * * * *