3GPP TSG-SA2 Meeting #154-AH-e	S2-2300265r08

# Online, , 16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3860 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SFC CR 23.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SFC |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusions of TR 23.700-18, following changes are proposed for the normative work. - A sentence in clause 5.6.7 Application Function influence on traffic routing is added to mention that a subset of the same interface can be used by an AF to influence 5G traffic steering towards a SFC - The AF description is updated to include SFC aspect - A reference for IETF spec related to SFC is added - A couple of abbreviations are added |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add text explaining how to enble AF to influence 5G traffic steering to support SFC in the specification. Additionally, a new reference and a couple of abbreviations are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for SFC in R18 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.8.2.11.6, 6.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The referred clause 5.6.X in TS23.501 is the CR4069 of TS23.501 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G NSWO	5G Non-Seamless WLAN offload

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

EAC	Early Admission Control

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSR	Mobile Base Station Relay

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Medium Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MINT	Minimization of Service Interruption

ML	Machine Learning

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network Slice AS Group

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NSWO	Non-Seamless WLAN offload

NSWOF	Non-Seamless WLAN offload Function

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SF	Service Function

SFC	Service Function Chaining

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TA	Tracking Area

TAI	Tracking Area Identity

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

NEXT CHANGE (2)

5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. | NOTE 9. |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included and the action can also indicate that a notification of the first buffered and/or a notification of first discarded packet is requested (see clause 5.8.3.2). For drop action, a notification of the discarded packet may be requested (see clause 5.8.3.2). |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). | NOTE 8. |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN), or to 5G VN internal (i.e. local switch). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI) (NOTE 8). Any extension header stored for this packet shall be added. The time stamps should be added in the GTP-U header if QoS Monitoring is enabled for the traffic corresponding to the PDR(s). |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. | NOTE 8. |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains one of the following policies identified by a TSP ID in the PCC rule. The Forwarding policy may refer to: - an N6-LAN steering policy to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator, or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7. or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Metadata (NOTE X) | Metadata the UPF needs to add to traffic sent over a SFC | The metadata information is associated with a TSP ID related to an N6-LAN steering policy |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and N19 forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and used to determine the Forwarding Policy included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and in clauses 5.6.X and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The Forwarding Policy action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for UL/DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 of TS 29.244 [65]. NOTE 7: The use of "5G VN internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. NOTE 8: When in architectures defined in clause 5.34, a FAR is sent over N16a from SMF to I-SMF, the FAR sent by the SMF may indicate that the I-SMF is to locally determine the value of this attribute in order to build the N4 FAR rule sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 9: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE X: The use of Metadata is described in clause 5.6.X. How the UPF transforms the metadata into actual information sent with the traffic (e.g., in the encapsulation header) is based on local policies related with the Forwarding Policy and not specified. |  |  |

Editor’s Note:  It is FFS how to configure the FAR if the same traffic is related to two TSP ID.

NEXT CHANGE (3)

6.2.10	AF

The Application Function (AF) interacts with the 3GPP Core Network in order to provide services, for example to support the following:

-	Application Function influence on traffic routing (see clause 5.6.7);

-	Application Function influence on Service Function Chainging (see clause 5.6.x);

-	Accessing Network Exposure Function (see clause 5.20);

-	Interacting with the Policy framework for policy control (see clause 5.14);

-	Time synchronization service (see clause 5.27.1.8);

-	IMS interactions with 5GC (see clause 5.16).

Based on operator deployment, Application Functions considered to be trusted by the operator can be allowed to interact directly with relevant Network Functions.

Application Functions not allowed by the operator to access directly the Network Functions shall use the external exposure framework (see clause 7.3) via the NEF to interact with relevant Network Functions.

The functionality and purpose of Application Functions are only defined in this specification with respect to their interaction with the 3GPP Core Network.

END OF CHANGES