SA WG2 Meeting #S2-154AH	S2-2300281

# Jan 16 – 20, 2023, Electronic

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3864. | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy control enhancements to support multi-modality flows coordinated transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile,Nokia,Nokia Shanghai Bell, Samsung, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conclusions of KI#1 and KI#2 for FS_XRM are described in TR 23.700-60 in clauses 8.1 and 8.2. To include these aspects in TS 23.501, this paper proposes to introduce a new clause for the general description of policy enhancement to support multi-modality flows coordinated transmission, and some related clause update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A general description about policy enhancement to support multi-modality flows coordinated transmission. Related clause update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Coordinated transmission for multi-modal services cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changes, all the texts are new >>>

### 5.37.X 	Policy control enhancements to support multi-modality flows coordinated transmission

Multi-modality services consist of multiple flows that are closely related and need strong application coordination and transmission coordination.

For 5GS transmission of flows that require coordinated transmission, the following policy control features are provided for the case there the flows are associated with a single UE:

-	The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, service requirements, alternative service requirements for GBR traffic, a common ID and QoS monitoring for multiple IP data flows associated to a multi-modal (XRM) application.

-	The common ID is provided by AF for each flow, which means the multiple flows consist a group of flows and are closely related, therefore PCF should consider this parameter to implement the coordinated policy control.

-	AF may provide QoS monitoring requirement for a group of flows at the same time to the NEF/PCF. The AF may indicate to 5GS further optional parameters (e.g., time period).

-	The PCF generates policies based on AF requirements for all data flows associated to a multi-modal (XRM) service, by considering the common ID.

-	When the PCF receives the requirements of multi-modal flows from the AF, the PCF generates the QoS monitoring policy and implements the QoS monitoring mechanism at the same time.

-	The multi-modal flows are carried in single PDU session for one UE. When the AF sends guidance to PCF for the determination of proper URSP rules for the UE, the AF should handle the consistency requirement for the multi-modal flows. The PCF may verify the requested parameters (which are described in clause 4.15.6.10 of TS 23.502 [3]) with regards to the common ID.

NOTE: 	If the UE determines that there are more than one existing PDU Session which mathces, it is up to UE implementation to guarantee the same PDU session is selected for multi-modal flows.

In addition to the features that are provided for the case there the flows are associated with a single UE, the following features are provided for the case where the flows are associated with more than one UE:

-	In this release, in the scenrio where the UEs are served by different PCFs, the PCF(s) take policy decisions according to the input provided by the AF. There is no support for policy coordination among the multiple PCF(s). Policy decisions are taken by each PCF separately on a per PDU session basis.

-	The same DNN/S-NSSAI combination for the multi-modality XRM service should be selected among multiple UEs.When the AF sends guidance to PCF for the determination of proper URSP rules for the UE, AF should handle the consistency requirement for the multi-modal flows among multiple UEs. The PCF(s) generate or update the URSP rules for multiple UEs with considering the common ID. The existing URSP Rule evaluation framework is used to ensure that the UE selects the correct DNN/S-NSSAI combination for the traffic.

## <<< End of changes >>>