SA WG2 Meeting #S2-154AH	S2-2300281r03

# Jan 16 – 20, 2023, Electronic

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3864. | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy control enhancements to support multi-modality flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile,Nokia,Nokia Shanghai Bell, Samsung, Xiaomi, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conclusions of KI#1 and KI#2 for FS_XRM are described in TR 23.700-60 in clauses 8.1 and 8.2. To include these aspects in TS 23.501, this paper proposes to introduce a new clause for the general description of policy enhancement to support delivery of multi-modality services, and some related clause update. Replaced “common ID” with Multimodal service identifier as that is more meaningful |  |  |  |  |  |  |  |  |
| Summary of change: |  | A general description about policy enhancement to support multi-modality services. Related clause update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | multi-modal services delivery cannot be supported using PCC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes, all the texts are new >>>

5.37.X 	Policy control enhancements to support multi-modality services

Multi-modality services consist of an application with several data flows that related to each other and may come from different sources (see TS 22.261.  Each data flow (single-modal data) may be seen as one type of data (for example audio, video, positioning, haptic data) associated with the same communication service/application.

Some of the data flows may be fully inter-dependent, all necessary for the multimodal application to function correctly (for example audio-tactile, or visual-tactile). Other data flows may be complementary and fully independent (for example ambient information).

Data flows that comprise a multimodal service may come from a single UE, either via a single device or via multiple devices connected to the single UE that can access the 5GS, or from multiple UEs. The AF may establish one or multiple AF sessions for the delivery of multimodal services.

The expected multimodal application behaviour is that those data flows that are closely related and require strong application coordination for correct delivery of a multi-modal application, are transmitted in a single PDU session by a single UE. This enables the application (AF) to handle multiple types of data in a single procedure and single AF session. However, those data flows that contribute to the service experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multipe UEs.

Policy and charging control framework for the delivery of multimodal services in the 5G System are defined in TS 23.503 [45], according to the following principlesfor single UE :

-	The procedure for AF session setup with required QoS, is reused for multimodal applications interacting with 5GC either directly to the PCF or via NEF.The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, service requirements, for each media that comprise the multimodal service alternative service requirements for GBR traffic, a correlation ID for multimodal service and QoS monitoring for multiple IP data flows associated to a multi-modal (XRM) application.

-	. The AF may provide to the PCF a correlation ID for multimodal service , as an explicit indication that the application trafficsare related to a multimodal service. The PCF may use this information to derive the correct PCC rules and apply QoS policies for data flows that are part of a specific multimodal applications.

-	The AF may provide QoS monitoring requirement for data flows associated to a multimodal application to the PCF (either directly or via NEF). The PCF generates the authorized QoS Monitoring policy for these service data flows.

-

-	When the PCF receives the requirements of multi-modal flows from the AF, the PCF generates the QoS monitoring policy and implements the QoS monitoring mechanism .

-	The multi-modal services that are carried in single PDU session for one UE, the AF may provide  guidance to PCF for the determination of proper URSP rules for the UE.

In addition to the features that are provided for the case that the flows are associated with a single UE, the following features are provided for the case where the flows are associated with more than one UE:

-	In this release, in the scenrio where the UEs are served by different PCFs, the PCF(s) take policy decisions according to the input provided by the AF. There is no support for policy coordination among the multiple PCF(s). Policy decisions are taken by each PCF separately on a per PDU session basis.

-	The same DNN/S-NSSAI combination for the multi-modality XRM service should be selected among multiple UEs. The AF may provide  guidance to PCF for the determination of proper URSP rules for the UE. The existing URSP Rule evaluation framework is reused.

In case multiple UEs are involved, AF provides the information to the PCF (either directly or via NEF) using multiple UE specific procedures.

-	For multimodality flows transmitted by multiple UEs, the AF may allocate the same correlation ID for multimodal service to all UE PDU sessions that componse a multimodal service. The PCF may take this information into account (e.g. to allocate a specific QoS profile) when processing each PDU session independently.

<<< End of changes >>>