SA WG2 Meeting #S2-154AH	S2-2300281r107

# Jan 16 – 20, 2023, Electronic

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3864. | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy control enhancements to support multi-modal flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile,Nokia,Nokia Shanghai Bell, Samsung, Xiaomi, Ericsson, InterDigital Inc., China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conclusions of KI#1 and KI#2 for FS_XRM are described in TR 23.700-60 in clauses 8.1 and 8.2. To include these aspects in TS 23.501, this paper proposes to introduce a new clause for the general description of policy enhancement to support delivery of multi-modality services, and some related clause update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A general description about policy enhancement to support multi-modal services. Related clause update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | multi-modal services delivery cannot be supported using PCC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes, all the texts are new >>>

5.37.X 	Policy control enhancements to support multi-modal services

Multi-modal services consist of several data flows (named as multi-modal flows) that related to each other and may come from different sources. Each data flow (single-modal data) may be seen as one type of data (for example audio, video, positioning, haptic data) associated with the same communication service.

Data flows that comprise a multi-modal service may come from a single UE, either via a single device or via multiple devices connected to the single UE that can access the 5GS, or from multiple UEs.

For the single UE case, the expected multi-modal application behaviour is that those data flows that are closely related and require strong application coordination for correct delivery of the multi-modal application data, are transmitted in a single PDU session.

-	The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, service requirements, for each media that comprise the multi-modal service , a correlation ID for multi-modal service and QoS monitoring requirements for multiple IP data flows associated to a multi-modal  service.

-	The AF may provide to the PCF a correlation ID for multi-modal service, as an explicit indication that the application traffics are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and apply QoS policies for data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirement for data flows associated to a multi-modal service to the PCF (either directly or via NEF). The PCF generates the authorized QoS Monitoring policy for these service data flows.

Editor’s Note: How to support the QoS monitoring for multimodal flows at the same time is FFS.

-	The multi-modal services are carried in single PDU session for one UE.

In addition to the features that are provided for the case that the flows are associated with a single UE, the following features are provided for the case where the flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modalservice should be selected among the multiple UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

-	The AF may allocate the same correlation ID for multi-modal service to all UE PDU sessions that componse a multi-modal service. The PCF may take this information into account (e.g. to allocate a specific QoS profile) when processing each PDU session independently.

Editor's note:	How to support the change of the correlation ID for multimodal service is FFS.

<<< End of changes >>>