SA WG2 Meeting #S2-154AH	S2-2300283r01

# Jan 16 – 20, 2023, Electronic

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3792 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF support of 5GS jitter value monitoring based on QoS monitoring mechanism and exposed to AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Tencent, Samsung, Nokia Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the conclusion of 8.7 in TR 23.700-60 for Key Issue #7 policy enhancements to minimize jitter has been concluded, 5GS should support to monitor the 5GS jitter value based on AF request with utilizing the QoS monitoring method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GS support to monitor the 5GS jitter value based on AF request with utilizing the QoS monitoring method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF cannot get the jitter value |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.37.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

5.37.X		 5GS transmission jitter monitoring and reporting

For XR and media services, AF sends the requirement for jitter monitoring (i.e. between UE to PSA UPF) to 5GS. Based on the QoS monitoring mechanism described in clause 5.33.3, the 5GS transmission jitter value is calculated or monitored by PCF and then reports the jitter value to AF. The jitter monitoring related parameters from AF to PCF includes the jitter measurement period, and jitter report frequency as described in TS 23.503 [12] clause 6.1.3.xx.

When PCF gets the jitter requirement, the related QoS monitoring policy is generated for a service data flow. Then PCF triggers the QoS monitoring procedure, and gets the UL/DL packet delay from SMF. PCF may calculate the 5GS transmission jitter value between UE to PSA UPF based on the UL/DL packet delay values and then reports the jitter value to AF/NEF.

NOTE: By implementation, the PCF calculates the 5GS transmission jitter value between UE to PSA UPF based on  the UL/DL packet delay values.

<<< End of changes >>>