3GPP TSG-SA2 Meeting #154AH-e 		S2-2300285

16 - 20 January, 2023, Electronic meeting

Title:	[DRAFT] LS Response on TSCTSF usage determination in AF session with required QoS

Response to:	S2-2300032/C3-225700

Release:	Rel-17

Work Item:	IIoT

Source:	SA2

To:	CT3

Cc:

Contact person:	Dan Wang

wangdanyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-230xxxx

1	Overall description

SA2 thanks CT3 for their LS on TSCTSF usage determination in AF session with required QoS. SA2 has the following answers to the 3 questions from CT3.

Q1	What is the scenario where the AF can update the required QoS from normal QoS requirement into time sensitive QoS requirement?

Answer:

Some discussion happen in SA2 and below is the common understanding. Generally, the AF can reuse the existing AF session to request new service requirement, so the scenario of AF initially request normal QoS requirement and then create new request for time sensitive QoS requirement is possible. And the traffic flows corresponding to the two QoS requirements may have the same or different IP 5-tuple.

Q2	Is it possible for the AF to update the required QoS from time sensitive QoS requirement into normal QoS requirement? And then will the NEF always keep the initial decision about triggering the TSCTSF or not as determined in the initial request?

Answer:

Based on the Q1 answer, it is possible for the AF to add new QoS requirement with reusing the established AF session. When AF request the new QoS requirement with reusing the AF session, the NEF should always keep the initial decision about the TSCTSF invocation. SA2 has decided that NEF should make the decision about whether to invoke TSCTSF based on AF ID, DNN/S-NSSAI, and local configuration. Therefore during the AF session existing period, the AF ID, DNN/S-NSSAI should not be changed, so the initial decision about TSCTSF should not be changed. SA2 has approved the S2-230xxx to reflect the above.

Q3	If the answer to Question 1 is that the scenario exists, then could it be possible to have any solution where the NEF does not reject the AF update request?

Answer:

Based on the answer to Q2, if AF follows the SLA between the AF and 5GS, the AF should request with the correct AF ID, DNN/S-NSSAI. And the NEF can make the correct decision about TSCTSF invocation in the initial request, and the following AF request will not be rejected.

While if the AF make requirement with DNN/S-NSSAI of normal QoS requirement, the NEF may authorize the AF request without invoking the TSCTSF. And then AF make requirement for time sensitive QoS requirement but with using the DNN/S-NSSAI of normal QoS requirement, NEF may reject the AF request.

SA2 has discussed and agreed that it is not suitable to create a new solution at this time (R17 stage-2 work has been frozen, while R18 work item do not include such aspect), so there is no solution to help NEF do not reject the AF update request. SA2 has agreed to introduce a cause value to indicate the reason of failure, and the detailed is described in S2-230xxx.

.

2	Actions

To CT3

ACTION: 	SA2 asks CT3 group to take the above information into consideration.

3	Dates of next TSG SA WG2 meetings

TSG-SA2 Meeting #155		Feb 20 – 24, 2023				Athens

TSG-SA2 Meeting #156   	Apr 17 – 21, 2023				Emeeting