3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300302

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3870 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing ENs for TSN TN integration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are still some ENs in TS 23.501: Editor's Note 3: Whether the buffer duration is considered in the above formula is FFS. Editor's Note 5: Whether the buffer duration is considered in the above formula is FFS. The formula are related to MaxLatency and AccumulatedLatency. As described in IEEE 802.1Qcc: MaxLatency and AccumulatedLatency are the required/real latency in the path, i.e. the delay due to buffer in the end user are considered in these two parameters. Editor's Note 6: it is FFS if and how the Service Data Flows can be used to identity streams in the TN. RAN can be only aware of QoS Flow but not SDF, so it cannot differentiate different SDFs even as a Talker. It is proposed to remove the EN directly. Editor’s Note 7: How SMF/CUC derives the input information based on the TimeAwareOffset and where this input information is mapped to Gate Control configuration information is FFS. There are 2 options: one is the SMF just transfers the TimeAwareOffset to the TLs and TL will know the exact window by related information, e.g periodicity, TimeAwareOffset, and EarlistTransmit offset by itself. The other is SMF/CUC calculates the window by itself and send the related information, e.g. adminbase time directly to the TLs. In the second case, the SMF still needs to provide per stream level information to the TLs. It is proposed to go with the first option. It is simpler and gives more flexibility to the TLs. Editor’s Note 8: The handling of received FailureCode, Failed TalkerStatus, (Partial) Failed Listener Status, and FailedInterfaces information is FFS. The SMF/CUC can react on most failure codes directly. For example, it may select a different VLAN in the TSN TN or selects a different PCP value for the TN stream. Thus, it is up to SMF/CUC implementation to handle failures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the EN as above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Still FFS in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex M.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex M (normative):
Interworking with TSN deployed in the Transport Network

# M.1	Mapping of the parameters between 5GS and TSN UNI

Editor's note:	The information provided here is a guidance for better understanding of the function. Content of this Annex will be updated to a functional level detail once stage 3 work is stable.

The details of the parameters in the TSN UNI are specified in IEEE Std 802.1Qcc [95]) and IEEE P802.1Qdj [146].

The SMF/CUC derives the End Station related information for the stream requirements towards the TN CNC for the QoS Flow as follows:

a)	For the Talker group:

‐	StreamID: can be generated by the SMF/CUC based on the End Station MAC address acting as Talker, and PDU Session ID and QFI. The MAC address is either pre-configured at the SMF/CUC or provided by the AN-TL or CN-TL to the SMF/CUC (e.g., as part of the EndStationInterfaces information).

-	StreamRank: set to zero for ARP priority values 1-8; set to one for other ARP values.

‐	EndStationInterfaces: If the AN-TL and CN-TL are supported, the SMF/CUC receives the EndStationInterfaces (MacAddress, InterfaceName) from the AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported the SMF/CUC sets the information based on pre-configuration.

-	DataFrameSpecification (optional): When it is present it specifies how the TN can identify packets of the TN stream in order to apply the required TSN configuration. If the AN-TL and CN-TL are supported, the SMF/CUC does not include the DataFrameSpecification to the TN CNC.

If the AN-TL and CN-TL are not supported, the SMF/CUC derives the DataFrameSpecification based on the N3 tunnel end point addresses/ports that are used for the QoS Flow. The SMF/CUC instruct the UPF and NG-RAN to assign a separate N3 tunnel end point address for each QoS Flow. This ensures the TN can distinguish the QoS Flows based on the N3 tunnel destination IP addresses.

NOTE 1:	IPv6 can be used in the N3 tunnel end point addresses to provide sufficient address space.

If the DataFrameSpecification is provided to the TN CNC, the TN CNC configures the edge bridge to perform the stream transformation based on the provided the DataFrameSpecification

Editor's note:	It is FFS if the interface between the SMF/CUC and TN CNC could allow the SMF/CUC to indicate the TEID and QFI of the given QoS Flow to the TN CNC.

‐	TrafficSpecification elements:

‐	Interval: derived from the Periodicity of the traffic as indicated in the TSCAI.

‐	MaxFramesPerInterval: specifies the the maximum number of frames that the Talker transmits in one Interval.

‐	MaxFrameSize: derived from the Burst Size of the QoS Flow. The PCF transfers the Burst Size to the SMF/CUC. The SMF/CUC sets MaxFrameSize based on the following formula: Burst Size of the QoS Flow - the framing bits which is not used for transferring in 5GS, (e.g. CRC + the GTP-U tunnel overhead).

-	TransmissionSelection: specifies the algorithm that the Talker uses to transmit the Stream's traffic class. If no algorithm is known, the value zero (strict priority) is used.

‐	TSpecTimeAware group (optional, present only if the traffic in the QoS Flow is time-synchronized):

‐	EarliestTransmitOffset: the earliest offset within the Interval.

For uplink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in UL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UE-DS-TT Residence Time + 5G-AN PDB - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in UL direction + UE-DS-TT Residence Time + 5G-AN PDB > M x Interval duration.

would be true.

For downlink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in DL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UPF Residence Time - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in DL direction + UPF Residence Time > M x Interval duration.

would be true.

‐	LatestTransmitOffset: the last chance within an interval should leave enough time to transfer a packet with MaxFrameSize. Derived from the end of the interval, subtracted by the sum of Jitter and the time to transfer a packet with MaxFrameSize.

‐	Jitter: derived based on local configuration.

‐	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one (no redundancy) or other value (if redundancy is required).

‐	MaxLatency: set to CN PDB subtracted by UPF Residence Time and maximum possible buffer duration in Talker. Maximum possible buffer duration is set to LatestTransmitOffset subtracted by EarliestTransmitOffset up to Buffer capability sent by the Talker.

Editor's note:	Whether the buffer duration is considered in the above formula is FFS.

Editor's note:	Whether the UPF Residence Time is already considered in CN-PDB, or SMF/CUC needs to subtract it from CN-PDB, is FFS.

-	InterfaceCapabilities (optional): If the AN-TL and CN-TL are supported, the SMF/CUC collects InterfaceCapabilities from AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported, the SMF/CUC leaves the InterfaceCapabilities empty.

b)	For the Listener group:

-	Stream ID and Stream Rank: that were generated for the Talker of the TN stream are also used by the SMF/CUC for the Listener.

-	EndStationInterfaces: derived as with the corresponding information for the Talker group.

-	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one.

‐	MaxLatency: derived as with the corresponding information for the Talker group.

-	InterfaceCapabilities: derived as with the corresponding information for the Talker group.

c)	For the Status group: The Status group contains the end station communication-configuration provided by TN CNC to the SMF/CUC:

‐	Stream ID.

‐	StatusInfo.

‐	AccumulatedLatency: If the AccumulatedLatency is included from TN CNC to SMF/CUC for a stream in DL direction, the SMF/CUC may use the AccumulatedLatency to update the TSCAI BAT to the NG-RAN; the SMF sets the TSCAI Burst Arrival Time in downlink direction as the sum of the TSCAC BAT value in downlink direction and AccumulatedLatency, and the buffer duration in Talker in CN-TL. The buffer duration in CN-TL is zero if TimeAwareOffset for the Talker group is not present, and TimeAwareOffset - EarliestTransmitOffset if the TimeAwareOffset is present for the Talker group.

‐	InterfaceConfiguration (optional):

‐	MAC Address (optional, present only if the respective InterfaceCapability contains a value for Active Destination MAC and VLAN Stream identification in CB-StreamIdenTypeList, and stream transformation is performed in AN-TL and CN-TL).

‐	VLAN Tag (optional, present only if the respective InterfaceCapability contains that it is VlanTagCapable and a value for Active Destination MAC and VLAN Stream identification in CB-StreamIdenTypeList, and the stream transformation is performed in AN-TL and CN-TL).

‐	IPv4/IPv6 Tuples (optional, but not supported in this release of the specification).

‐	TimeAwareOffset (optional, present only if the traffic is time-synchronized, AN-TL and CN-TL is supported, and TSpecTimeAware elements were provided in the stream requirements).

If the InterfaceConfiguration is included from TN CNC to SMF/CUC, and if the AN-TL/CN-TL are supported, and AL-TL/CN-TL support the Stream Transformation as described in IEEE Std 802.1Qcc [95], the SMF/CUC can instruct the UPF and NG-RAN to assign for each QoS Flow an individual TSN Transport address by providing the InterfaceConfiguration to the AN-TL/CN-TL via TL-Container. The Talker in AN-TL/CN-TL shall use the indicated InterfaceConfiguration, e.g. source MAC address, multicast destination MAC address, VLAN ID, as assigned by the TN CNC for the data stream in a QoS Flow. The TN can identify the streams based on the Stream Transformation that is applied in the Talker in the AN-TL/CN-TL. This allows to use a single GTP-U tunnel as defined for non-TSN Transport networks.

Editor's note:	It is FFS if and how the Service Data Flows can be used to identity streams in the TN.

If the TimeAwareOffset is included from TN CNC to SMF/CUC, the SMF/CUC should derive input information based on the TimeAwareOffset, which allows to configure Gate Control information as defined in IEEE Std 802.1Q [98] at the AN-TL (for streams in UL direction) and the CN-TL port (for streams in the DL direction), i.e., AdminBaseTime, AdminCycleTime, AdminControlListLength, and AdminControlList). The AN-TL or CN-TL acting as Talker buffers the data burst until the time indicated in the TimeAwareOffset is reached.

If the SMF/CUC receives a TimeAwareOffset from TN CNC for a downlink stream (i.e. for a Talker in the UPF/CN-TL), the SMF/CUC adds the received TimeAwareOffset value to the TSCAI BAT in the DL direction in the TSCAI and updates the NG-RAN for the new TSCAI.

‐	FailedInterfaces (optional) provides a list of one or more physical ports of failed end stations or bridges to locate the interfaces in the physical topology that caused the failure. It is up to implementation how the SMF reacts when it receives FailedInterfaces.

NOTE 2:	It is assumed that the end station communication-configuration will contain at least the same information as defined for the status.

# * * * * End of changes * * * *