3GPP TSG-WG SA2 Meeting #154-AH-e  	S2-2300304

Elbonia, January 16 – 20, 2023	(revision of S2-220xxxx)

# Title:	[DRAFT] LS Reply on UL scenario of reactive RAN feedback for burst sending time adjustment

# Response to:	LS (S2-2300044/R2-2213070) on Reply LS on UL scenario of reactive RAN feedback for burst sending time adjustment from RAN2

# Release:	Release 18

# Work Item:	TRS_URLLC

Source:	SA2

To:	RAN2

Cc:	RAN3

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR 3872

1. Overall Description:

Thanks RAN2 for the feedback on UL scenario of reactive RAN feedback for burst sending time adjustment.

SA2 would like to answer the question as following:

Question:

- In this adaptation mechanism applied to the UL, how dynamically would the UE reported information change, and what would be the delay requirement for providing such information?

SA2 Answer:

For reactive UL RAN feedback, it can happen at the beginning of the traffic transmission after QoS flow establishment, however it can also happen when there is change on the burst sending time of the application or the resource status in the RAN.

There is no exact value on the delay requirement for providing the UE reported information, but SA2 expects that the UE reported information should be provided to RAN as soon as possible.

Besides, SA2 confirms the requirement for RAN to provide reactive feedback for burst sending time adjustment for UL scenario and understands RAN2 needs more time to discuss the details to achieve it, so SA2 agrees to leave the details (incl. whether the UE provides the BAT offset to the RAN or RAN can determine the BAT offset based on other information provided by the UE) to RAN2. Please see the details in the attachment.

2. Actions:

To RAN2 group.

ACTION: 	SA2 requests RAN2 group to

take the above information into account to achieve the Reactive RAN feedback for UL scenario and update their specifications accordingly.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #155		Feb 20 – 24, 2023				Athens, GR

TSG-SA2 Meeting #156		Apr 17 – 21, 2023				TBD