3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300309r02

# Elbonia, January 16 – 20, 2023		(revision of S2-2210348)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0564 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Accuracy Checking Capability to NWDAF Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#1 in TR 23.700-81 define that NWDAF can have the accuracy checking capability and consumers of NWDAF can request the accuracy information to such NWDAFs with such capability. Such definitions need to be included in NWDAF procedure descriptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Clause in 6.2D is included in the specification to describe the definitions related to Accuracy Information Checking and Provisioning to NF Consumers as well as the procedure used by NF Consumers and NWDAF with AnLF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 agreements on KI#1 will not be properly captured in R18 NWDAF description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D (NEW), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.2D	AnLF Analytics Accuracy Monitoring Procedures

### 6.2D.1	General

The Analytics Accuracy Information comprises the value determined for the accuracy of an analytics ID used in the network. Optionally it may include an indication that the determined accuracy value for the analytics ID does not meet the preferred level of accuracy requested for the analytics ID.

When multiple NWDAFs are deployed, some NWDAFs may be specialized with the analytics accuracy checking capability. When an NWDAF containing AnLF has the accuracy checking capability, such NWDAF is able to:

-	Receive a subscription or request for analytics IDs via Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations with the indication for activating the mechanisms for checking the accuracy of such analytics ID.

-	Provide the accuracy information to the consumer via Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Response.

The request or subscription to analytics accuracy information triggers the monitoring and check of Analytics ID(s) and generation of analytics accuracy information. The Analytics accuracy information may be requested per Analytics ID and scoped using the same parameters defined in the Target of Analtyics Reporting (i.e., UEs, any UE, group of UEs) and Analytics Filter Information (e.g., for a specific area, specific slice) of the requested Analytics ID.

When the accuracy checking is activated in an NWDAF containing the ANLF, the NWDAF may store for a period of time the necessary information to determine the analytics IDs accuracy and provide the accuracy information to consumers when requested or use it for its internal processes.

NWDAF containing AnLF generates the accuracy information based on any of the following information:

- Comparing predictions and its corresponding ground truth data.

NOTE 3:	The ground truth data and the corresponding prediction is to be defined per Analytics ID.

- Comparing changes in internal configuration for the analytics ID generation (e.g., data collection parameters).

- Previous existent records of analytics accuracy information.

### 6.2D.2	Procedures for Analytics Accuracy Information Subscription

This procedure is used by NF consumers of analytics ID to subscribe simultaneously to receive analytics output and analytics accuracy information related to the requested analytics ID for NF consumer. Figure 6.1X.2-1 shows the procedure for accuracy information subscription and provisioning.

Figure 6.2D.2-1: Analytics Accuracy Information Subscribe

1.	The NWDAF with AnLF, register the NWDAF NF Profile in NRF including the accuracy checking capability.

2.	The NWDAF service consumer discovers at NRF the NWDAF with accuracy checking capability.

3.	The NWDAF service consumer subscribes to analytics accuracy information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be included in the subscription to trigger the accuracy information checking and provisioning are listed in clause 6.1.3.

4.	When a subscription request is received, the NWDAF determines whether the subscription is only for analytics output generation or if it includes the Analytics Accuracy request. In case Analytics Accuracy request is included, NWDAF starts the moniroting, checking, and generation of the accuracy information related to the analytics ID indicated in the subscription and according with the parameters defined in Analytics Accuracy Request Information in clause 6.1.3.

5.	NWDAF performs the data collection and analytics output generation.

6a.	NWDAF provides the analytics output according with the parameters defined in Analytics Reporting Information included in the subscription request.

6b.	NWDAF provides the analytics accuracy information for the analytics ID according with the parameters defined in the Analytics Accuracy Request Information included in the subscription request.

7.	NWDAF may determine the low or insufficient accuracy for an analytics ID and may notify the NWDAF Service consumer with the Pause Analytics Output Consumption indication.

8.	(Optional) NWDAF Service Consumer may decide to stop the consumption of analytics output without unsubscribing to the analytics ID, based on its own logic or based on a received notification from NWDAF with the Stop Analytics Output Consumption indication. The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provide the parameter Stop Analytics Subscription request in the Analytics Accuracy Request Information.

9.	When NWDAF determines an improvement in the accuracy of an analytics ID (e.g., meet the accuracy requirement of the analytics consumer) or when the Pause Analytics Subscription time window requested by an NWDAF Service Consumer is finished, NWDAF notifies the NWDAF Service Consumer of the accuracy information for the analytics ID to resume the consumption of the analytics output, therefore reactivating an existing analytics ID subscription that have been previously stopped.

10.	(Optional) NWDAF Service Consumer based on its own logic can notify the NWDAF Service Consumer to resume the provisioning of analytics output, therefore reactivating an existing subscription to analytics ID that have been paused either by NWDAF Service Consumer request (Step 8) or by NWDAF indication (Step 7). The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provide the parameter Resume Analytics Subscription request in the Analytics Accuracy Request Information.

# * * * * End of changes * * * *