3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300309r07

# Elbonia, January 16 – 20, 2023		(revision of S2-2210348)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0564 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Accuracy Checking Capability to NWDAF Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#1 in TR 23.700-81 define that NWDAF can have the accuracy checking capability and consumers of NWDAF can request the accuracy information to such NWDAFs with such capability. Such definitions need to be included in NWDAF procedure descriptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Clause in 6.2D is included in the specification to describe the definitions related to Accuracy Information Checking and Provisioning to NF Consumers as well as the procedure used by NF Consumers and NWDAF with AnLF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 agreements on KI#1 will not be properly captured in R18 NWDAF description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D (NEW), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2D	AnLF Analytics Accuracy Monitoring Procedures

6.2D.1	General

The Analytics Accuracy Information comprises a set of parameters as defined in Clause 6.1.3..

When multiple NWDAFs are deployed, some NWDAFs may be specialized with the analytics accuracy checking capability. When an NWDAF containing AnLF has the accuracy checking capability, such NWDAF is able to:

-	Receive a subscription for analytics IDs via Nnwdaf_AnalyticsSubscription_Subscribe operation with the indication for activating the mechanisms for checking the accuracy of such analytics ID as defined in Clause 6.1.3.

-	Provide the accuracy information to the consumer via Nnwdaf_AnalyticsSubscription_Notify.

Editor’s Note: Whether AnLF can provide only the accuracy information to a consumer (ie., with no analytics output) is FFS.

The request or subscription to analytics accuracy information triggers the monitoring and check of Analytics ID(s) and generation of analytics accuracy information. The Analytics accuracy information may be requested per Analytics ID and scoped using the same parameters defined in the Target of Analtyics Reporting (i.e., UEs, any UE, group of UEs) and Analytics Filter Information (e.g., for a specific area, specific slice) of the requested Analytics ID.

Editor’s Note: A comprehensive definition of what triggers AnLF to start monitoring Analytics ID accuracy is FFS.

When the accuracy checking is activated in an NWDAF containing the ANLF, the NWDAF may store for a period of time the necessary information to determine the analytics IDs accuracy and provide the accuracy information to consumers when requested or use it for its internal processes.

NWDAF containing AnLF generates the accuracy information based on any of the following information:

- Comparing predictions and its corresponding ground truth data, which are collected corresponding to analytic ID requested at the time which the prediction refers to.

NOTE 3:	The ground truth data and the corresponding prediction is to be defined per Analytics ID.

- Comparing changes in internal configuration for the analytics ID generation (e.g., data collection parameters, change in data distribution from a Data Source ).

- Previous existent records of analytics accuracy information.

6.2D.2	Procedures for Analytics Accuracy Information Subscription

This procedure is used by NF consumers of analytics ID to subscribe simultaneously to receive analytics output and analytics accuracy information related to the requested analytics ID for NF consumer. Figure 6.1X.2-1 shows the procedure for accuracy information subscription and provisioning.

Figure 6.2D.2-1: Analytics Accuracy Information Subscribe

1.	The NWDAF service consumer selects the appropriated NWDAF with AnLF according with Clause 5.2 and subscribes to analytics accuracy information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be included in the subscription to trigger the accuracy information checking and provisioning are listed in clause 6.1.3.

2.	When a subscription request is received, the NWDAF containing AnLF determines whether the subscription is only for analytics output generation or if it includes the Analytics Accuracy request.

In case Analytics Accuracy request is included, the NWDAF containing AnLF starts the analytics accuracy moniroting, , and generation of the analytics accuracy information related to the analytics ID indicated in the subscription and according with the parameters defined in Analytics Accuracy Request Information in clause 6.1.3. The NWDAF containing AnLF is to compute analytics accuracy information by comparing predictions and its corresponding ground truth data.

The NWDAF containing AnLF already started to perform the analytics accurary moniroting and analytics accuracy information generation, which may be triggered by other NWDAF service consumer(s) before.

In addition to the received request from the NWDAF service consumer, based on local policy, NWDAF containing AnLF may determine to start the analytics accurary moniroting and analytics accuracy information generation.

Editor’s Note: It is FFS how to handle the multiple requests from one or more NWDAF service consumers for the analytics accurary moniroting and analytics accuracy information generation.

Editor’s Note: the call flow only shows a subscription/notification model and it is FFS whether how to support request-response model.

3.	The NWDAF containg AnLF performs the data collection and analytics output generation.

Editor’s Note: Whether and how additional steps are needed to describe data collection for analytics accuracy determination is FFS.

Editor’s Note: It is FFS whether and how to separate the “input data collection” and “ground truth data collection” into two different steps because they are performed in quite different time stamps.

4a.	The NWDAF containing AnLF provides the analytics output according with the parameters defined in Analytics Reporting Information included in the subscription request.

4b.	The NWDAF containing AnLF provides the analytics accuracy information for the analytics ID according with the parameters defined in the Analytics Accuracy Request Information included in the subscription request.

5.	When deterimingthe low or insufficient accuracy for an analytics ID i.e. the deviation of the output analytics using the trained ML model from the ground truth data (which are collected from Data Producer NF corresponding to analytic ID requested at the time which the prediction refers to) is greater than the Reporting Threshold(s) (which are locally configured or received in the Subscribe request, ,the NWDAF containing AnLF may notify the NWDAF Service consumer with the Stop Analytics Output Consumption indication.

6.	(Optional) NWDAF Service Consumer may decide to stop the consumption of analytics output without unsubscribing to the analytics ID, based on its own logic or based on a received notification from NWDAF with the Stop Analytics Output Consumption indication. The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provide the parameter PauseAnalytics Subscription request in the Analytics Accuracy Request Information.

7.	When NWDAF determines an improvement in the accuracy of an analytics ID (e.g., meet the accuracy requirement of the analytics consumer) or when the Pause Analytics Subscription time window requested by an NWDAF Service Consumer is finished, NWDAF notifies the NWDAF Service Consumer of the accuracy information for the analytics ID to resume the consumption of the analytics output, therefore reactivating an existing analytics ID subscription that have been previously stopped.

8.	(Optional) NWDAF Service Consumer based on its own logic can notify the NWDAF Service Consumer to resume the provisioning of analytics output, therefore reactivating an existing subscription to analytics ID that have been paused either by NWDAF Service Consumer request (Step 6) or by NWDAF indication (Step 5). The NWDAF Service Consumer invokes the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Subscription Correlation ID to modify an existing subscription and provide the parameter Resume Analytics Subscription request in the Analytics Accuracy Request Information.

# * * * * End of changes * * * *