3GPP TSG-WG SA2 Meeting #154AH-e 	S2-2300319r01

# E-meeting, January 16–20, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-25 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI #1: Update to address ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CR intends to address and remove some Editor’s Notes in the Conclusion of KI #1. Editor’s Notes: Editor's note: The details of which existing/new SIB information the UE uses to determine that the RAN timing synchronization status has changed is FFS and will be coordinated with RAN WGs. Based on LS from RAN2 (R2-2213048), (i) the size of the SIB is important and it is good to keep the new information size reasonable, (ii) multiple UEs going to connected state at the same time should be avoided. On the one hand, Alternatives 1.3 and 1.4 in Annex 1 would require including all the set of status parameters in the SIB. Depending on the size of this set, it might increase the size of the SIB message. On the other hand, Alternatives 1.1 and 1.2 provide just a small set of parameters to help the UE decide whether it needs to go to connected state to receive the actual status report (Report ID=Cell group ID, Event ID, or timestamp). However, Alternatives 1.1(a) and 1.2 may generate the problem of multiple UEs going to connected state simultaneously. Alternative 1.1(b) minimizes this issue since there is a list of status characteristics at UEs that are predefined or standardized. However, predefined and standardized information implies it is known and could be misused is ways that could hinder the security and interfere with the thought business model. Alternative 1.3 openly exposes the status to every UE. Alternative 1.4 cyphers the information in the SIB message but implies additional complexity. In order to overcome the issues present in Alternative 1.1(b), it is proposed Alternative 1.1(c) where the network provides the list the characteristics (or codebook) to the UE in a dedicated manner. TSCTSF may be responsible of generating the codebooks which can be provided to gNB via OAM (preferred) or via NGAP, and then gNB delivers the codebook to the UE when it is registered via dedicated RRC message. Note that in both 1.1(b) and 1.1(c) an error handling is necessary when the provided index to the set of time sync characteristics is does not exist at the UE. In such case the UE must transition to Connected state. Editor's note: Providing RAN timing synchronization status information to the UE in RRC_Idle and RRC_Inactive state is FFS. It is relevant to note that the LS from RAN2 (R2-2213048) states: “in connected mode the UE can be provided with more accurate information”. This implies that a consideration is to be made when providing the service for UEs in Idle or Inactive states especially when the requirements on accuracy (error budget) is very tight and when the service is offered publicly. Moreover, since the use of SIB messages is susceptible to malicious insertions and Report ID may be replaced, many UEs may simultaneously and unnecessary transition to Connected state. Therefore a Note with the above statements is added and the corresponding Editor’s Note is removed. Editor's note: Which clock quality metrics (e.g. clock accuracy, traceability to UTC, frequency stability) will be provided to the UE and will be used to determine whether the clock quality is acceptable or not is FFS. It is proposed to use a short set of parameters that sufficiently indicate the UE about the status of the time synchronization service: synchronization state ("Locked”, “Holdover”, “Freerun”) [optional], source type (e.g, “PTP”, “GNSS”, “other”) [optional], clock quality descriptor (e.g., “clockClass”, “ClockAccuracy”, “gnss-rx-time-error”) [mandatory]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the following Editor’s Note and adding a proposal to deliver information to the UE using SIB in Annex A, and adding it to the conclusion in clause 8.5: Editor's note: The details of which existing/new SIB information the UE uses to determine that the RAN timing synchronization status has changed is FFS and will be coordinated with RAN WGs. Removing the following Editor’s Note and adding Note to indicate that UEs in Connected mode can be provided with more accurate timing information, and that there are risks to consider Idle or Inactive UE states given that SIB messages are unprotected and broadcasted. Editor's note: Providing RAN timing synchronization status information to the UE in RRC_Idle and RRC_Inactive state is FFS. Removing the following Editor’s Note and adding a proposal on the needed clock quality parameters to be provided to UE, or to be used by RAN to determine whether the clock quality is acceptable or not. Editor's note: Which clock quality metrics (e.g. clock accuracy, traceability to UTC, frequency stability) will be provided to the UE and will be used to determine whether the clock quality is acceptable or not is FFS. # ---r01--- Adding two new sub-clauses (A.1.1.1 and A.1.1.2) where we describe in details how codebook(s) are generated (as well as provisioned to the NG-RAN and UE) and can be used to inform UEs in RRC_IDLE/INACTIVE state about currently applicable time sync information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes remain unsolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5, A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

## 8.5	Conclusion for KI #1: 5GS network timing synchronization status and reporting

The following bullet points summarize the principles for the way forward:

-	Detecting and reporting RAN and UPF timing synchronization status to TSCTSF.

-	NG-RAN and UPF/NW-TT can detect timing synchronization degradation/failure/improvement locally.

NOTE 1:	The detection is performed based on information provided by time synchronization protocols used in the transport network for both RAN and UPF, or, in the case of NG-RAN, using information provided by a local GNSS receiver. However, in any case, the details on how exactly NG-RAN/UPF detects timing synchronization degradation/failure/improvement locally are beyond the scope of 3GPP.

Two options are defined for the TSCTSF to detect the timing synchronization status information of RAN and UPF/NW-TT:

1)	TSCTSF may receive network timing synchronization status information of RAN and UPF/NW-TT directly from OAM.

2)	Alternatively, TSCTSF may receive network timing synchronization status information of RAN and UPF/NW-TT using control plane signalling at node level:

-	For UPF/NW-TT case the TSCTSF may use UMIC.

-	For NG-RAN case the TSCTSF may obtain NG-RAN network timing synchronization status information via the AMF (i.e. AMF uses NGAP signalling to configure the NG-RAN reporting).

The network timing synchronization status information from RAN or UPF/NW-TT can contain the following parameters: node's synchronization state, node's synchronization performance, primary source description, and primary source event.

Editor's note:	How to support additional methods (NGAP, control plane signalling, etc.) to obtain timing synchronization status from NG-RAN requires RAN feedback.

-	UE determining that the RAN timing synchronization status changed using:

-	SIB broadcast information to enable UEs in RRC_IDLE and RRC_INACTIVE and in the case of RRC_CONNECTED UEs, dedicated RRC signalling, to enable UEs to determine that:

-	the timing synchronization status of the cell that the UE is camping on has changed;

-	the timing synchronization status of the new cell the UE is camping on after cell reselection is different compared to the timing synchronization status of the cell that the UE was previously camping on.

-	If the UE has determined that the RAN timing synchronization status has changed and the UE has been requested by the TSCTSF to connect to the network in the case the RAN timing synchronization status changes, the UE performs a registration (if the UE is in RRC_IDLE) or the UE Triggered Connection Resume in RRC Inactive procedure (if the UE is in RRC_INACTIVE).

-	Information to be broadcasted via SIB to the UEs in RRC_IDLE or RRC_INACTIVE consists of a Report ID, which contains Cell Group ID and Event ID, according to Alternative 1.1c in clause A.1.1. This information included in the SIB is optional. Based on this information, the UE can determine its status using codebook(s) where a Report ID may be used as an index referring to the corresponding time synchronization status characteristics in the codebook. In case of error, the UE may require transitioning to RRC_CONNECTED in order to explicitly receive a status report via unicast RRC message (or a new codebook).

NOTE 2:	UEs in RRC_CONNECTED can be provided with more accurate service, given that propagation delay compensation methods can only be applied in RRC_CONNECTED state. The use of SIB messages for UEs in RRC_IDLE and RRC_CONNECTED is susceptible to malicious insertions that can change the behavior of the UE, either by multiple UEs transitioning simultaneously and unnecessarily into RRC_CONNECTED or by receiving wrong status information. Therefore, it is up to the network operator to keep UEs which require time synchronization status reports always in RRC_CONNECTED or to limit the case to local/private threat-free scenarios.

-	Providing RAN timing synchronization status information to the UE in RRC_Connected state:

-	If a UE is subscribed for Access Stratum Time Synchronization (ASTI) in the UDM (see clause 8.6), then the "Access and Mobility Subscription data" may additionally contain the following clock quality reporting control information:

-	Clock quality detail level: indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication;

-	Clock quality acceptance criteria for the UE (if the clock quality level equals "acceptable/not acceptable indication": the clock quality acceptance criteria for the UE (e.g. acceptable clock accuracy, acceptable frequency stability, etc.).

NOTE 3:	Whether and which clock quality information to provide to the UE depends on the needs of the time service consumer (referred to as client network operator hereafter). Therefore, the clock quality detail level and clock quality acceptance criteria are based on the parameters and their values specified in the agreement between the 5G network operator and the client network operator. The clock quality acceptance criteria refer to the quality with which 5G access stratum time needs to be delivered to and received by the UE (i.e. also considering propagation delays). Additional inaccuracies in the UE, e.g. if the 5G access stratum time is delivered to devices attached to the UE, are not included in the clock quality acceptance criteria because they are assumed to be budgeted by the client network operator when agreeing the required clock accuracy with the 5G network operator.

-	If an AF requests Access Stratum Time Synchronization (ASTI) for a UE, then the AF may provide clock quality reporting control information to TSCTSF. TSCTSF provides the clock quality reporting control information to AMF.

-	When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to NG-RAN, AMF also includes the clock quality reporting control information.

-	Based on the clock quality reporting control information received from AMF, RAN reports its timing synchronization status to the UE using unicast RRC:

-	If clock quality detail level is set to "clock quality metrics", then the RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. Clock quality metrics refers to information such as clock accuracy, traceability to UTC, frequency stability, etc.

-	If clock quality detail level is set to "acceptable/not acceptable indication", then the RAN provides an acceptable indication to the UE if the RAN's timing synchronization status matches the acceptance criteria received from AMF; otherwise RAN indicates "not acceptable" to the UE.

- The UE will be provided with the following clock quality metrics: synchronization state ("Locked”, “Holdover”, “Freerun”) [optional], source type (e.g, “PTP”, “GNSS”, “other”) [optional], clock quality descriptor (e.g., “clockClass”, “ClockAccuracy”, “gnss-rx-time-error”) [mandatory], or these parameters may also be used to determine whether the clock quality is acceptable. For the latter case, NG-RAN transfers this status (acceptable/not acceptable) to the UE.

-	When determining the clock quality metrics for a UE and when determining whether clock quality is acceptable or not acceptable for a UE, RAN considers whether propagation delay compensation is performed.

NOTE 4:	Clock quality metrics and the acceptable/not acceptable indication refer to the quality with which 5G access stratum time is delivered to and received by the UE (i.e. also considering propagation delays). In addition, the UE can, for example, update clock quality metrics to reflect internal inaccuracies in the UE before providing the clock quality metrics to devices connected to the UE.

-	Determining UEs impacted by RAN timing synchronization status degradation/improvement:

-	TSCTSF subscribes to receive notifications for UE presence in Area of Interest information (Area of Interest is set to a list of RAN node IDs that have the same RAN timing synchronization status) from AMF for UEs that AF requested time synchronization for or which are configured for (g)PTP-based time synchronization based on subscription.

-	When activating time synchronization for a UE, TSCTSF requests the UE to connect to the network via AMF (i.e. to perform a registration if the UE is in RRC_IDLE or the UE Triggered Connection Resume in RRC Inactive (if the UE is in RRC_INACTIVE) in the case when the UE later detects that the RAN timing synchronization status has changed while the UE is in RRC_IDLE or RRC_INACTIVE.

-	TSCTSF correlates information about impacted RAN nodes and the UE location information received from AMF to determine the UEs impacted by RAN timing status degradation/failure/improvement.

Editor's note:	Whether alternatively NG-RAN can be responsible for determining the impacted UE(s) and sending the NG-RAN timing synchronization status reports to the AMF via NG-AP signalling, together with the impacted UE(s) is FFS.

-	Determining UEs impacted by UPF timing synchronization status degradation or improvement (only for the case when UPF/NW-TT is involved in providing time information to DS-TT):

-	TSCTSF determines the UEs for which an impacted UPF/NW-TT is configured to send (g)PTP messages.

-	Informing AFs about network timing synchronization status degradation or improvement:

-	If TSCTSF has determined UEs impacted by RAN or UPF timing synchronization status degradation or improvement or failure then TSCTSF informs the AF about the timing synchronization status for those UEs if the AF was the requester of the time synchronization service.

-	The AF may subscribe to time synchronization service status for a UE (or group of UEs) for which the AF requests or has requested time synchronization service (for ASTI or (g)PTP services).

-	For the subscribed AFs the TSCTSF provides time synchronization service status.

-	The TSCTSF may perform the following:

-	For AFs that requested ASTI service, the TSCTSF may indicate whether it can support the ASTI service or not as per the requested criteria.

-	For AFs that requested PTP service, the TSCTSF may indicate whether it can support the PTP service or not as per the requested criteria.

-	For AFs that subscribe for ASTI/PTP service status update (i.e. change in support status), the TSCTSF may provide notification towards the AF when there is a change in support status.

-	Deactivating/reactivating/updating time synchronization services based on RAN/UPF timing synchronization status changes:

-	PTP case: For UEs that are part of a PTP instance and which are impacted by RAN or UPF time synchronization status degradation or improvement:

-	If TSCTSF determines that the Time synchronization error budget provided by AF can still be met, then TSCTSF may update the clockQuality information sent in Announce messages (see clause 7.6.2 of IEEE 1588 [8]) for the PTP instance using existing procedures and existing PMIC/UMIC information. The handling of Announce messages follows existing procedures as described in TS 23.501 [2].

-	If TSCTSF determines that the Time synchronization error budget provided by AF cannot be met (see above) then TSCTSF informs the AF about the intention to temporarily remove the UE/DS-TT from the PTP instance and performs the action using existing procedures in clause K.2.2.1 and clause K.2.2.4 of TS 23.501 [2]) after receiving the confirmation. If the AF declines the intention, the TSCTSF keeps the service active.

-	If TSCTSF determines that the Time synchronization error budget provided by AF can be met again then TSCTSF adds the DS-TT PTP port to the PTP instance again and also re-activates the Grandmaster functionality.

-	ASTI case: TSCTSF updates the access stratum time distribution indication to "enable" or "disable" and forwards the attribute to the serving NG-RAN nodes for the impacted UEs via AMF depending on whether the Time synchronization error budget can or cannot be met (following Rel-17 operations as described in clause 4.15.9.4 of TS 23.502 [3]). However, before updating the access stratum time distribution indication from “enable” to “disable”, the TSCTSF needs to inform the AF (if the ASTI service was activated based on the AF request) about the intention and receive the confirmation; otherwise, the TSCTSF keeps the indication unchanged.

>>>> Next Change <<<<

## A.1.1	Alternative 1: gNB provides a reference report ID within SIB

In this alternative when there is a new RAN timing synchronization status report available at the gNB, the gNB includes in the SIB a status report ID as a notification for the UEs reading the SIB. The report ID can be an optional integer information element. This report ID enables the UE to know there is new information available at the NG-RAN that is not available locally at the UE. There are three options for the UE to determine RAN timing synchronization status information with the report ID:

a)	The UE can actively retrieve the RAN timing synchronization status information from the network by entering RRC_Connected. In order to determine if a report ID is associated to a new report, the UE uses status report ID and the SIB information to identify the serving gNB in the cell. For report ID composition, the report ID is constructed from a pre-agreed (known values at the UE and network side) set of values. The report ID is constructed from a cell group ID and event ID elements:

-	Cell group ID is an integer allocated by the gNB that identifies a group of cells controlled by the same gNB.

-	Event ID is an integer value.

b)	Report ID is an index that maps to a pre-defined and/or standardized time synchronization characteristics thus the UE can automatically determine this without having to move to RRC_CONNECTED state. The report ID is composed by one integer which values are standardized or operator defined that are known at the UE and the NG-RAN node.

To limit the possible permutations of report IDs, in addition to the report ID mapping to time synchronization characteristics, the UEs or AFs may receive additional time synchronization characteristics via SLA or dedicated signalling. The decision depends on the time synchronization characteristics that should be considered. For example, the following parameters can be considered: Lock state, Parent Time Source, Clock class, Clock stability, Clock identifier, Physical layer frequency availability, Holdover specification

c)	Report ID is an index that maps to a set of time synchronization characteristics which is part of a list or codebook. This set of time synchronization characteristics/parameters indicates the currently applicable time synchronization status information. The codebook(s) are provisioned to the NG-RAN and UE and after receiving a codebook (or indication about the codebook) and Report ID (i.e., an index in the codebook) in SIB9, the UE in RRC_IDLE/INACTIVE state can determine the reported time synchronization status information without having to move to RRC_CONNECTED state. The codebook generation as well as its provisioning to NG-RAN and UE are described in clause A.1.1.1. How a codebook is generated is up to implementation. The codebooks are sent to UEs that have subscription to the time synchronization service during registration.

How NG-RAN reports currently applicable time synchronization status information to the UE with respect to UE’s subscription (i.e., considering Clock Quality Reporting Control Information and “Clock Quality Detail Level” in particular) is described in clause A.1.1.2.

An overall procedure for SIB including a reference report ID is illustrated in Figure A.1.1-1.

UE/DS-TT

UE has already access stratum time distribution configured

1. SIB9 or RRC (with codebook for Alt c).)

2. Primary source event

For alternative a)

4. SIB                    (RTI, status report ID)

3. gNB generates RAN timing synchronization status report and report ID

6. UE reads status report ID

7. UE enters RRC_CONNECTED state

9. DLInformationTransfer        (RTI, RAN timing sycnrhonization status report, report ID)

NG-RAN

AMF

TSCTSF

The RAN releases the UE

5. SIB                    (RTI, status report ID)

8. gNB realizes the UE is subscribed to RAN timing synchronization status

10. For alternative b) the UE uses the report ID as an index to determine the RAN timing sycnrhonization status report

11. For alternative c) the UE uses the Report ID as an index to determine the RAN timing sycnrhonization status report from a codebook at the UE

UE/DS-TT

UE has already access stratum time distribution configured

1. SIB9 or RRC (with codebook for Alt c).)

2. Primary source event

For alternative a)

4. SIB                    (RTI, status report ID)

3. gNB generates RAN timing synchronization status report and report ID

6. UE reads status report ID

7. UE enters RRC_CONNECTED state

9. DLInformationTransfer        (RTI, RAN timing sycnrhonization status report, report ID)

NG-RAN

AMF

TSCTSF

The RAN releases the UE

5. SIB                    (RTI, status report ID)

8. gNB realizes the UE is subscribed to RAN timing synchronization status

10. For alternative b) the UE uses the report ID as an index to determine the RAN timing sycnrhonization status report

11. For alternative c) the UE uses the Report ID as an index to determine the RAN timing sycnrhonization status report from a codebook at the UE

Figure A.1.1-1: Procedure for gNB provisioning status report ID in SIB

1.	The UE has received reference time information using unicast RRC or SIB9. For alternative c), the unicast RRC message may contain a codebook and a reference ID for the codebook. The RAN releases the UE to RRC Inactive or RRC Idle state.

2.	The NG-RAN node detects a primary source event (e.g. degradation, failure, recovery).

3.	The NG-RAN generates a RAN timing synchronization status report and an associated status report ID.

4-5.	The NG-RAN node broadcasts a status report ID in the cell using SIB to notify the primary source event to the UEs camping in the cell.

6.	The UE reads SIB and the status report ID and:

-	Alternative a), if the UE does not have stored locally the RAN timing synchronization status report corresponding to the status report ID, the UE retrieves a new RAN timing synchronization status report corresponding to the status report ID the NG-RAN Otherwise, the UE uses the locally stored RAN timing synchronization status report and steps 7-9 are skipped; or

-	Alternative b), the UE uses the status report ID as an index to map to the pre-defined and/or standardized characteristics. Steps 7-9 are skipped.

-	Alternative c), the UE uses the status Report ID as an index to map to the codebook. Steps 7-9 are skipped.

7.	In the case of alternative a), or if index was not found for alternative b) in step 10 and for alternative c) in step 11, the UE enters RRC_CONNECTED.

8.	In the case of alternative a), or if index was not found for alternative b) in step 10 or for alternative c) in step 11, after UE moves to RRC_CONNECTED mode, the NG-RAN determines the UE is subscribed to RAN timing synchronization status (e.g. based on configuration provided by the TSCTSF via AMF).

9.	In the case of alternative a), or if index was not found for alternative b) in step 10 or for alternative c) in step 11, the NG-RAN node sends the last available RAN timing synchronization status report with its associated status report ID to the UE via dedicated RRC signalling. The UE may store the RAN timing synchronization status report with the corresponding status report ID locally for a configured time or until deregistration, and thus avoid the need to reconnect with the network.

10.	In the case of alternative b) the UE uses the report ID as an index to map to the pre-defined and/or standardized characteristics that describe the RAN timing synchronization status. If the index is not found in pre-defined/standardized list of characteristics, then the UE performs steps 7-9.

11. In the case of alternative c) the UE uses the Event ID as index to map to a set of time synchronization characteristics in the codebook. If the index is not found in the codebook or if the codebook is not found or its validity has expired, then the UE will perform steps 5 in clause A.1.1.2.

## A.1.1.1.	Codebook(s) provisioning

Procedures for codebook(s) provisioning is as follows (see Figure A.1.1.1-1):

1.	Codebook(s) generation. Two approaches may be used to generate a time synchronization status information codebook or sets of codebooks.

1A.	Core network node (e.g., AF, TSCTSF or AMF) generates codebook(s) and for that, it may consider the following aspects:

    - NG-RAN and/or UPF node-level information about network’s time synchronization performance as reported to the TSCTSF directly via OAM or indirectly via AMF using NGAP signaling.

    - Configurations/parameters of the current time synchronization services managed by the core network node, e.g., Uu time synchronization error budget, UE subscription data, including “Clock Quality Reporting Control Information”, Spatial Validity Condition, Temporal Validity Condition and other parameters that may be provided for both (g)PTP and ASTI services (as per clause 4.15.9.3 and 4.15.9.4 of TS 23.502)

    - UE location information as codebooks may be generated per a tracking area or per a cell/group of cells.

    - NG-RAN node’s “Time Accuracy Class” which may be provided to the TSCTSF/AMF.

1B.	A set of codebook instances are deployment-specific, may rely on input from the AF requesting a (g)PTP and/or ASTI-based time synchronization service, and valid only in a particular area of the network.

2.	Providing codebook(s) to the NG-RAN.

2A.1	NG-RAN is provided with codebook(s) by the TSCTSF via AMF or directly by AMF.

2B.1	NG-RAN node is configured by OAM with all codebooks that can be referenced by the core network node (e.g., TSCTSF or AMF), and the core network node is aware of what codebooks are applicable to that NG-RAN, tracking area(s) or list(s) of cells.

2.2.	NG-RAN stores the codebook(s) or information on which codebook instance to apply.

Codebook(s) or indication about which codebook instance to apply may be signaled to NG-RAN via:

    - Non-UE associated signaling, e.g., during NG interface Setup/Reconfiguration procedure;

    - UE associated signaling, e.g. during Initial Context Setup/Modification procedure or PDU session resource management procedures;

    - Paging procedure.

3.	Providing codebook(s) to the UE (options):.

3A.1	NG-RAN sends the codebook(s) to the UE with the corresponding subscription to time synchronization services using the RRC protocol.

3B.1	The codebook(s) are sent from a TSCTSF (via an AMF) or from the AMF using Non-Access Stratum (NAS) signaling, e.g. during registration procedure in the Registration Accept message.

3.2.	UE stores the received codebook(s) until new codebook(s) are provided or they are deleted, e.g. by explicit signaling from the network due to, e.g. invalidations of the Temporal Validity Condition, Spatial Validity Condition, service update request, etc.

4.	The Codebook(s) are provisioned to the NG-RAN and UE. The network may start reporting currently applicable time synchronization status information to UEs.

Figure A.1.1.1-1: Codebook(s) provisioning to the NG-RAN and the UE.

## A.1.1.2.	Indicating time synchronization status to the UE using the codebook approach

Figure A.1.1.2-1: Indicating currently applicable time synchronization status information to UEs in RRC_IDLE/INACTIVE state.

Procedure for reporting currently applicable time synchronization status information to the UE in RRC_IDLE and RRC_INACTIVE states using the codebook approach is as described in Figure A.1.1.2-1:

0.	Prerequisites for time synchronization status reporting to UE:

0.1.	UE has already an ASTI-based time synchronization service configured

0.2.	NG-RAN is provisioned with time synchronization status information codebook(s) and/or information about which codebook instance(s) to apply, e.g. in specific TA, list(s) of cells, etc.

0.3.	UE with the corresponding subscription is provisioned with time synchronization status information codebook(s). The provisioning happens while a UE is in RRC_CONNECTED state.

0.4.	UE transits to RRC_IDLE/INACTIVE state

NOTE:	The codebook approach applies to UEs in RRC_IDLE and RRC_INACTIVE states; UEs in RRC_CONNECTED states may get informed about currently applicable time synchronization status using dedicated RRC messages.

1.	Determining changes in time synchronization status and report

1.1.	NG-RAN and/or UPF detects a time synchronization degradation/failure/improvement event and reports the node-level related information to the TSCTSF as described in clause 8.5.

1.2.	Determining UEs with ongoing time synchronization services that shall be informed about changes in time synchronization status or provided with new characteristics/parameters describing currently applicable time synchronization status (as described in clause 8.5).

2.	(If the Clock Quality Detail Level for the UE is set to “Clock Quality Metrics”): NG-RAN identifies a codebook instance if multiple codebooks are provisioned (e.g. per a tracking area, per a list of cells, or per UE subscription level) and determines an index in the codebook with the corresponding time synchronization status characteristics to be provided to UE.

(If the Clock Quality Detail Level for the UE is set to “Acceptable/Not Acceptable” indication): either

  - NG-RAN will use an index different from the previously reported and if there is a change then the UE will transit into RRC_CONNECTED state in step 3 to receive the actual status report (in the form acceptable/not acceptable as applicable for this UE), or,

  - NG-RAN will use an index from the emended codebook that has been modified by the NG-RAN in step 0.2 and provisioned to the UE in step 0.3 and whose indices indicate their corresponding value: “Acceptable” or “Not Acceptable”.

3.	NG-RAN sends SIB9 with an index indicating the appropriate information in the codebook and, if multiple codebooks are provisioned (per TA, list(s) of cells, “Clock Quality Detail Level”), codebook instance identifier.

4.	The UE (while being in RRC_IDLE/INACTIVE) determines the reported time synchronization status and/or it needs to transit to RRC_CONNECTED state in order to receive the status information (in step 5) and/or new time synchronization status information codebook(s) (in step 6-7).

5.	UE transits to RRC_CONNECTED state:

  - If the UE determines in step 2, it needs to move to RRC_CONNECTED in order to get information about changes in time synchronization status

  - If the UE moves between tracking areas and another codebook is applicable in another TA.

  - If the validity period of its stored codebook expires.

  - If an unknown codebook instance ID or an index is received in SIB9.

6.	NG-RAN is re-provisioned with new codebook(s).

7.	UE is re-provisioned with new codebook(s).

>>>> End of Changes <<<<