3GPP TSG SA WG2 Meeting SA2#154-AH-e		S2-2300322r03

January 16 – 20, 2023, E-meeting, Elbonia

Title:	[DRAFT] Proposed method for Time Synchronization status reporting to UE(s)

Release:	Rel-18

Work Item:	FS_TRS_URLLC

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact person: 	 TBD

Attachment: 	CR#0002 for TR 23.700-25

# 1	Overall description

SA2 has agreed the attached CR as update to conclusion for the key issue #1 “5GS network timing synchronization status and reporting”.

With respect to informing UEs in RRC_Inactive/Idle about a change of the RAN clock quality, SA2 has endorsed the following:

-	The gNB includes in SIB9 a reference report ID as a notification for the UEs reading the SIB9 that there is new clock quality information available. The UE compares the reference report ID with locally stored reference report ID to determine if it had retrieved the last available clock quality information already.

-	The reference report ID consists of the scope of the report ID and an Event ID (an integer). Scope may either identify a group of cells within a single gNB or a group of cells across gNBs. The latter would reduce the amount of signalling even further since then UEs that move to another gNB would not need to retrieve the clock quality details.

SA2 question: SA2 would like to kindly request RAN2 and RAN3 to provide feedback whether both scopes (group of cells per gNB, group of cells across gNBs) can be beneficial and supported.

With respect to the point raised by RAN2 in the LS response on 5GS time synchronization status report towards the UE(s) in R2-2213048 ("If dedicated information is required, from RAN2 perspective, it would be good to avoid all UEs going to connected at the same time."), SA2 has concluded the following:

From SA2 perspective, it is not required that the UE always transitions to RRC_CONNECTED immediately to retrieve the latest available clock quality information. In order to reduce RACH access from many UE(s) (to move back to RRC_CONNECTED state) at the same time, the following option has been considered pending RAN2 feedback:

It can be required that RAN randomizes the UE(s) re-connecting back to the network, i.e., to spread the UEs' connection attempts in the time domain, e.g., over the course of (1) few seconds (2) one minute, (3) several minutes. RAN2 is expected to determine whether and how this can be achieved.

SA2 question: SA2 would like to kindly request RAN2 to provide feedback whether this approach is feasible.

With respect to providing

RAN’s latest clock quality information to the UE in RRC_Connected state SA2 has concluded the following:

If a UE is subscribed for Access Stratum Time Synchronization (ASTI) in the UDM (see clause 8.6), then the "Access and Mobility Subscription data" may additionally contain the following clock quality reporting control information:

C	lock quality detail level: indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication;

Clock quality acceptance criteria for the UE (if the clock quality level equals "acceptable/not acceptable indication": the clock quality acceptance criteria for the UE. Acceptance criteria can be defined based on the following attributes: time source, traceability to UTC or GNSS, synchronization state, clock accuracy, clock class, frequency stability.

Attributes that can be used for clock quality acceptance criteria depends on RAN capabilities to provide them and pending RAN WGs feedback.

SA2 question: SA2 would like to kindly request RAN3 to provide feedback whether the following attributes are available in RAN: time source, traceability to UTC or GNSS, synchronization state, clock accuracy, clock frequency stability.

# 2	Actions

To RAN2, RAN3

ACTION: 	SA2 kindly asks for feedback as requested above.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting #155   Feb 20 – 24, 2023 	Athens, Greece

TSG SA WG2 Meeting #156E   April 17 -21, 2023		Online