SA WG2 Meeting #154-AH-e	S2-2300335r03

E-meeting,  2023-01-16 – 2023-01-20

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3875 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of support for Jitter Measurement and End of Data Burst reporting to the NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following the conclusions for XRM work item in TR 23.700-60, the 5GS must be enhanced to support jitter measurement and End of Data Burst reporting to the NG-RAN |  |  |  |  |  |  |  |  |
| Summary of change: |  | New text added to 23.501 to describe the enhancements required to support the measurement and reporting of jitter information and End of Data Burst to the NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Jitter Measurement and End of Data Burst reporting to the NG-RAN are not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.11, 5.8.2.11.12, 5.27.2.1, 5.4.X.1 (new), 5.4.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

##### 5.8.2.11.11	Session Reporting Rule

The following table describes the Session Reporting Rule (SRR) that defines the detection and reporting events that the UPF shall report, that are not related to specific PDRs of the PDU Session, as follows:

-	Per QoS Flow per UE QoS Monitoring Report, as specified in clause 5.33.3.2.

-	Change of 3GPP or non-3GPP access availability, for an MA PDU session.

-    Per PDR Jitter Measurement Report.

Table 5.8.2.11.11-1: Attributes within Session Reporting Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this SRR. |  |
| Rule ID | Unique identifier to identify this information. | Used by UPF when reporting. |
| QoS Monitoring per QoS Flow Control Information | Indicates the UPF to apply the QoS Monitoring report for one or more QoS Flows. | The IE is defined in clause 7.5.2.9 of the TS 29.244 [65]. See NOTE 1. |
| Access Availability Control Information | Indicates the UPF to report when an access type becomes available or unavailable for an MA PDU Session. | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. |
| Jitter Measurement Control Information | Indicates the UPF to report jitter information for a Periodicity for one or more PDR. | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. |
| NOTE 1: The QoS Monitoring per QoS Flow Control Information may contain an Indication of local event notification. The Indication of local event notification includes a Notification Target Address that identifies the recipient of the information being notified by the UPF (Local NEF/AF). The Indication of local event notification also indicates that the UPF reports the information via Nupf_EventExposure_Notify service operation. |  |  |

##### 5.8.2.11.12	Session reporting generated by UPF

The UPF sends the session report to inform the SMF the detected events for a PDU Session that are related to an SRR. The UPF may support notification to the AF possibly via local NEF as described in clause 6.4 of TS 23.548 [130].

Table 5.8.2.11.12-1: Attributes within Session Reporting

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to the SRR which triggered the report. |  |
| Rule ID | Unique identifier to identify the Session Reporting Rule within a session which triggered the report. | Used by UPF when reporting. |
| QoS Monitoring Report | Indicates the QoS Monitoring result for one or more QoS Flows. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |
| Access Availability Report | Indicates the change of 3GPP or non-3GPP access availability, for an MA PDU session. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |
| Jitter Measurement Report | Indicates the UPF to jitter measurement result for a Periodicity for one or more PDR. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |

## NEXT CHANGE

##### 5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. | NOTE 9. |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included and the action can also indicate that a notification of the first buffered and/or a notification of first discarded packet is requested (see clause 5.8.3.2). For drop action, a notification of the discarded packet may be requested (see clause 5.8.3.2). |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). | NOTE 8. |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN), or to 5G VN internal (i.e. local switch). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI) (NOTE 8). Any extension header stored for this packet shall be added. The time stamps should be added in the GTP-U header if QoS Monitoring is enabled for the traffic corresponding to the PDR(s). An End of Data Burst indication should be added to the last PDU of each Data Burst in the GTP-U header for UE power management enhancement. See clause 5.4.X.2. |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. | NOTE 8. |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains one of the following policies identified by a TSP ID: - an N6-LAN steering policy to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator, or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7. or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and N19 forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The TSP ID action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for UL/DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 of TS 29.244 [65]. NOTE 7: The use of "5G VN internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. NOTE 8: When in architectures defined in clause 5.34, a FAR is sent over N16a from SMF to I-SMF, the FAR sent by the SMF may indicate that the I-SMF is to locally determine the value of this attribute in order to build the N4 FAR rule sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 9: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. |  |  |

## NEXT CHANGE

### 5.27.2	TSC Assistance Information (TSCAI) and TSC Assistance Container (TSCAC)

#### 5.27.2.1	General

TSC Assistance Information (TSCAI) is defined in Table 5.27.2-1 and describes TSC traffic characteristics for use in the 5G System. TSCAI may be used by the 5G-AN, if provided by SMF. The knowledge of TSC traffic pattern is useful for 5G-AN as it allows more efficiently scheduling of QoS Flows that have a periodic, deterministic traffic characteristics either via Configured Grants, Semi-Persistent Scheduling or with Dynamic Grants.

The TSCTSF determines the TSC Assistance Container (defined in Table 5.27.2-2) based on information provided by an AF/NEF as described in clause 5.27.2.3 and provides it to the PCF for IP type and Ethernet type PDU Sessions. In the case of integration with IEEE TSN network, the TSN AF determines TSC Assistance Container as described in clause 5.27.2.2 and provides it to the PCF for Ethernet PDU Sessions. The PCF receives the TSC Assistance Container from the TSCTSF or the TSN AF and forwards it to the SMF as part of PCC rule as described in clause 6.1.3.23a of TS 23.503 [45].

The SMF binds a PCC rule with a TSC Assistance Container to a QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. The SMF uses the TSC Assistance Container to derive the TSCAI for that QoS Flow and sends the derived TSCAI to the NG-RAN. The Periodicity, Burst Arrival Time, and Survival Time components of the TSCAI are specified by the SMF with respect to the 5G clock. The SMF is responsible for mapping the Burst Arrival Time and Periodicity from an external clock (when available) to the 5G clock based on the time offset and cumulative rateRatio (when available) between the external clock time and 5GS time as measured and reported by the UPF. The SMF determines the TSCAI as described in clause 5.27.2.4.

A Survival Time, which indicates the time period an application can survive without any data burst, may be provided by TSN AF/AF either in terms of maximum number of messages (message is equivalent to all packets of a data burst) or in terms of time units. Only a single data burst is expected within a single time period referred to as the periodicity.

The SMF may send an update of the TSCAI to the NG-RAN as defined in clauses 4.3.3.2, 4.9.1.2.2 and 4.9.1.3.2 of TS 23.502 [3].

Table 5.27.2-1: TSC Assistance Information (TSCAI)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts, or between two consecutive time instances where a silent traffic period ends and an active traffic starts. |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Survival Time (optional) | Survival Time, as defined in TS 22.261 [2], refers to the time period an application can survive without any data burst. |
| Burst Arrival Time Window (BAT Window) (optional) (NOTE 1) (NOTE 2) | Indicates the acceptable earliest and latest arrival time of the first packet of the data burst at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Capability for BAT adaptation (optional) (NOTE 1) | Indicates that the AF will adjust the burst sending time according to the network provided Burst Arrival Time offset (see clause 5.27.2.5). |
| Jiiter Information (optional) | Jitter information associated with the Periodicity (see clause 5.4.X.1) |
| NOTE 1: Only one of the parameters (BAT Window or Capability for BAT adaptation) can be provided. NOTE 2: The parameter will only be provided together with Burst Arrival Time. |  |

Editor's note:	The details of the Jitter Information are FFS and will be coordinated with RAN WGs.

Table 5.27.2-2: TSC Assistance Container (TSCAC)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The time when the first packet of the data burst arrives at the ingress port of 5GS for a given flow direction (DS-TT for uplink, NW-TT for downlink). |
| Survival Time (optional) | It refers to the time period an application can survive without any data burst, as defined in TS 22.261 [2]. |
| Time Domain (optional) | The (g)PTP domain of the TSC flow. |
| Burst Arrival Time Window (BAT Window) (optional) (NOTE 1) (NOTE 2) | Indicates the acceptable earliest and latest arrival time of the first packet the data burst at the ingress port of 5GS for a given flow direction (DS-TT for uplink, NW-TT for downlink). |
| Capability for BAT adaptation (optional) (NOTE 1) | It indicates that the AF will adjust the burst sending time according to the network provided Burst Arrival Time offset (see clause 5.27.2.5). |
| NOTE 1: Only one of the parameters (BAT Window or Capability for BAT adaptation) can be provided. NOTE 2: The parameter will only be provided together with Burst Arrival Time. |  |

## NEXT CHANGE

## 5.4	3GPP access specific aspects

### 5.4.X	UE power management enhancements

#### 5.4.X.1	Periodicity and jitter for periodic traffic

To enhance UE power management schemes like connected mode DRX the NG-RAN may be provided with information on traffic periodicity and associated jitter measurements collected by the SMF from the UPF.

The PCF may provision periodicity and optionally default jitter values within PCC Rules using the information provided by the AF and/or using local operator policies, and an indication to perform jitter measurements.

When SMF receives PCC rules with an indication to perform jitter measurements, it binds the PCC rule to a QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45].

SMF then forwards the Periodicity and default jitter if available to the NG-RAN. The SMF also requests the UPF to report the Jitter information using the N4 Session Modification procedure, see clause 5.8.2.11.11.

At reception of Jitter measurements from the UPF, the SMF includes the Jitter information together with the associated Periodicity in the TSC Assistance Information and forwards it to the NG-RAN, see clauses 5.8.2.11.12 and 5.27.2.

NOTE Z1:	In order to prevent frequent updates from the UPF, the UPF sends the Jitter Measurement Report periodically or only when the jitter is larger than a threshold.

#### 5.4.X.2	End of Data Burst Indication

To enhance UE power management schemes like connected mode DRX and extend UE battery lifetime, an indication of End of Data Burst may be provided to the NG-RAN.

The PCF can provision the following PDU Set Handling Information using the information provided by the AF and/or using local operator policies:

-	Service Protocol Description: Indicates header and payload type to be used for PDU Set Identification (e.g. RTP header extension, H.264/RTP, etc.)

The application server provides an indication of the End of Data Burst in the last PDU of the Data Burst. The UPF identifies that indication in the DL traffic based on the Description of Service Protocol and provides an End of Data Burst indication to the NG-RAN over GTP-U.

Editor's note: Support of new RTP header extension for End of Data Burst indication by the application server depends on progress in SA4 5G_RTP WI.

NOTE Z2:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence..

## END of CHANGES