SA WG2 Meeting #154-AH-E	S2-2300344r01

January 16 – 20, 2023, Elbonia

Source:	Ericsson

Title:	KI#1: conclusion proposals on subscriptions and PDU sessions

Document for:	Approval

Agenda Item:	9.20.1

Work Item / Release: 	FS_5WWC_Ph2

Abstract: This paper provides two principles on subscriptions and PDU sessions and assesses the solutions for KI#1 based on them.

# 1. Introduction

In the solutions for KI#1 in the TR, two types of subscription data for UEs and non-3GPP devices behind an RG have been considered

- “Independent subscriptions”: Regular subscriptions in UDM without any dependency on 5G-RG’s subscription. An independent subscription has a SUPI. These subscriptions are considered for UEs and AUN3 devices

- Data related to devices without “independent subscriptions”:   The information for each device could be stored in UDR, e.g., as policy data as part of the 5G-RG’s policy subscription, or as Application Data, and the device does not have an independent subscription/SUPI. This type of “subscription data” can be used for NAUN3 devices.

Furthermore, solutions for KI#1 either use “independent PDU Sessions” per each device or carry the devices’ traffic in the 5G-RG’s PDU session, which are here referred to as “dependent PDU Sessions”. This paper proposes two principles regarding subscriptions and PDU Sessions and assesses the solutions based on them. Sol#7 and 23 which focus on the QoS provisioning to the local network behind the RG are out of scope of this discussion.

# 2. Discussion

## 2.1	Principles

This section proposes two principles on device subscriptions and PDU Sessions, which may be dependent on or independent from the 5G-RG’s subscription and PDU session.

- Principle 1: Each PDU session is associated with only one SUPI / independent subscription

- Principle 2: Only devices/users with independent subscriptions may have their own Registration and PDU session / Devices with no “independent” subscription/SUPI need to be piggybacked on the RG’s Registration and PDU Session.

Principle 1 should not be violated since if multiple SUPIs share a single PDU session, their subscription should have compatible QoS parameters like session-AMBR and therefore, their subscriptions would be dependent. It also becomes complex for the SMF to enforce SM subscription data for multiple subscribers (SUPIs) on a single PDU Session.  Principle 2 is violated and devices without independent subscription/SUPI have their own Registration and PDU session, then it is unclear how authentication is performed, how they are charged etc. Therefore, devices with no “independent” subscription/SUPI need to be piggybacked on the RG’s Registration and PDU Session.

2.2	Discussion on solutions and principles

Solutions for KI#1 can be categorized in six classes based on the following assumption for devices behind 5G-RG:

- Devices have independent subscription or not

- Devices have independent or dependent Registration and PDU sessions

The table below shows these six categories of solutions.

Sol#6 proposes to have independent PDU session for NAUN3 devices, whose subscription are “virtual”. It is thus not clear how to charge these devices, unless their “virtual” subscriptions are related somehow to the 5G-RG’s subscription. Sol#6 thus violates Principle 2.

Sol#20 and 22 support AUN3 devices with independent subscriptions that use 5G-RG’s Registration and PDU Session, which thus violates Principle 1. It can be acknowledged that L3 Prose also allow multiple remote UEs (each with its own SUPI) to share the relay UE’s PDU Session, which is somehow a similar model as in Sol#20 and Sol#22. However, in the Prose case there is no separate charging or policy control for the remote UE’s traffic, and the level of complexity is thus considerably less compared to the 5WWC KI#1 case.

Table 1: solutions for KI#1 categorized in terms of subscription and PDU session types. The red cells violate one of the above-mentioned principles.

|  | Independent PDU session | Dependent PDU session |
| --- | --- | --- |
| Devices with independent subscription | Solutions for UEs: Sol#2, 5, 9, 20 Solutions for AUTN3 devices: Sol#3, 25 Solutions for NAUTN3 devices: | Solutions for AUTN3 devices: Sol#22, 20 |
| Devices without independent subscription | Solutions for NAUTN3 devices: Sol#6 | Solutions for AUTN3 devices: Sol# Solutions for NAUTN3 devices: Sol#8, 20, 21, |
| Devices with no subscription data | ----- | Solutions for NAUTN3 devices: Sol#1, 4,24 |

## 2.6. Conclusion

The following principles are assumed:

- 	Each Registration and PDU Session cannot be associated with more than one SUPI (Principle 1);

- 	Devices that don’t have “independent” subscription/SUPI should not have independent Registration or PDU sessions (Principle 2). I.e. Devices that don’t have “independent” subscription/SUPI need to be piggy-backed on the RG’s Registration and PDU Session.

Sol#6, 22 and 20 (for AUN3 devices) violate these principles and are not progressed for normative work.

## 2.7 	Proposed Evaluation

As per instructions from the rapporteur, evaluation proposals are included in the discussion part rather than the proposed changes (this section is therefore repeating parts of the above discussion):

The solutions can be categorized into six classes based on the following aspects:

- The solution includes

  - no device-specific subscription data in 5GC,

  - data related to devices without independent subscriptions independent device subscription.

- The solution includes

  - independent PDU sessions for each device

  - the device uses 5G- RG’s PDU sessions (“dependent PDU sessions”).

The solution categorization is presented in the following table:

Table 1: solutions for KI#1 categorized in terms of subscription and PDU session types

|  | Independent PDU session | Dependent PDU session |
| --- | --- | --- |
| Devices with independent subscription | Solutions for UEs: Sol#2, 5, 9, 20 Solutions for AUTN3 devices: Sol#3, 25 Solutions for NAUTN3 devices: | Solutions for AUTN3 devices: Sol#22, 20 |
| Devices without independent subscription | Solutions for NAUTN3 devices: Sol#6 | Solutions for AUTN3 devices: Sol# Solutions for NAUTN3 devices: Sol#8, 20, 21, |
| Devices with no subscription data | ----- | Solutions for NAUTN3 devices: Sol#1, 4,24 |

The Solutions in the gray cells either provide dependent PDU Sessions for independent subscriptions or independent PDU sessions for devices without independent subscription and are not desirable since:

- If multiple devices share a PDU session, their subscription should have compatible QoS parameters like session-AMBR and hence, their subscriptions should be dependent. Therefore, devices with independent subscriptions cannot have dependent PDU sessions.

- If devices without independent subscription have independent PDU sessions, then it is unclear how authentication is performed, how they are charged etc. Therefore, devices with no “independent” subscription/SUPI need to be piggybacked on the RG’s Registration and PDU Session.

# 3. Proposed changes

The following text is proposed for TR 23.700.17.

# * * * * First change * * *

## 8.1	Key Issue #1: Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG

As the technical issue are generally different for NAUN3 devices, AUN3 devices and UE(s), the conclusions are split up into 3 parts:

-	support of NAUN3 devices,

-	support of AUN3 devices,

-	support of UE(s).

This sub-clause addresses only common aspects:

-

The solution that is selected for the normative phase will not violate the following two principles:

- 	Principle 1: Each PDU session is associated with only one SUPI / independent subscription

- 	Principle 2: Only devices/users with independent subscriptions may have their own Registration and PDU session / Devices with no “independent” subscription/SUPI need to be piggybacked on the RG’s Registration and PDU Session.

-	Principle 3: A device can use a RG to access to 5GC only if this RG is itself duly registered

-	Principle 4: the network may be able to control the aggregate traffic of all the PDU Sessions using a wireline access

Here independent means that the subscription is independent from the 5G-RG’s subscription.

Solution 7 is endorsed for normative work (providing non-3gpp QoS assistance information for a Qos flow to the 5G RG) and applies for any kind of device with following clarifications including the NOTE(s) below:

-	this applies for 5G RG only

-	this may apply for both wireline and FWA access

-	non-3gpp QoS assistance information for a Qos flow may correspond to the same information that is contained in the Additional QoS Information (as specified in TS 23.502 [3] clause 4.12a.5 and in Table 9.3.1.1-2 of TS 24.502 [3]) by the SMF to the 5G-RG in NAS PDU session establishment and PDU session modification signalling; furthermore non-3gpp QoS assistance information for a Qos flow may also include ARP and Periodicity information.

NOTE: 	How 5G-RG uses the non-3gpp QoS assistance information to enforce QoS in non-3gpp networks is outside the scope of 3GP

NOTE: 	Transferring information like Periodicity to the 5G RG is not meant to support TSC/TSN like flows but to support consumer real time applications like XR (extended Reality , etc…)

-	SMF may additionally include a DSCP value in the non-3GPP QoS assistance information. The DSCP value may be used by the 5G-RG to derive e.g., IEEE 802.11 user-priority (UP) information in case the non-3GPP access is based on IEEE 802.11. The details of deriving information from DSCP are beyond the scope of 3GPP.

The following principles from solution 23 are endorsed for normative work:

-	5G-RG may initiate the UE requested PDU Session Modification procedure to provide a Requested Non-3GPP delay budget to the network. PCF creates/updates PCC rules based on the Requested Non-3GPP delay budget received from the SMF, e.g., by reducing the PDB for the related flows by the Requested Non-3GPP delay budget received from the SMF or by modifying the 5QI. The PCF may also alternatively apply a different Non-3GPP delay budget, based on operator policy. The accepted non-3GPP delay budget is provided to the 5G-RG.

NOTE:	The details of how to determine the non-3GPP delay budget are beyond the scope of 3GPP. It is assumed that the 5G-RG will limit the frequency of triggering the UE-initiated PDU Session Modification request to provide the non-3GPP delay budget to the network to avoid unnecessary signaling. It is up to CT1 to discuss to potentially introduce a timer to limit how often a 5G-RG is allowed to request a delay budget.

# * * * * End of changes * * * *