SA WG2 Meeting #S2-154AH-E	S2-2300368

16 – 20 Jan. 2023, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	MediaTek Inc.

Title:	KI#4: Update the conclusion

Document for:	Approval

Agenda Item: 	9.24.1

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes to update the conclusions on KI#4

# 1	Proposal

It is proposed to adopt the following proposal into TR 23.700-08.

# 1	Discussion

## 1.2	PLMN selection for PNI-NPN as a Hosting network

In the SA2#154, it has interim conclusion in clause 8.4.4 on the following:

-	For PNI-NPN as hosting network associated with CAG ID, the UE only considers an entry in the Allowed CAG list valid if and while all conditions (if there is any) for that entry are met. This may potentially initiate a new registration procedure to a PLMN

For the above conclusion on the last sentence that "This may potentially initiate a new registration to a PLMN" should be deleted as the reasons below:

-	No solution in the TR is proposed to change PLMN selection

-	the use cases for the UE to change current serving PLMN is not clear

Based on the above, the PLMN selection specified in TS 23.501 should not be impacted

Therefore, this conclusion should be revised as below:

-	For PNI-NPN as hosting network associated with CAG ID, the UE only considers an entry in the Allowed CAG list valid if and while all conditions (if there is any) for that entry are met. This may potentially initiate a new registration procedure to a PLMN

Proposal 2: based on the above, the conclusion should be revised as below:

-	For PNI-NPN as hosting network associated with CAG ID, the UE only considers an entry in the Allowed CAG list valid if and while all conditions (if there is any) for that entry are met. This may potentially initiate a new registration procedure to a PLMN

For below two NOTEs:

NOTE 1:	Whether a new network selection mode is required for UE to initiate hosting network selection is to be determined by CT WG1.

NOTE 2:	Details regarding priority list for hosting network selection, including if a new selection mode is required, is up to CT WG1 to decide.

The above two NOTEs should be only associated with an SNPN as hosting network. Therefore, they are revised as:

NOTE 1:	Whether a new network selection mode is required for UE to initiate hosting network selection for an SNPN (i.e. to select an SNPN as hosting network) is to be determined by CT WG1.

NOTE 2:	Details regarding priority list for hosting network selection for an SNPN (i.e. to select an SNPN as hosting network), including if a new selection mode is required, is up to CT WG1 to decide.

Proposal 3: based on the above, the conclusion should be revised as below:

NOTE 1:	Whether a new network selection mode is required for UE to initiate hosting network selection for an SNPN (i.e. to select an SNPN as hosting network) is to be determined by CT WG1.

NOTE 2:	Details regarding priority list for hosting network selection for an SNPN (i.e. to select an SNPN as hosting network), including if a new selection mode is required, is up to CT WG1 to decide.

For below NOTE, since SA1 has responded in LS (S1-223540): there is no requirement for the authorization for manual selection of a hosting network, the NOTE can be removed due to no requirements to do so from SA1’s response:

NOTE 3:	If SA WG1 requires that the home network authorization is needed before the UE performs manual selection of a hosting network, such authorization can be added during normative work.

Proposal 4: based on the SA1’s response, the NOTE can be removed directly

* * * * Start of First Changes * * * *

### 8.4.4	Conclusion for how the localized service information is used by UE

The following principles based on the evaluation in clause 7.4.4 are recommended for the normative work:

1.	If UE uses home network credential to access a hosting network:

a.	When the end user intends to access localized service and the validity conditions of localized service are met, the UE initiates hosting network selection using the hosting network selection information.

i.	For SNPN as hosting network, the UE can switch between PLMN selection and hosting network selection following Rel-17 specification for SNPN selection. How the UE switches among the network selections is up to UE implementation.

ii.	For PNI-NPN as hosting network associated with CAG ID, the UE only considers an entry in the Allowed CAG list valid if and while all conditions (if there is any) for that entry are met.

NOTE 1:	Whether a new network selection mode is required for UE to initiate hosting network selection for an SNPN (i.e. to select an SNPN as hosting network) is to be determined by CT WG1.

NOTE 2:	Details regarding priority list for hosting network selection for an SNPN (i.e. to select an SNPN as hosting network), including if a new selection mode is required, is up to CT WG1 to decide.

b.	Hosting network selection needs to be authorized by the home network, via UE initiated SoR procedure with SoR information including certain authorized criteria e.g. time. After the home network authorization, the UE is allowed to initiate hosting network selection, applicable for both automatic and manual hosting network selection.

i.	For manual hosting network selection, the UE presents available localized service information it has received to the end user.

NOTE 3:	If SA WG1 requires that the home network authorization is needed before the UE performs manual selection of a hosting network, such authorization can be added during normative work.

c.	When authorized criteria of the hosting network selection are no longer met, the UE stops hosting network selection.

2.	If the UE needs to obtain a new set of credentials/subscription to access the hosting network:

a.	It is up to UE implementation to decide how to switch to the new subscription profile for accessing hosting network.

3.	The UE determines SNPN access mode is activated/de-activated using implementation specific means as specified in existing Release 17, or using received localized service/hosting network assistance information as input.

* * * * End of Changes * * * *