3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300382r05

# E-meeting, January 16 - 20, 2023											 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0168 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization information about multi-path transmission for L2 Remote UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-33 clause 8.5 "Key Issue #5: Support of multi-path transmission for UE-to-Network Relay" includes the following conclusion: - For multi-path transmission via direct Uu path and Layer 2 U2N Relay UE: - The NG-RAN receives the authorization information indicating whether a UE is authorized to perform multi-path transmission via direct Uu path and Layer 2 U2N Relay from the AMF. The authorization information is determined by the AMF based on the related UE's 5G ProSe Capability (i.e. the UE supports 5G ProSe Layer-2 Remote UE defined in Rel-17) and the ProSe Service Authorisation included in the subscription data received from UDM. Normative work is needed for the above conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §4.3.4 - Add authorization information from AMF to NG-RAN about multi-path transmission via direct Uu path and L2 U2N Relay for L2 Remote UE. §5.7 - Add subscription data about multi-path transmission via direct Uu path and U2N Relay for a Remote UE. §6.6.2 - Add authorization information from AMF to NG-RAN about multi-path transmission via direct Uu path and L2 U2N Relay for L2 Remote UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description about authorization information from AMF to NG-RAN related to multi-path transmission for a Remote UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4, 4.3.5, 5.7, 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 4.3.4	AMF

In addition to the functions defined in TS 23.501 [4], the AMF performs the following functions:

-	Select a PCF supporting 5G ProSe Policy/Parameter provisioning based on indication of 5G ProSe Capability as part of the "5GMM capability" in the Registration Request.

-	Store the 5G ProSe Capability.

-	Forward the 5G ProSe Capability to PCF in Npcf_UEPolicyControl_Create Request.

-	Obtain from UDM the subscription information related to 5G ProSe and store them as part of the UE context data.

-	Obtain PC5 QoS parameters from the PCF and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about the following:

-	5G ProSe Direct Discovery and 5G ProSe Direct Communication (i.e. as 5G ProSe-enabled UE for ProSe Direct Discovery, as 5G ProSe-enabled UE for ProSe Direct Communication),

-	5G ProSe UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay),

-	Multi-path transmission via direct Uu path and 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	Provision the NG-RAN with PC5 QoS parameters related to 5G ProSe Direct Communication.

-	Optionally support security procedures over Control Plane for 5G ProSe UE-to-Network relaying as defined in TS 33.503 [29].

* * * * Start of Next Change * * * *

### 4.3.5	UDM

In addition to the functions defined in TS 23.501 [4], the UDM performs the following functions:

-	Subscription management for 5G ProSe Direct Discovery and Communication.

-	Subscription management for 5G ProSe UE-to-Network Relay Discovery and Communication.

-	Subscription management for multi-path transmission via direct Uu path and 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

Editor's Note:	It is FFS whether subscription management is needed for Layer-3 Remote UE's multi path transmission via direct Uu path and Layer 3 UE-to-Network Relay.

-	Subscription management for 5G ProSe UE-to-UE Relay Discovery and Communication.

* * * * Start of Next Change * * * *

## 5.7	Subscription to 5G ProSe

The subscription information in the UDM contains information to give the user permission to use 5G ProSe.

At any time, the operator can amend or remove the ProSe UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use 5G ProSe.

The following subscription information is defined for 5G ProSe:

-	subscription for open 5G ProSe Direct Discovery for NR PC5:

-	open 5G ProSe Direct Discovery Model A.

-	subscription for restricted 5G ProSe Direct Discovery for NR PC5:

-	restricted 5G ProSe Direct Discovery Model A;

-	restricted 5G ProSe Direct Discovery Model A with application-controlled extension;

-	restricted 5G ProSe Direct Discovery Model A with "on demand" announcing;

-	restricted 5G ProSe Direct Discovery Model B.

-	subscription for Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for 5G ProSe Layer-2 Remote UE access via 5G ProSe Layer-2 UE-to-Network Relay.

-	when authorized, subscription for 5G ProSe enabled UE’s multi-path transmission via direct Uu and 5G ProSe Layer-2 UE-to-Network Relay

-	subscription for 5G ProSe Layer-3 Remote UE access via 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for multi-path transmission via direct Uu path and 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

Editor's Note:	It is FFS whether subscription is needed for Layer-3 Remote UE's multi path transmission via direct Uu path and Layer 3 UE-to-Network Relay.

-	UE-PC5-AMBR for NR PC5.

-	PC5 QoS parameters as defined in clause 5.6.1 used by NG-RAN.

-	the list of the PLMNs authorized for 5G ProSe services, including:

-	the list of the PLMNs where the UE is authorised for open 5G Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to perform Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 Remote UE.

* * * * Start of Next Change * * * *

### 6.6.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the 5G ProSe Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the 5G ProSe Capability for 5G ProSe operation.

-	The 5G ProSe Capability indicates whether the UE supports one or more of the following ProSe capabilities:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE; and

-	5G ProSe Layer-3 Remote UE.

-	The AMF obtains the 5G ProSe subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to use 5G ProSe services based on UE's 5G ProSe Capability and the ProSe Service Authorisation included in the subscription data received from UDM as specified in clause 5.7. ProSe NR UE-PC5-AMBR is also provided to the AMF as part of the subscription data for 5G ProSe services. The AMF stores the authorized 5G ProSe Capability.

-	The AMF sends the authorized 5G ProSe Capability for 5G ProSe operation to PCF. Based on the received 5G ProSe Capability from the AMF, the PCF provides the PC5 QoS parameters for 5G ProSe to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use 5G ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"5G ProSe authorised" information, including one or more of the following:

-	whether the UE is authorized to use 5G ProSe Direct Discovery;

-	whether the UE is authorized to use 5G ProSe Direct Communication;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE;

-

whether the UE is authorized to use multi-path transmission via direct Uu path and 5G ProSe Layer-2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	ProSe NR UE-PC5-AMBR, used by NG-RAN for the resource management of UE's PC5 transmission for 5G ProSe services in network scheduled mode.

-	the PC5 QoS parameters for 5G ProSe used by the NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	If the UE is authorised to use 5G ProSe services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

* * * * End of Changes * * * *