3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300389

# E-meeting, January 16 - 20, 2023											 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0622 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on enhanced Network Performance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At SA2#154, S2-2211331 (CR#0566R2) introduces enhancement to Network Performance Analytics to support AIML data traffic policies. The above CR adds the following Analytics Filter Information. - Traffic type (overall traffic, GBR traffic or Delay-critical GBR traffic) of interest (mandatory if Target of Analytics Reporting is set to "any UE", optional otherwise); Because "Traffic type" was introduced in Rel-18 and "Area of Interest" is mandatory if Target of Analytics Reporting is set to "any UE" since Rel-16, It is considered that "Traffic type" does not have to be mandatory even if Target of Analytics Reporting is set to "any UE". |  |  |  |  |  |  |  |  |
| Summary of change: |  | §6.6.1 - Modify "Traffic type" as optional parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mandating "Traffic type" if Target of Analytics Reporting is set to "any UE" is not appropriate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.6	Network Performance Analytics

6.6.1	General

With Network Performance Analytics, NWDAF provides either statistics or predictions on the gNB status information, gNB resource usage, communication performance and mobility performance in an Area of Interest; in addition, NWDAF it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Network Performance";

-	Target of Analytics Reporting: either a single UE (SUPI ), or a group of UEs (an Internal Group ID that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested) or any UE;

-	Analytics Filter Information:

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target of Analytics Reporting is set to "any UE", optional otherwise);

-	Optionally, Traffic type of interest (overall traffic, GBR traffic or Delay-critical GBR traffic);

NOTE:	If Traffic type of interest is not provided, overall traffic is considered.

-	Optionally, a list of analytics subsets that are requested among those specified in clause 6.6.3;

-	Optionally, a preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.6.3);

-	Optionally, preferred order of results for the list of Network Performance information:

-	ordering criterion: "number of UEs", "communication performance" or "mobility performance";

-	order: ascending or descending;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics information (see clause 6.6.3) in order to be notified by the NWDAF;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

* * * * End of Changes * * * *