SA WG2 Meeting #154AHE (e-meeting)	S2-2300392r03

# Jan 16 – 20, 2023											                                   (revision of S2-XXXXXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3887 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.orrg/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of 5GS Information Exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, China Mobile, Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction the conclusion part as described in clause 8.3 in TR 23.700-60. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce support of 5GS information exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The following information is not supported to be exposed to the AF: UL and/or DL congestion information, UL and/or DL data rate, events of “GFBR can no longer be guaranteed” and “GFBR can be guaranteed again” for a GBR QoS Flow. Averaging Window provisioned by AF is also not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20, 5.20x, 5.20x.1, 5.X, 5.X.Y1, 5.X.Y2, 5.X.Y3, , 5.7.1.2a, 5.7.2.4, 7.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.20	External Exposure of Network Capability

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability and Analytics reporting capability. The Monitoring capability is for monitoring of specific event for UE in 5G System and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5G System. The Policy/Charging capability is for handling access and mobility management, QoS and charging policies for the UE based on the request from external party. The Analytics reporting capability is for allowing an external party to fetch or subscribe/unsubscribe to analytics information generated by 5G System (this is further defined in TS 23.288 [86]).

Monitoring capability is comprised of means that allow the identification of the 5G network function suitable for configuring the specific monitoring events, detect the monitoring event, and report the monitoring event to the authorised external party. Monitoring capability can be used for exposing UE's mobility management context such as UE location, reachability, roaming status, and loss of connectivity. Monitoring capability can also be used for exposing QoS monitoring result such as UL and/or DL packet delay, round trip packet delay, UL and/or DL congestion information, UL and/or DL data rate, or the events of “GFBR can no longer be guaranteed” and "GFBR can be guaranteed again" for a GBR QoS Flow. AMF stores URRP-AMF information in the MM context to determine the NFs that are authorised to receive direct notifications from the AMF. UDM stores URRP-AMF information locally to determine authorised monitoring requests when forwarding indirect notifications. The Monitoring capability also allows AF to subscribe to the group status changes for a group, either a 5G VN group as described in clause 5.29.2, as well as a group configured by OA&M. In this case the AF is notified if the group member list is updated or a group member is no longer subscribed to the group.

Provisioning capability allows an external party to provision the Expected UE Behaviour or the 5G-VN group information or ECS Address Configuration Information or service specific information to 5G NF via the NEF. The provisioning comprises of the authorisation of the provisioning external third party, receiving the provisioned external information via the NEF, storing the information, and distributing that information among those NFs that use it. The externally provisioned data can be consumed by different NFs, depending on the data. In the case of provisioning the Expected UE Behaviour, the externally provisioned information which is defined as the Expected UE Behaviour parameters in clause 4.15.6.3 of TS 23.502 [3] or Network Control parameter in clause 4.15.6.3a of TS 23.502 [3] consists of information on expected UE movement, Expected UE Behaviour parameters or expected Network Configuration parameter. The provisioned Expected UE Behaviour parameters may be used for the setting of mobility management or session management parameters of the UE. In the case of provisioning the 5G-VN group information the externally provisioned information is defined as the 5G-VN group parameters in clause 4.15.6.7 of TS 23.502 [3] and it consists of some information on the 5G-VN group. In the case of provisioning ECS address, the externally provisioned information is defined as the ECS Address Configuration Information in clause 4.15.6.3d of TS 23.502 [3]. The affected NFs are informed via the subscriber data update as specified in clause 4.15.6.2 of TS 23.502 [3]. The externally provisioned information which is defined as the Service Parameters in clause 4.15.6.7 of TS 23.502 [3] consists of service specific information used for supporting the specific service in 5G system. The provisioned Service Parameters may be delivered to the UEs. The affected NFs are informed of the data update.

Policy/Charging capability is comprised of means that allow the request for session and charging policy, enforce QoS policy, apply accounting functionality and requests to influence access and mobility management policies. It can be used for specific QoS/priority handling for the session of the UE, and for setting applicable charging party or charging rate.

Analytics reporting capability is comprised of means that allow discovery of type of analytics that can be consumed by external party, the request for consumption of analytics information generated by NWDAF.

An NEF may support CAPIF functions for external exposure as specified in clause 6.2.5.1.

An NEF may support exposure of NWDAF analytics as specified in TS 23.288 [86].

The NEF may support exposure of 5GS and/or UE availability and capabilities for time synchronization service as specified in clause 5.27.1.8.

An NEF may support exposure of event based notifications and reports for NSACF as specified in clause 5.15.11.

An AF may only be able to identify the target UE of an AF request for external exposure of 5GC capabilities (e.g. Data Provisioning or for Event Exposure for a specific UE) by providing the UE's address information. In this case the NEF first needs to retrieve the Permanent identifier of the UE before trying to fulfil the AF request. The NEF may determine the Permanent identifier of the UE, as described in clause 4.15.3.2.13 of TS 23.502 [3], based on:

-	the address of the UE as provided by the AF; this may be an IP address or a MAC address;

-	the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF based on the requesting AF; this is needed if the UE address is an IP address.

The NEF may provide an AF specific UE Identifier to the AF:

-	that has explicitly requested a translation from the address of the UE to a unique UE identifier (via Nnef_UEId service); or

-	that has implicitly requested a translation from the address of the UE to a AF specific UE Identifier by requesting external exposure about an individual UE identified by its address.

The AF may have its own means to maintain the AF specific UE Identifier through, e.g. an AF session. After the retrieval of an AF specific UE Identifier the AF shall not keep maintaining a mapping between this identifier and the UE IP address as this mapping may change.

The AF specific UE Identifier shall not correspond to a MSISDN; it is represented as a GPSI in the form of an External Identifier. When used as an AF specific UE identifier, the External Identifier provided by the 5GCN shall be different for different AF.

NOTE 1:	This is to protect user privacy.

* * * Next Changes * * *

## 5.20x Efficient 5GS Information Exposure

### 5.20x.1	General

Some real time network information (e.g. UL and/or DL packet delay, round trip packet delay, UL and/or DL congestion information, UL and/or DL data rate, or the event “GFBR can no longer be guaranteed” for a GBR QoS Flow) are useful for application layer. In order to expose network information timely and efficiently to AF or AS, the following applies:

-	The UPF may be required to expose i.e. QoS monitoring results as defined in clause 5.33.3, to AF. The UPF obtains QoS monitoring result as defined in clause 5.33.3. The UPF may be instructed to report network information directly to an AF or an AS, i.e. bypassing the SMF and the PCF. as described in clause 6.4.2.1 of 23.548[130]. Alternatively, the UPF may be instructed to report the information to the AF via SMF/PCF/NEF.

-	When the exposed network information is provided by the NG-RAN, the NG-RAN may be instructed to report the information via the tunnel between the NG RAN and PSA UPF, as defined in clause 5.33.3. Alternatively, the NG-RAN may be instructed to report the information via control plane to the AF, e.g. via AMF/SMF/PCF/NEF.

NOTE 1:	In the scenario of Edge Computing, the AS can be an Edge Application Server (EAS), the AF can be local AF, the UPF can be local UPF

* * Next Changes * * *

### 5.X	QoS Monitoring

#### 5.X.Y1		Congestion information monitoring

The congestion information is the UL and/or DL congestion information at the NG RAN in QoS flow level. The NG-RAN may be required to provide the QoS Monitoring on the UL and/or DL congestion information measurement. The UPF may be required to monitor the UL and/or DL congestion information reported from the NG-RAN.

The QoS Monitoring on the UL and/or DL congestion information measurement is per QoS flow level.

According to the QoS Monitoring request for UL and/or DL congestion information from SMF, the NG-RAN initiates UL/DL congestion information measurement for a QoS Flow based on the QoS Monitoring request from SMF. The NG-RAN reports the measured UL and/or DL congestion information to the PSA UPF in the GTP-U header of the UL data packet or dummy UL packet when the reporting condition is satisfied. According to the QoS Monitoring request for UL and/or DL congestion information from SMF, the PSA UPF monitors the congestion information report from the GTP-U header of the UL packets. The PSA UPF reports the received UL and/or DL congestion information to the target NF as the QoS Monitoring request from the SMF.

NOTE:	The criteria for RAN to determine its congestion information is up to RAN implementation.

#### 5.X.Y2	Data rate monitoring

The data rate is the UL and/or DL data rate for a QoS flow. Either the NG-RAN or the UPF may be required to provide the QoS Monitoring on the UL and/or DL data rate measurement. When the NG-RAN perform the data rate measurement, the PSA UPF is required to monitor data rate report from the NG-RAN.

The QoS Monitoring on the UL and/or DL data rate measurement is per QoS flow level, or per PDU Session level. And it can be applied to a Non-GBR or GBR QoS flow.

According to the QoS Monitoring request for UL and/or DL data rate from SMF, either the UPF or the NG-RAN is required to initiate data rate measurement for a QoS Flow based on the QoS Monitoring request from SMF. When the NG RAN is required to initiate the measurement, NG-RAN may be instructed to reports the measured data rate to the PSA UPF in the GTP-U header of the UL data packet or dummy UL packet when the report condition is satisfied; The PSA UPF is required to monitor the data rate report from the GTP-U header of the UL packets. For the data rate result either measured by the PSA UPF or reported by the NG-RAN, the PSA UPF is responsible to report the data rate result to the target NF as the QoS Monitoring request from the SMF.

NG-RAN may be instructed to report the data rate via control plane during the lifetime of the QoS Flow. The Bitrate Report can be used for a Non-GBR or a GBR QoS Flow.

NOTE: the reported data rate may be used by the application for codec or rate adaption.

#### 5.X.Y3	QoS notification monitoring

The QoS Notification Control for a GBR QoS flow is as defined in clause 5.7.2.4. When the NG-RAN is instructed to perform QoS Notification Control and report the QoS notification (e.g. events of “GFBR can no longer be guaranteed” and “GFBR can be guaranteed again”) via the tunnel between the NG RAN and UPF as defined in clause 5.7.2.4, the PSA UPF is required to monitor the event report from the NG-RAN by the SMF.

The QoS Monitoring on the Events of “GFBR can no longer be guaranteed” and “GFBR can be guaranteed again” is per GBR QoS flow level.

According to the QoS Monitoring request for the events of “GFBR can no longer be guaranteed” and “GFBR can be guaranteed again” from SMF, the PSA UPF is required to monitor the event report from the GTP-U header of the UL packets. The PSA UPF reports the received event to the target NF as the QoS Monitoring request from the SMF.

.

* * Next Changes * * *

#### 5.7.1.2a	Alternative QoS Profile

The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN. If the SMF provides a new prioritized list of Alternative QoS Profile(s) to the NG-RAN (if the corresponding PCC rule information changes), the NG-RAN shall replace any previously stored list with it.

An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt.

NOTE 1:	There is no requirement that the GFBR monotonically decreases, nor that the PDB or PER monotonically increase as the Alternative QoS Profiles become less preferred.

When the NG-RAN sends a notification to the SMF that the QoS profile is not fulfilled, the NG-RAN shall, if the currently fulfilled values match an Alternative QoS Profile, include also the reference to the Alternative QoS Profile to indicate the QoS that the NG-RAN currently fulfils (see clause 5.7.2.4). The NG-RAN shall enable the SMF to determine when an NG-RAN node supports the Alternative QoS feature but cannot fulfil even the least preferred Alternative QoS Profile.

NOTE 2:	To reduce the risk that GBR QoS Flows are released in case of RAN resource limitations (and then experience difficulties in being re-established), Application Functions can set the least preferred Alternative Service Requirement to an undemanding level.

* * Next Changes * * *

#### 5.7.2.4	Notification control

##### 5.7.2.4.1	General

The QoS Parameter Notification control indicates whether notifications are requested from the NG-RAN when the "GFBR can no longer (or can again) be guaranteed" for a QoS Flow during the lifetime of the QoS Flow. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation).

The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile and may indicate the NG-RAN to report the QoS notification (i.e. the events of “GFBR can no longer be guaranteed” and “GFBR can be guaranteed again”) via the tunnel between the NG-RAN and PSA UPF as defined in clause 5.X.

* * * Next Changes * * *

## 7.3	Exposure

Network exposure is described in clause 5.20, 5.20x and in clause 4.15 of TS 23.502 [3].

* * * End of Changes * * *