3GPP TSG-SA2 Meeting #154 AH-e	S2-2300421

# EMEETING, 16th – 20th January, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3897 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN communication configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN traffic QoS and routing differentiation need to be supported by 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe PIN communication configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PIN is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.X.c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 5.X	Support for Personal IoT Networks

### 5.X.c	Session management enhancement for PIN service support

### 5.X.c.1	PDU Session Establishment for PIN

When PDU Sessions associated with a PIN are established by PEGCs and PEMCs, a SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. If a UE's subscription data indicates it is allowed to be a PEGC and the UE is PEGC-capable, IPv6 Prefix Delegation as described in clause 4.6.2.3 of TS 23.316 [84], or DHCP proxy performed by PEGC are allowed to be applied by the SMF for IP address allocation of PINEs. If IPv4 is used for allocating IP addresses for PINEs, the PEGC shall act as DHCP proxy between the PINE and the SMF/UPF over User Plane.

When a PDU Session associated with a PIN is established, the SMF enables traffic routing via 5G core network between PEMC and any PEGC if the PIN is active.

One PEGC may serve more than one PINs but one PIN will be served by one or more PDU sessions, and the different PIN is served by the different PDU session(s).

### 5.X.c.2	Session management related policy control

For PIN traffic via PEGC/5GC, the 5GC supports the policy control. For PIN traffic of one PIN via 5GC, one PDU session with user plane is established by the UE with PEGC. The policy control is based on session management procedures as specified in TS 23.502 [3] and TS 23.503 [45].

A PEGC or an AF are able to configure traffic QoS and traffic routing for a PIN. The traffic identified by the packet filters includes one or more of following types:

-	Between two PINEs, which is via 5G core network when the two PINEs connect to different PEGCs;

-	Between PINE and PEMC via a PEGC and 5G core network;

-	Between PINE and external DN via a PEGC and 5G core network;

-	Between PEGC and external DN via 5G core network.

# * * * * End of change * * * *