3GPP TSG-SA2 Meeting #154 AH-e	S2-2300422

# EMEETING, 16th – 20th January, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3898 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN policy configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN policy needs to be provisioned to UEs, PIN activation/deactivation (i.e., PIN status) needs to be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe PIN data configuration for a PIN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PIN is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.X.b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 5.X	Support for Personal IoT Networks

### 5.X.b	PIN data configuration

5.X.b.1	General

A PIN may be dynamically created by an AF requesting 5G core network, or may be pre-configured. The UDR stores the information described in clause 5.2.3.3.1 of TS 23.502 [3] for the PIN in the "PIN Data" Data Subset of the Subscription Data.

5.X.b.2	UE policy delivery

For UE with PEGC/PEMC registered in 5GS, the 5GC supports the PRSP delivery to UE based on the policy information provided from the AF for PIN to PCF as specified in TS 23.502 [3] and TS 23.503 [45] for policy delivery.

A PIN may be dynamically created by an AF requesting 5G core network, or may be pre-configured. The UDR stores the information described in clause 5.2.3.3.1 of TS 23.502 [3] for the PIN in the "PIN Data" Data Subset of the Subscription Data.

When a PIN is created, pre-configured, or deleted, the URSP needs to be updated, which results in the URSP rule related to PDU Session associated with the PIN being added or removed. In order to support the case that PEMC is collocated with PEGC, the URSP update is applied to PEMCs and PEGCs.

If a PIN is instructed to be deactive by an AF, the PRSP rule related to the PIN is marked as deactive. If a PIN is instructed to be active by the AF, the PRSP rule related to the PIN is marked as active.

5.X.b.3	Session management policy control

If a PIN is instructed to be deactive or deleted by an AF, the PDU Sessions related to the PIN are released by the network, in case of deletion, the configuration for the PIN in UDR, SMF, and PCF are removed.

If a PIN is instructed to be active by the AF, the communication configurations for the PIN are reinstalled after the PDU Sessions associated with the PIN have been established.

# * * * * End of change * * * *