3GPP TSG-SA2 Meeting #154-AH-e 	S2-2300445r02

# E-meeting, Jan 16 – 20, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0628 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for Federated Learning among Multiple NWDAFs in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To address the 3 Editor’s notes left in 6.2C Editor’s Note: Procedure of MTLF registration and discovery with respect to FL is FFS. - Propose the registration and discovery procedure for FL in 6.2C.1. MTLF (FL server or FL client) register s to NRF with its NF profile, so the FL server and FL clients can be selected when FL is needed. Editor’s Note: Definition of FL capability is FFS. - Clarify that the FL capability type can be FL server or FL client. Editor’ Note: Further details on providing the accuracy of FL model process is for FFS. - Propose the exchanging process of model metric among consumer, FL server and FL client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Federated Learning procedure in 6.2C to remove 3 ENs |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2C	Federated Learning among Multiple NWDAFs

6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML model.

6.2C.2	Procedures

6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2X.2.1-1: Registration and Discovery procedure for Federated Learning

1-3.  NWDAF containing MTLF as FL server or FL clients registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability information including FL capability Type (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

FL capability of a FL client optionally contains computation capability.

4-6  NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as FL server, the MTLF first discovers and selects FL server from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), If FL server is currently doing a FL for the Analytics ID, Time Period of Interest, Service Area.

Once the FL server (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as FL Client(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, Data availability by the FL Client, Time Period of Interest, Interoperability Indicator.

7.  FL Server sends Federated Learning preparation request to the FL Client(s) with Interoperability information. In the preparation request, the following parameters may be added: Available data requirement, availability time requirement.

8.  FL Client(s) decides whether to join the Federated Learning process based on its availability, computation and communication capability, and Interoperability information.

9.  FL Client(s) sends response to FL Server indicating if it will join the FL procedure.

10.  FL Server conducts selection of FL Client(s).

6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

- The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, using Nnwdaf_MLModelProvision service as defined in TS 23.288 [5], including Analytics ID,  pre-determined status (e.g. requested model metric, and/or total training time) and/or periodically reporting condition (e.g. training round/time).

NOTE: The model metric is used to reflect the performance of the ML model, which may consist of e.g. Accuracy, Precision, Recall rate or MAE (Mean Absolute Error), MAPE (Mean Absolute Percentage Error), MSE (Mean Squared Error) for different types of models. The detailed model metric format is out of scope of 3GPP.

1.	FL server sends a request to the selected NWDAF containing MTLF(FL client) that participates in the Federated learning to perform the local model training for Federated Learning, including model metric. FL Server includes initial ML Model Information, a Federated Learning execution flag, FL Correlation ID, guideline information (e.g., maximum response time for FL Client to provide interim local ML model information) in the preparation message.

2.	Each FL client collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3.	During Federated Learning training procedure, each FL client further trains the retrieved ML model from the FL server based on its own data, and reports interim local ML model information to the FL server. Each FL client also computes local model metric and reports it to the FL server.

The ML model information are exchanged between the FL client(s) and the FL server during the FL training process.

Editor's note:	The ML model information exchanged between the FL client(s) and the FL server is FFS.

Editor's note:	The services in step 1, 3, 5a and 6 to enable FL based ML model training is for FFS, and the Figure 6.2C.2.1-1 will be updated accordingly.

4.	The FL server aggregates all the local ML model information retrieved at step 3, to update the global ML model. The FL server can also calculate the global model metric and compare with the local model metrics retrieved at step 3 for further decision.

5a.	[Optional] Based on the consumer request, the FL server updates the training status (e.g., global model metric) to the consumer periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status ( i.e. requested accuracy is achieved or training time expires).

5b.	[Optional] Consumer decides whether the current model can fulfil the requirement, e.g. global model metric is satisfactory for the consumer. The consumer modifies subscription (stops or continues the training process) according to the consumer decision.

5c. [Optional]	According to the request from the consumer, FL server updates or terminates the current FL training process.

6.	If the FL procedure continues, FL server sends the aggregated ML model information to each FL client for next round model training.

7.	Each FL client updates its own ML model based on the aggregated ML model information distributed by the FL server at step 6.

NOTE:	The steps 2-7 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the FL server may send the globally optimal ML model information to the consumer.

END of CHANGES