3GPP TSG-SA2 Meeting #154-AH-e	S2-2300446r01

# Electronic Meeting, January 16th - January 20th 2023					(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0629 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for direct member selection assistance to AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, MediaTek Inc., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2022-12-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion of AIMLsys KI#7, if this new member selection assistance functionality is not used, AF can select the members based on existing network exposure information as described in clause 4.15 of TS 23.502 [4]. The CR proposes to introduce a procedure that AF selects the members based on network exposure information collected directly from 5GC in case that member selection assistance functionality via NEF is not used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the detailed procedure that AF selects the members based on network exposure information collected directly from 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for AF to select the members based on 5GC information when member selection assistance functionality is not used, will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.x(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** Start of Changes (All the text is new)***

#### 4.15.X	Procedure for direct member selection assistance to AF

Figure 4.15.x-1 shows the procedure that AF selects the AI/ML members based on network exposure information collected directly from 5GC by the AF in case that member selection assistance functionality within NEF is not used (e.g. trusted AF by an Operator).

Figure 4.15.x-1: Procedure for member selection

1.		The AF retrieves the user consent subscription data, indicating whether the network is allowed to expose UE related 5GC information, for the UEs in the candidate list from the UDM by invoking existing Nudm_SDM_Get service operation.

NOTE: Whether AF needs to check user consent is determined by SA3 WG.

2.	The AF subscribe interested events (e.g., UE mobility event, QoS monitoring events, User plane events, etc.) from NFs using existing Event Exposure Services of NFs.

3.  The AF requests analytics from NWDAF by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation, such as UE Communication Analytics, User Data Congestion Analytics, DN performance Analytics, QoS sustainability Analytics, etc, as defined in TS 23.288 [50].

NOTE 1:	In Steps 1-3, the AF maintains the candidate list itself before collecting direct member selection assistance information and may include the Target UE Identifier(s) for the UEs in the candidate list in the request as described in clause 5.6.7 of TS 23.501 [3].

4.	The AF selects members based on the information collected in Steps 2-3.

NOTE 2:	What information needs to be collected by the AF to perform member selection is decided by the AF itself.

NOTE 3:	AF keeps subscribing the events / analytics from NFs to update the members.

# *** End of Changes ***