3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300447r05

Online, January 16-20, 2023	(revision of S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0630 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNA_Ph3 DCCF relocation in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The process of DCCF relocation needs to be reflect in TS23.288 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Clause 6.2.6.3.1 - Introduce general information about DCCF relocation 2.Clause 6.2.6.3.Y - Introduce DCCF relocation initiated by DCCF 3.Clause 6.2.6.3.X - Introduce DCCF relocation initiated by consumer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCCF relocation is not updated to reflect conclusions in TR23.700-81 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.1, 6.2.6.3.X (new), 6.2.6.3.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

6.2.6.3.1	General

This clause specifies procedures for data collection using the DCCF described in clause 5A for cases other than obtaining analytics from an NWDAF (which is specified in clause 6.1.2). Two options are supported: data delivered via the DCCF, according to clauses 6.2.6.3.2 and 6.2.6.3.3 and data delivered via a messaging framework according to clauses 6.2.6.3.4 and 6.2.6.3.5. Which option to be used is determined by DCCF configuration.

Due to UE mobility of single UE subscription, the source DCCF may not be able to serve the UE anymore, in such case the data consumer may either select a new DCCF by itself as described in clause 6.2.6.3.X, or indicate source DCCF is allowed to execute the DCCF relocation procedure as described in clause 6.2.6.3.Y. The indicator indicates whether the DCCF performs DCCF reselection or not. If there is no indication from the consumer, the consumer performs DCCF reselection.

Editor’s Note: It is FFS to determine how DCCF reselection and relocation work in case of data collection of group of UEs or any UE.

Editor's Note:	How the service consumer determines whether the DCCF can support relocation procedure is FFS.

* * * Second Change (all new text) * * *

6.2.6.3.X	DCCF (re-)selection initiated by consumer

The procedure depicted in Figure 6.2.6.3.X-1 is used by a data consumer (e.g. NWDAF or central DCCF) to obtain UE(s) data and be notified of events via the DCCF using Ndccf_DataManagement_Subscribe service operation and when DCCF cannot serve the UE(s) anymore, data consumer will reselect DCCF.

Figure 6.2.6.3.X-1: Procedure for DCCF relocation initiated by consumer

0.	Service consumer subscribes to source DCCF.

1.	Source DCCF may notify the service consumer that it cannot serve the subscription anymore, e.g., when location of UE(s) falls outside the serving area of the DCCF. A cause code is added with the notification (e.g., UE(s) moved outside DCCF serving area). The DCCF may send pending data to the data consumer.

2.	Service consumer for the DCCF determines to select a new instance of DCCF. The service consumer discovers and selects the target DCCF as described in clause 6.3.19 of TS 23.501 [2]. The service consumer may perform the DCCF selection due to internal triggers or the subscribed UE mobility event or by receiving the notification from the source DCCF in step 1.

3.	The service consumer sends a subscription request to the target DCCF using Ndccf_DataManagement_Subscribe request.

4.	The service consumer may unsubscribe from the source DCCF.

5.	Target DCCF may subscribe to relevant data source(s), if not yet subscribed.

* * * Third Change (all new text) * * *

6.2.6.3.Y	DCCF and optionally MFAF relocation initiated by DCCF

The procedure depicted in Figure 6.2.6.3.Y-1 is used by a data consumer (e.g. NWDAF or central DCCF) to obtain UE data and be notified of events via the DCCF using Ndccf_DataManagement_Subscribe service operation and when DCCF or MFAF cannot serve the UE anymore, DCCF and MFAF will execute the relocation.

Figure 6.2.6.3.Y-1: Procedure for DCCF relocation initiated by DCCF

- The data consumer subscribes to source DCCF. The consumer NF provides an indicator in the subscription request. The indicator is used by DCCF to determine if the relocation procedure is applicable.

1.	Source DCCF subscribes UE mobility events from AMF. The UE ID is provided by the data consumer in step 0.

2.	If UE moves out of the service area of the source DCCF, source DCCF determines UE DCCF data subscription to be transferred to target DCCF, e.g., triggered by a UE mobility event notification from AMF. If UE moves out of the service area of the source MFAF, source DCCF determines UE MFAF data subscription to be transferred to target MFAF.

3.	Source DCCF may query the NRF to discover and selects the target DCCF and/or MFAF, e.g. based on the UE location information received from AMF.

Conditional on MFAF transfer:

4..	Source DCCF using the Nmfaf_3daDataManagement service to request transfer of UE MFAF data subscription context to the target MFAF

5.	Target MFAF retrieves MFAF subscription from Source MFAF

6. 	Target MFAF accepts the data subscription(s) transfer

7.	Target MFAF responds to the DCCF indicating the transfer is complete. The response may contain a Transaction Reference ID from the Target MFAF.

Conditional on DCCF transfer

8.	Source DCCF using Ndccf_DataManagement_Transfer Request service operation to transfer of UE data subscription to the target DCCF.

9.	Target DCCF accepts the data subscription(s) transfer.

10.	If an MFAF is being used, the Target DCCF uses the Nmfaf_3daDataManagement_Configure service to configure the target MFAF.

11.	Target DCCF confirms UE data subscription transfer to the source DCCF. The confirmation includes the Subscription Correlation ID used by the Target DCCF.

For DCCF or MFAF transfer:

12.	[Optional] Target DCCF subscribes to the relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the data subscription and repeat step 1 to subscribes UE mobility events from AMF.

NOTE 1: for MFAF context transfer without DCCF context transfer, the Target DCCF is also the Source DCCF.

13.	Source DCCF informs the data consumer about the successful UE DCCF or MFAF data subscription transfer using a Ndccf_DataManagement_Notify message. The notification may contain a Subscription Correlation ID provided by the target DCCF.

14.	[Optional] Source DCCF unsubscribes with the data source(s) that are no longer needed for the remaining UE data subscriptions.

NOTE 2:	At this point, UE data subscription transfer is deemed completed.

15-17.	Target DCCF or MFAF collects data from the data source(s) and notifies the data consumer using a Ndccf_DataManagement_Notify message.

* * *End of Changes * * *