3GPP TSG-WG SA2 Meeting #154AH-e	S2-2300463r01

# 16 – 20 January, 2023, Electronic meeting		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0267 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modifying PLMN Operator Class related to NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Inspur, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2022-12-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on TS23.273, NF can request UE location via either Ngmlc service interface or AMF event exposure. However, only AMF (as NF) has a related procedure to enable the location request, which is NI-LR. NWDAF is one kind of NF, and when requesting location service, it is better to be PLMN Operator LCS Clients, as defined in TS22.071. LCS can be offered without a subscription to basic telecommunication services. LCS is available to the following categories of LCS clients: Value-Added Services LCS Clients – use LCS to support various value-added services. These clients can include UE subscribers as well as non-subscribers to other services. PLMN Operator LCS Clients – use LCS to enhance or support certain O&M related tasks, supplementary services, IN related services and bearer services and teleservices. Emergency Services LCS Clients – use LCS to enhance support for emergency calls from subscribers. Lawful Intercept LCS Clients – use LCS to support various legally required or sanctioned services. As defined in Table 7.1-1, when NWDAF acts as an LCS client, the client type is PLMN Operator LCS client, it is proposed to add a corresponding UE LCS privacy profile, i.e. PLMN Operator class. It is proposed to modify as follows: PLMN Operator Class: LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS cClient supporting a bearer service, teleservice or supplementary service to the target UE - LCS client supporting NWDAF analytics Besides, user consent as defined in TS23.288 is also a kind of privacy check, the relationship between PLMN Operator class and use consent needs further study. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify NWDAF PLMN Operator class. And add EN to check the relationship between NWDAF privacy check in LCS and user consent in eNA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not correct definition of PLMN Operator Class. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2.4, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.4.2.2.4	PLMN Operator Class

The PLMN operator class defined in clause 9.5.3.4 of TS 23.271 [4] is supported.

NOTE:	The PLMN Operator Class (except O&M LCS client in the VPLMN) can be applied to SNPN.

In case of NWDAF as an NF requesting UE location, PLMN Operator Class shall be:

- LCS client supporting NWDAF analytics

Editor’s Note: the difference between privacy verification with this new defined PLMN Operator Class and user consent as defined in TS23.288 is FFS.

# * * * * Second change * * * *

## 7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of TS 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy profile data type and included data, refer to clause 5.4.2.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Privacy Indication | M O | Indication of one of the following mutually exclusive global settings: - Location is disallowed - Location is allowed (default) Time period when the Location Privacy Indication is valid |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client or AF not in the external LCS client list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Location not allowed (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC External LCS client list: a list of zero or more LCS clients, AFs and LCS Client groups with the following data for each entry: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [2]. The following data may be present for each service type in the list: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS client supporting a bearer service, teleservice or supplementary service to the target UE - LCS client supporting NWDAF analytics |
| User Plane Connection | O | Indication of one of the following mutually exclusive global settings: - UE is allowed to report periodic or triggered location events via user plane to an LCS Client or AF - UE is not allowed to report periodic or triggered location events via user plane to an LCS Client or AF (default) |

The Mobile Originating data is defined in table 7.1-1 containing the LCS MO-LR services that a UE can receive.

Table 7.1-2: LCS Mobile Originated data for a UE Subscriber

| MO-LR Data | Presence | UDM data |
| --- | --- | --- |
| Mobile Originated data | M | List of MO-LR services allowed for a UE subscriber: - Basic Self Location (UE can receive its own location) - Autonomous Self Location (UE can receive location assistance data) - Transfer to Third Party |
| List of Assistance Data Types for MO-LR | O | A list of one or more types of location assistance data that may be provided to the UE in the MO-LR procedure. |

Table 7.1-3: LCS broadcasting data for a UE subscriber

| Broadcasting Data | Presence | Description |
| --- | --- | --- |
| List of Assistance Data Types | O | A list of one or more types of location assistance data for which ciphering keys should be provided to the UE if requested by the UE when the assistance data is broadcast using ciphering. |

# * * * * End of changes * * * *