3GPP TSG-SA WG2 Meeting #154 AH	S2-2300499r02

Emeeting, Jan 16 - 20, 2022

# Title:	[Draft] Reply LS on PDU Set Handling

# Response to:	(R2-2213351) LS on PDU Set Handling

# Release:	Rel-18

# Work Item:	XRM

Source:	[Tencent, will be] SA2

To:	RAN2

Cc:	SA4, RAN3

Contact Person:

Name:	Lei Yixue

Tel. Number:	86-10-17710237811

E-mail Address:	yixuelei@tencent.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

The 5G QoS model is based on QoS Flows. A QoS Flow is characterised by a QoS profile that includes a single set of 5G QoS characteristics, i.e. a single value for each of the characteristics. The 5G QoS characteristics describe the packet forwarding treatment that a QoS Flow receives edge-to-edge between the UE and the UPF in terms of the following performance characteristics:

1	Resource type (Non-GBR, GBR, Delay-critical GBR);

2	Priority Level;

3	Packet Delay Budget (including Core Network Packet Delay Budget);

4	Packet Error Rate;

5	Averaging window (for GBR and Delay-critical GBR resource type only);

6	Maximum Data Burst Volume (for Delay-critical GBR resource type only).

Based on the conclusion from the FS_XRM study (See TR 23.700-60), SA2 will define following new 5G QoS characteristics taking the PDU Set concept, i.e., the agreement that a PDU Set consists of one or more PDUs to which a QoS characteristic is applicable, into consideration

- PDU Set Delay Budget (PSDB)

- PDU Set Error Rate (PSER)

- PDU Set Integrated Indication (PSII)

It shall be noted that SA2 maintains legacy framework whereby a single set of the 5G QoS characteristics, i.e., a single value for each characteristic is included in the QoS profile associated with a given QoS Flow.

Hence SA2 clarifies that all packets associated with a given QoS Flow are subject to the same forwarding treatment described by the 5G QoS characteristics provided to NG-RAN in the QoS profile.

Finally, it shall be noted that ‘importance’ is not an element of the 5G QoS characteristics and consequently not part of the QoS profile.

As concluded by SA2 in the FS_XRM study, the PDU Set information ‘PDU Set importance’ may be used by NG-RAN for PDU Set level packet discarding in presence of congestion.

2. Actions:

To CT3

ACTION: 	SA2 kindly asks RAN2 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA WG2#155	Feb 20 – 24, 2023		E-meeting