3GPP TSG-SA WG2 Meeting #154 AH	S2-2300499r13

Emeeting, Jan 16 - 20, 2022

# Title:	[Draft] Reply LS on PDU Set Handling

# Response to:	(R2-2213351) LS on PDU Set Handling

# Release:	Rel-18

# Work Item:	XRM

Source:	[Tencent, will be] SA2

To:	RAN2

Cc:	SA4, RAN3

Contact Person:

Name:	Lei Yixue

Tel. Number:	86-10-17710237811

E-mail Address:	yixuelei@tencent.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA2 thanks RAN2 for the LS on PDU set handling.

Regarding to the question in R2-2213351 on PDU set and QoS flow mapping, SA2 would like to reply RAN2 questions as follows:

Q1: In order to decide how PDU sets could be mapped in radio protocols, RAN2 is wondering if different PDU sets could have different characteristics (for instance importance, PSER, and/or PSDB) and if so, which characteristics can be different and with which granularity (e.g. QoS flow, individual PDU Sets…)

SA2 Answer:  Based on the conclusion from the FS_XRM study (See TR 23.700-60), SA2 agreed to define new 5G QoS for PDU Set concept. The PDU Set comprises of one or more PDUs for which the following PDU Set QoS are applicable:

- PDU Set Delay Budget (PSDB)

- PDU Set Error Rate (PSER)

SA2 also agrees to define PDU Set importance and PDU Set Integrated handling Indication (PSIHI).  All the PDU Sets within one QoS flow should apply the same PSER, PSDB and PSIHI. But the PDU Set importance of the different PDU Sets within one QoS flow can be different. SA2 will further discuss how PDU Set Importance is interpreted across QoS Flows and how it relates to the Priority Level of the QoS Flow.

Q2: RAN2 would also like to know whether different types of PDU set can be mapped to the same QoS flow and if so whether RAN should have the ability to treat those differently over the air interface.  If RAN should have such an ability, RAN2 would like to know based on what information signalled to the gNB this would be based on.

SA2 Answer:

SA2 has agreed that 1)PDU sets with different PDU Set QoS(i.e. PSER and PSDB) are mapped to different QoS flows since each QoS flow binds to one PSER, one PSDB at most; 2) PDU sets with different ‘PDU Set importance’ information can be mapped into the same QoS flow in 5GC.

As concluded by SA2 in the FS_XRM study, the PDU Set information ‘PDU Set importance’ may be provided by the UPF to NG-RAN via GTP-U header of user plane packet. It may be used by NG-RAN for PDU Set level packet discarding in presence of congestion.

In this reply SA2 kindly asks RAN2 whether the definition of PSER is consistent with their understanding of the link layer support when for a QoS flow the size of the PDU Set (i.e., composed of different number of PDUs) varies between different PDU Sets or when PSIHI has indicated not all PDUs need to be delivered.

2. Actions:

To CT3

ACTION: 	SA2 kindly asks RAN2 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA WG2#155	Feb 20 – 24, 2023		E-meeting