3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300508r01

# Janurary 16 – 20, 2023, Elbonia		(revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3923 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network exposure support for XR services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud, Xiaomi, MediaTek, vivo, CATT, Ericsson? , Samsung?, Lenovo?, Huawei?, China Mobile?, OPPO? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2022-12-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusions of KI#3 in TR23.700-60, certain network information could be exposured to the XR applications via API or ECN marking. These kind of exposure information and capabilities need to be added in the related clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In new clause 5.37.X, the general descriptions of network exposure support for XR is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network exposure solutions of XRM is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 2, 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

……

[x]	IETF RFC 9330: "Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[y]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S) ".

# * * * * Second change (all text new) * * * *

5.37.X Network exposure for XR/media service

5.37.X.1	 General

The cooperation between 5GS and XR/media service is required to support better user experience. 5GS information exposures can be supported with using IP layer ECN marking for L4S (Low Latency, Low Loss and Scalable Throughput) according to [x] and [y] (see clause 5.37.X.2), or API based method. When the PDU session is established or modified, how to support ECN marking for L4S and API based exposure can be determined.

5.37.X.2 Support ECN marking for L4S

The 5GS may use the IP layer ECN marking for L4S for the uplink and/or downlink traffic per QoS Flow, to expose congestion information based on the AF request. In 5G System, L4S may be used for GBR and non-GBR QoS Flows. ECN marking for L4S in the IP header is supported in either the NG-RAN (see TS 38.300 [27]), or the PSA UPF.

Based on the AF request, the PCF indicates SMF whether the NG-RAN or UPF shall execute the ECN marking for L4S using an indicator or a pre-defined 5QI.

A QoS flow is either L4S enabled or not L4S enabled. In order to map the traffic flow to a QoS Flow with ECN marking for L4S enabled, the packet filters in the PDR provided by the SMF to the PSA UPF can either reuse existing IP-5 tuples or IP ECN-capable indication for L4S transport (i.e., IP header with ECT(1) codepoint).

The establishment of the QoS flows can e.g., be:

- set up statically when a PDU session is established based on configuration in SMF or PCF, or

- dynamically based on detection of elements in the IP header whereby SMF or PCF triggers a QoS flow enabled for L4S, or

- by authorization requests by an AF using existing API for AF session with required QoS, see TS 23.502 [3], clause 4.15.6.6.

ECN marking for L4S in the IP header is supported in either the NG-RAN or the PSA UPF as the following:

- For ECN marking for L4S by the NG-RAN, NG-RAN performs ECN marking (i.e., ECN field with Congestion Experience (CE) codepoint) for uplink and/or downlink in IP layer of the received packets, based on the explicit indication provided by SMF or the pre-defined 5QI in the local configuration.

NOTE 1:	The criteria for NG-RAN to determine when to perform the ECN marking for L4S is up to RAN implementation.

- For ECN marking for L4S by the PSA UPF, PSA UPF performs ECN marking (i.e., CE codepoint) for L4S for uplink and/or downlink IP layer of the received packets, based on congestion information reported from NG-RAN via GTP-U header or dummy GTP-U packets.

During PDU session establishment/modification, a QoS Flow level explicit indication may be provided to PSA UPF from SMF to allow the ECN marking for the purpose of L4S. The SMF provides an explicit indication or the pre-defined 5QI based on the received PCC rule or local configuration to indicate the NG-RAN to report the congestion information for the UL and/or DL of the QoS Flow to the UPF via the GTP-U header of the uplink user plane packets or dummy GTP-U packets. When there is no congestion or congestion ends, the PSA UPF stops ECN marking (i.e., CE codepoint is removed).

The PSA UPF supporting ECN marking for L4S should report the capability to SMF within the N4 association procedure as described in clause 4.4.3 in TS 23.502.

NOTE 2:	The criteria for NG-RAN to determine when to report the congestion information via GTP-U is up to RAN implementation.

NOTE 3:	If the network operator wants to apply the ECN marking for L4S, it shall guarantee that any sender (UE or Server) requesting classic ECN congestion control will not tag its packets with the ECT(1) in order to avoid conflicted usage of ECT(1) in L4S. Otherwise, L4S is not supported in network.

Editor’s Note: During UE mobility, e.g., NG-RAN handover or local PSA UPF relocation, whether there are other impacts to the above methods is FFS.

5.37.X.3 API based exposure per QoS flow

Based on the AF request, the 5GS may support the following QoS Monitoring result exposure per QoS Flow based on the QoS Monitoring as defined in clause 5.33.3 or clause 5.X:

- For the congestion information, based on the PCC rule from PCF, the SMF indicates NG-RAN to report the congestion information via GTP-U header or the dummy GTP-U packets, and indicates the PSA UPF to expose the congestion information via Nupf_EventExposure service as specified in clause 5.2.26 of TS 23.502 [3] or via SMF/PCF/NEF.

Congestion information reported by NG-RAN may be exposed via SMF/PCF/NEF.

Based on the QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to PSA UPF. The PSA UPF, based on the indication from SMF, exposes this information via Nupf_EventExposure service as specified in clause 5.2.26 of TS 23.502 [3] or via SMF/PCF/NEF.

-     Data rate information may be exposed to the AF as one of the following:

-	Based on the SMF indication, data rate may be measured and reported by PSA UPF via Nupf_EventExposure service as specified in clause 5.2.26 of TS 23.502 [3] or via SMF/PCF/NEF.

-	Based on the SMF indication, the RAN may support the exposure of the data rate information via SMF/PCF/NEF.

-       For the delay difference for two QoS Flows, the PCF enables the QoS monitoring for the two QoS Flows and derives the delay difference, and notifies when the delay difference between two QoS Flows exceeds a threshold. The delay measurement for individual QoS Flow is based on QoS monitoring in clause 5.33.3.

- Round trip delay for multiple QoS Flows of the XR service ( e.g. the UL and DL are separated into two flows) is obtained by the PSA UPF based on the QoS Monitoring for round trip delay measurement of individual QoS Flow as described in clause 5.33.3. Round trip delay for multiple QoS Flows of the XR service can be reported by the PSA UPF via Nupf_EventExposure service as specified in clause 5.2.26 of TS 23.502 [3] or via SMF/NEF/PCF.

The AF may provide the Alternative QoS parameter set requirements and Averaging Window to the NEF/PCF for the GBR QoS Flow as specified in clause 4.15.6.6 of TS 23.502 [3].

Estimated bandwidth for 5QI may be exposed by NWDAF (according to information described in clause 6.9.2 in TS 23.288 [59]) to AF.

# * * * * End of changes * * * *