SA WG2 Meeting #154AHE emeeting	S2-2300509r01

# Jan 16th – Jan 20th, 2023; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0088 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion in the clause 8.1.2 of TR 23.700-48, the EAS discovery part for the option D is concluded as follows: - For Option D, the H-SMF sends H-DNS server address included in PCO to UE via V-SMF during PDU Session Establishment/Modification procedure. The DNS query related to the edge computing (corresponding to FQDNs) can be routed to V-EASDF/Local DNS server in the VPLMN reusing IP replacement mechanism. In case of routing to V-EASDF, V-UPF replaces the destination IP address of DNS query (corresponding to FQDNs) to V-EASDF and routes this DNS query to V-EASDF, the EAS discovery procedure with V-EASDF which implements the functions of EASDF refers to TS 23.548. For the DNS query requiring DNS resolution in the HPLMN, the DNS resolution path is same as the normal path in the HR PDU Session. The procedure and restriction of Option D defined in clause 6.2.3.2.3 of TS 23.548 still apply in the HR roaming case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to describe the EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of the study item is not reflected in the normative specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.X (new) |  |  |  |  |  |  |  |  |
| ; |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First CHANGE (all new text)

#### 6.7.2.X	EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO

Based on the operator's configuration and local regulations, the IP replacement mechanism may be used for EAS discovery supporting HR-SBO:

-	For supporting HR-SBO, the H-SMF sends DNS server address of HPLMN included in PCO to UE via V-SMF during PDU Session Establishment/Modification procedure. The DNS query related to the edge computing (corresponding to FQDNs) can be routed to V-EASDF/Local DNS server in the VPLMN using IP replacement mechanism.

Editor’Note: it is FFS how the HSMF determines that it needs to send the DNS server address of HPLMN to be included in PCO to UE. The HSMF should not have to know that the VPLMN is using the option D

NOTE X: This EAS discovery procedure requests modification of IP address of DNS messages. Whether this is allowed or not is subject to local regulations. As this procedure for EAS discovery requires an UPF to detect target FQDN in DNS request, it can apply when DNS security applies e.g. it does not apply to usage of DoH or DoT.

Figure 6.7.2.X-1: EAS discovery procedure with V-EASDF using IP replacement mechanism for supporting HR-SBO

0.	The registration procedure is described in step 1 of clause 6.7.2.2.

1.	The HR-SBO PDU Session establishment is described in the step 2 Figure 6.7.2.2-1  with the following differences:

The V-SMF selects a L-UPF supporting UL-CL and PSA functionalities based on UE location information and the HR-SBO allowed indication in the step 4 of Figure 4.3.2.2.2-1 of TS 23.502[X].

Editor’s Note:	it is FFS when this L-UPF is selected and whether it corresponds to the V-UPF whose TEID is provided to HSMF at PDU Session establsihment

NOTE Y: The selected V-UPF supports both UL-CL and PSA functionalities based on the implementation, e.g. the UL-CL UPF and PSA UPF can be co-located in the single V-UPF.

The H-SMF also constructs PCO with DNS server address field set to DNS server address of HPLMN and sends the PCO to UE via V-SMF.

H-SMF may indicate to the UE either that for the PDU Session the use of the EDC functionality is allowed or that for the PDU Session the use of the EDC functionality is required in the step 13 of Figure 4.3.2.2.2-1 of TS 23.502.

Editor’s Note:	it is FFS why this EDC indication is specific to option D.

Editor’s Note:	it is FFS When the V-SMF selects the V-EASDF in this scenario.

In Step 13a and Step 13b, based on the FQDN range received from the optional VPLMN specific offloading policy, the V-SMF indicates the L-UPF to route DNS queries for the FQDN (range) query to V-EASDF. The V-SMF configures the V-UPF with IP replacement information (i.e. DNS server IP address and port number of HPLMN, V-EASDF IP address and port number). In uplink direction, V-UPF replaces the destination address of the DNS query for DNS requests targeting an FQDN eligible for HR-SBO related offload from DNS server IP address of HPLMN to V-EASDF IP address; In downlink direction, V-UPF replaces the source address of the DNS response for the FQDN from V-EASDF IP address to DNS server IP address of HPLMN.

NOTE Z: For the DNS query requiring DNS resolution in the HPLMN, the DNS resolution path is same as the normal path in the HR PDU Session.

2.	UE sends DNS query to DNS server of HPLMN.

3a.	If the DNS query does not match the FQDN range eligible for HR-SBO related offload, L-UPF delivers the DNS query via H-PSA via N9 and H-PSA delivers the DNS query to the DNS server of HPLMN.

3b.	If the DNS query matches the FQDN range eligible for HR-SBO related offload, the L-UPF delivers the DNS query to V-EASDF using IP replacement mechanism. The following EAS discovery procedure is based on step 3b.

4.	The EAS discovery procedure described in step 8-18 of clause 6.2.3.2.2 applies with the following differences:

This EAS discovery procedure is implemented in the VPLMN.

Editor’s Note:	it is FFS why the V-SMF interacts with H-SMF to performs UL-CL and L-PSA selection in the VPLMN.

In Step 16, the V-SMF interacts with H-SMF to performs UL-CL and L-PSA selection in the VPLMN based on V-EASDF notification and UE location. In some case, the selected UL-CL/L-PSA of VPLMN is the V-UPF selected in step 1, the UL-CL/L-PSA of VPLMN insertion may not be performed. Otherwise, the new UL-CL/L-PSA of VPLMN will be inserted between RAN and V-UPF. The traffic detection rules and traffic routing rules are provided to the V-UPF or the new UL-CL of VPLMN based on EAS information.

5.	The V-EASDF sends the DNS response including FQDN to the L-UPF. The L-UPF replaces the source address from V-EASDF to DNS server of HPLMN in the DNS response based on the V-SMF instructions and sends this DNS response to the UE directly or via UL-CL of VPLMN if existing.

Editor’s Note:	If some steps of this procedure overlap with the description of clause 6.7.2.2, the necessary reference to clause 6.7.2.2 will be done.

END OF CHANGE