3GPP TSG-WG SA2 Meeting #154AHE	S2-2300515

# Elbonia, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3924 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support URSP provisioning in EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of URSP Provisioning in EPS based on conclusion for KI#3 of FS_eUEPO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support the ePCO based URSP provisioning in EPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | URSP provisioning in EPS is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3740 TS 23.503 CR 0839 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.17.X URSP Provisioning in EPS

When the UE first registers in 5GS, the UE includes the Indication of URSP Provisioning Support in EPS in the UE Policy Container carried in Registration Request. Only when receiving this indication in the UE Policy Container, the PCF will provision the URSP to UE in EPS.

When the UE does Initial Attach to EPC, the UE includes the UE Policy Container ePCO in PDN Connectivity Request carrying default APN encapsulated in Attach Request; otherwise, the UE will include the UE Policy Container ePCO in the PDN Connectivity Request during the first request for PDN connectivity. The PDN Connection used by UE and SMF/PGW-C to convey UE Policy Container ePCO shall be kept when the UE is in the CONNECTED mode. When the UE is attached to EPS, the PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE  and subscribes to the applicable PCRTs to SMF+PGW-C. The PCF for the PDU Session sends the UE Policy Container in the same  PDN connection/PDU session in which the UE Policy Container was received. Then the PCF generates the corresponding UE Policy Container in a similar way as it is done in 5GC and then includes it into a new IE for sending it to the SMF over N7, then further to UE as described in TS 23.502 [3] clause 4.11.0a.5.

During EPS to 5GS mobility with N26, the UE Policy Association is terminated by PCF for PDU Session when it receives the indication of RAT type change from the SMF+PGW-C..

Editor’s note: It’s FFS on how to discover the same PCF for UE serving the UE in both EPS and 5GS.

During 5GS to EPS mobility with N26, the AMF doesn’t initiate the UE Policy Association termination procedure, instead the AMF will request the PCF for the UE to delay the termination of UE Policy Association for a configured period. The PCF for the UE initiates the UE Policy Association Termination procedure either after it receives the UE Policy Association Establishment request from PCF for the PDU Session t or after the configured period. The PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The PCF for the PDU Session discovers the address of PCF for the UE  serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS handover").

# * * * * End of changes * * * *