3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300522r02

# Jan 16 – 20, 2023, Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0634 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance NWDAF to enable Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion of KI#8, FS_eNA_ph3 in TR 23.700-81, a new Federated Learing mechanism has been introduced to do model training for some scenarios that faces user data privacy or security issues. In the conclusion, two new functionalities i.e. FL server and FL client are defined, some FL related capability information should be registered to NRF for FL server and FL client discovery. This CR is to generally describe the FL mechanism and refine registration and discovery procedure for FL server and FL client in TS 23.288 based on the conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding a general paragraph in clause 5 to describe the FL mechanism |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what is FL and how to register and discover FL server or FL client |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (all new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... TS/TR … CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new text) * * *

## 5.X 	Ferderated Learning

Federated learning is a machine learning technique that trains an ML Model across multiple decentralized entities holding local data samples, without exchanging/sharing local data. This approach stands in contrast to traditional centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server for short) and multiple NWDAFs containing MTLF acting as FL clients (called FL client for short), the main functionality includes:

FL server:

- discovers and selects FL clients to participant in an FL procedure

- requests FL clients to do local model training and to report local model information.

- generates global ML model by aggregating local model information from FL clients.

- sends the global ML model back to FL clients and repeats training iteration if needed.

FL client:

- locally trains ML model that tasked by the FL server with the available local data set, which is not allowed to share with others due to e.g. data privacy, data security, data access rights.

- reports the trained local ML model information to the FL server.

- receives the global ML model feedback from FL server and repeats training iteration if needed.

FL server or FL client register to NRF with their FL capability information as described in clause 5.2.

When NWDAF containing AnLF requests NWDAF containing MTLF to provide an ML model, the NWDAF containing MTLF determines whether the ML model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or data not available directly from data producer NF (e.g., due to data privacy, data security).  The NWDAF containing AnLF is not aware whether the ML model is trained based on FL or not.

When receiving the ML model provision request from the NWDAF containing AnLF, if the NWDAF containing MTLF can act as the FL server for the ML training, then Federated Learning is initiated by this NWDAF containing MTLF as FL server directly as described in clause 6.2C. Otherwise, the NWDAF containing MTLF should reject  ML model provision request,then the ML model consumer should try to select another NWDAF for ML model provisioning.

Before FL procedure is initiated by FL server, appropriate FL clients should be discovered by FL server as decribed clause 5.2.

When starting an FL procedure, the FL server is to provide an initial model to each FL client, and then each FL client is to perform local model training using their local data set. The data set in different FL clients have the same feature space for different samples (e.g. UE IDs). The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

* * * End of Changes * * *