SA WG2 Meeting #154-AH-e	S2-2300543r02

# Jan 16th - 20th, 2023; Elbonia	(was S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0641 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML model storage and retrieval via ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Lenovo Mobile Com. Technology? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is concluded in TR 23.700-81 for KI#4 as follow: 7. For storing ML models in ADRF, MTLF can store ML model in ADRF based on MTLF policy. 8. MTLF sends the ML model or ML model address to ADRF by using ADRF's ML model storage service. NOTE 2: How MTLF sends the ML model by using 3GPP defined service-based interface is determined in normative phase. 9. NF consumer(s) (MTLF or AnLF) retrieves the ML model or URL (where the model is stored) from ADRF by using ADRF ML model retrieval service. ADRF shall not duplicate the functionality as MLModelProvision Service. Therefore, the functional description of ADRF needs to be updated. Furthermore, since the logic of data management and ML model management in ADRF is different, new services of ADRF need to be defined for the purpose of ML model management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the functional description of ADRF. - Defining new ADRF services used for ML model storage, retrieval and delete. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ADRF do not support the function of ML model management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.B1, 10.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5B.1	General

The ADRF offers services that enable a consumer to store and retrieve ML models, data and analytics. The ML models are trained by NWDAF containing MTLF as described in clause 5.1, the analytics are produced by the NWDAF as described in clause 6.1 and data are collected as described in clause 6.2.

Data, ML model or analytics may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageRequest containing the data or analytics to be stored. The ADRF response provides a result indication.

-	a consumer sending the ADRF an Nadrf_MLModelManagement_StorageRequest containing the ML model or ML model address to be stored. The ADRF response provides a result indication.

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive data or analytics for storage. The ADRF then subscribes to the NWDAF or DCCF for data or analytics, providing ADRF Notification Address (+Notification Correlation ID). Analytics or Data are subsequently provided as notifications using DCCF, NWDAF or MFAF services (Ndccf_DataManagement Nnwdaf_DataManagement or Nmfaf_3caDataManagement service).

-	a consumer sending the ADRF an Nadrf_MLModelManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive the ML Model and ML Model profile or ML Model address for storage. The ADRF then subscribes to the NWDAF MTLF or DCCF for the ML Model and ML Model profile or ML Model address, providing ADRF Notification Address (+Notification Correlation ID). ML Model and ML Model profile or ML Model address are subsequently provided as notifications using DCCF, NWDAF or MFAF services (Ndccf_MLModelManagement Nnwdaf_MLModelManagement or Nmfaf_3caMLModelManagement service).

Data, ML model or analytics may be retrieved from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_RetrievalRequest request to the ADRF to retrieve data or analytics for a Storage Transaction Identifier or a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify. The ADRF determines the availability of the data or analytics in its repository and sends in the response to the consumer either the data or analytics.

-	a consumer sending an Nadrf_MLModelManagement_RetrievalRequest to the ADRF to retrieve ML Model for specific Analytics ID. The ADRF determines the availability of the ML model in its repository and sends in the response to the consumer the address of the ML model stored in ADRF or NWDAF containing MTLF.

-	a consumer sending an Nadrf_DataManagement_RetrievalSubscribe request to the ADRF to retrieve data or analytics for a specified data or analytics collection time window. If the time window includes the future and the ADRF has subscribed to receive the data or analytics, subsequent notifications received by the ADRF are sent by the ADRF to the notification endpoint.

The ADRF determines the availability of the data or analytics and sends a success/failure indication in the response to the consumer. The ADRF then sends one or more notifications using an Nadrf_DataManagement_RetrievalNotify to the Notification Address (+Notification Correlation ID) specified by the consumer. The notification(s) either provide the data or analytics or provide instructions to the endpoint to fetch the data or analytics using an Nadrf_DataManagement_RetrievalRequest.

-	a consumer sending an Nadrf_ML ModelManagement_RetrievalSubscribe request to the ADRF to retrieve ML Model and ML Model profile or ML Model address for a specified time window. If the time window includes the future and the ADRF has subscribed to receive the ML Model and ML Model profile or ML Model address, subsequent notifications received by the ADRF are sent by the ADRF to the notification endpoint.

The ADRF determines the availability of the ML model and ML model profile or ML model address and sends a success/failure indication in the response to the consumer. The ADRF then sends one or more notifications using an Nadrf_ML ModelManagement_RetrievalNotify to the Notification Address (+Notification Correlation ID) specified by the consumer. The notification(s) either provide the ML model and ML model profile or ML model address or provide instructions to the endpoint to fetch the ML model and ML model profile or ML model address using an Nadrf_ML ModelManagement_RetrievalRequest.

Data, ML model or analytics may be deleted from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_Delete or Nadrf_MLModelManagement_Delete request.

An ADRF may be configured to register the data it is collecting with a DCCF. The registration uses the Ndccf_ContextManagement service specified in clause 8.3.2. The registration may subsequently be used by the DCCF to obtain data from the ADRF as described in clause 6.2.6.3.6.

An ADRF may be configured to register the ML model and ML model profile, or ML model address it is collecting with a DCCF.

SECOND CHANGE

## 10.X	Nadrf_MLModelManagement service

### 10.X.1	General

Service Description: This service enables the consumer to store, retrieve and remove ML model(s) or ML model address(es) from an ADRF.

### 10.X.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store ML model(s) or ML model address(es). ML model(s) or ML model address(es) are provided to the ADRF in the request message.

Inputs, Required: ML model Filter information for the trained ML model(s) per Analytics ID(s), ML model file or Set of:

-	the tuple (NF instance ID of the NWDAF containing MTLF, address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF), when multiple ML models is not supported; or

-	the tuple (NF instance ID of the NWDAF containing MTLF, one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF).

NOTE:	The ADRF can download ML model files based on the ML model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML model and locally stores the ML model is left for implementation..

Inputs, Optional: Validity period, Spatial validity.

Outputs Required: Result Indication.

Outputs, Optional: Storage Transaction Identifier.

### 10.X.3	Nadrf_MLModelManagement_StorageSubscriptionRequest service operation

Editor's note:	The details of Nadrf_MLModelManagement_ StorageSubscriptionRequest service operation is FFS.

### 10.X.4	Nadrf_MLModelManagement_StorageSubscriptionRemoval service operation

Editor's note:	The details of Nadrf_MLModelManagement_StorageSubscriptionRemoval service operation is FFS.

### 10.X.5	Nadrf_MLModelManagement_RetrievalRequest service operation

Service operation name: Nadrf_MLModelManagement_RetrievalRequest

Description: The consumer NF uses this service operation to retrieve stored ML model(s) or the address(es)  of ML model stored in an NWDAF containing MTLF from the ADRF. The Nadrf_MLModelManagement_RetrievalRequest response either contains the ML model file, or the address of the ML model file and the NF instance ID of the NWDAF containing MTLF which stores the ML model file.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2.

Inputs, Optional: ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), ML Model Reporting Information (including e.g. ML Model Target Period), indication of support for multiple ML models, multiple ML models Filter Information to indicate the conditions for which multiple ML models are requested.

Outputs Required: When multiple ML models is not supported, the ML Model file or set of:

-	the tuple (Analytics ID, NF instance ID of the NWDAF containing MTLF, address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF); or

-	the tuple (Analytics ID, address (e.g. URL or FQDN) of Model file stored in ADRF).

When multiple ML models is supported, the ML Models files or set of:

-	the tuple (Analytics ID, NF instance ID of the NWDAF containing MTLF, one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF); or

-	the tuple (Analytics ID, one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF).

Outputs, Optional: None.

### 10.X.6	Nadrf_MLModelManagement_RetrievalSubscribe service operation

Editor's note:	The details of Nadrf_MLModelManagement_RetrievalSubscribe service operation is FFS.

### 10.X.7	Nadrf_MLModelManagement_RetrievalUnsubscribe service operation

Editor's note:	The details of Nadrf_MLModelManagement_RetrievalUnsubscribe service operation is FFS.

### 10.X.8	Nadrf_MLModelManagement_RetrievalNotify service operation

Editor's note:	The details of Nadrf_MLModelManagement_RetrievalNotify service operation is FFS.

### 10.X.9	Nadrf_MLModelManagement_Delete service operation

Service operation name: Nadrf_MLModelManagement_Delete

Description: This service operation instructs the ADRF to delete stored ML model file(s) or ML model address(es).

Inputs, Required: One of the following:

-	Storage Transaction Identifier; or

-	ML Model Filter Information per Analytics ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

END OF CHANGES