3GPP TSG-WG SA2 Meeting #154AH-e	S2-2300562r01

16 - 20 January, 2023, Electronic Meeting	(revision of S2-230xxxx)

Title:	[DRAFT] Reply LS on Handling of the Allowed PDU session status IE in Non-allowed service area

Response to:	LS on Handling of the Allowed PDU session status IE in Non-allowed service area (S2-2300029/C1-227197)

Release:	Rel-18

Work Item:	5GProtoc18

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Myungjune Youn

E-mail Address:	m.youn@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank CT1 regarding the LS on Handling of the Allowed PDU session status IE in Non-allowed service area. SA2 discussed CT1's scenario as described in the following.

CT1 would like to ask SA2 for guidance on an issue. The scenario is as follows: - The UE is registered to an AMF both via 3GPP and non-3GPP access. The UE is currently in a non-allowed area. The registration area (TAI list) for 3GPP access includes both TAs from the UE's non-allowed area and TAs from the allowed area. The UE is currently in a non-allowed area. - The UE has at least 1 PDU session activated via non-3GPP access which cannot be transferred to 3GPP access. - The UE is in IDLE mode both for 3GPP and non-3GPP access, when it receives a paging via 3GPP access indicating non-3GPP access and responds with a Service Request.

Question 1: Does the UE need to include the Allowed PDU session status IE also if it responds to the paging when it is in a non-allowed area?

SA2 Answer: Since a UE is not allowed to initiate Service Request for user plane data in non-allowed area, the UE indicates that no PDU Sessions can be re-activate over 3GPP access.

Question 2: Is the UE allowed to include the Uplink data status IE if it has uplink data pending if it responds to the paging when it is in a non-allowed area?

SA2 Answer: The UE is not allowed to request activation of user plane when the UE is in the non-allowed area.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#155	February 20 – 24, 2023	Athens, Greece

TSG-SA2 Meeting#156	April 17 – 21, 2023	E-Meeting