3GPP TSG-SA WG2 Meeting #154AH-E 		S2-2300588r01

Elbonia, Jan 16th – 20th, 2023	(revison of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0163 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On the Update of Group Message Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA2#154 meeting, the group message procedure following the principle of clause 8.4 of TR 23.700-47 was agreed to be adopted in TS 23.247 with four Editor’s notes. It was assumed that some further works were needed. The agreed procedures and general description provide a good basis for further update, while there is another aspect to be clarified, which is how does the AF make use of the information of non-MBS-supporting area provided by NEF to the AF. As discussed and described in some certain solutions in TR 23.700-47, the AF can use unicast transmission to the UEs located in the area not supporting MBS. It is proposed to clarify such as aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the usage of the non-MBS-supporting area from 5GC to AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear why to provide the area not supporting MBS from 5GC to the AF in TS 23.247. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15, 7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

7.5	MBS procedures for Group Message Delivery

Editor's note:	The service operation and parameters are FFS, and the related steps in this clause is to be aligned with clause 9.

Editor's note:	It requires further investigation on whether the service operation should align more with T8 interface (delivery & modify) or follow the SBI design principles (CRUD).

Editor's note:	It is FFS whether TLTRI (T8 Long Term Transaction Reference ID) should be kept or use Group Message Correlation ID instead.

Editor's note:	It is FFS for the case that the service area in the request is different with the service area associated with the TMGI which is provided during the TMGI allocation.

7.5.1	Group Message Delivery via MBS Broadcast

Figure 7.5.1-1: Group Message Delivery via MBS Broadcast

NOTE 1:	In the procedures referred to the TS 26.502 [18], the NEF is the MBS Application Provider.

1.	The AF sends Group Message Delivery Request to the NEF, containing the Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time, External Group Identifier.

2.	The NEF checks authorization of AF. If geographical area information or civic address information was provided by the AF as MBS service area, the NEF translates the MBS service area to Cell ID list or TAI list. The NEF may further check the MBS capability within the MBS service area.

NOTE 2:	The NEF is mandated for group message delivery.

3.	The NEF transforms the group message payload into a file, and determines the meta data information of the file (e.g. File URL, etc.). The NEF assigns a Group Message Correlation ID that identifies this Group Message Delivery Request.

If Application Service Provisioning hasn't been performed, step 4 to step 8 needs to be executed. Otherwise, they can be skipped.

4.	The NEF performs Application Service Provisioning towards the MBSF using Object Distribution Method as specified in TS 26.502 [18], which including invoking Nmbsf_MBSUserService_Create and Nmbsf_MBSUserDataIngestSession_Create on the MBSF.

5.	The MBSF performs MBS Session Creation as specified in clause 7.1.1.2 or clause 7.1.1.3.

6.	The MBSF performs Distribution Session Provisioning as specified in TS 26.502 [18]. The MBSF invokes Nmbstf_MBSDistributionSession_Create on the MBSTF, passing the parameters of the MBS Distribution Session received in step 4 to the MBSTF.

7.	The MB-SMF initiates the MBS Session Start for Broadcast procedure as specified in step 2 to step 9 in clause 7.3.1.

8.	If the MBSF performs the service announcement, it initiates the MBS User Service Announcement as specified in TS 26.502 [18]. The application may receive the appropriate information through the MBS-6 API from the MBS Client (see TS 26.502 [18]). The NEF may receive the service announcement information via Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation (see TS 26.502 [18]).

9.	The NEF sends Group Message Response to the AF, containing Group Message Correlation ID, Acceptance Status, Cause. The Acceptance Status indicates whether the group message delivery request is accepted or not. If not, the Cause indicates the appropriate failure reason. If the AF performs the service announcement, the service announcement information containing the file meta data can be optionally included in the Group Message Response. The NEF may further include the area where MBS is not supported within the MBS service area in the Group Message Delivery response to the AF.

10.	If the AF needs to perform the Service Announcement, the AF sends the application service announcement to the UE as specified in TS 26.502 [18].

For the UE located in the area where MBS is not supported, if the AF knows the UE location, AF may use unicast to send the data to those UEs.

11.	The NEF performs the User Data Ingestion towards the MBSTF as specified in TS 26.502 [18]. The NEF may push the file to the MBSTF or let MBSTF pull the file from the NEF.

12.	The MBSTF performs packetization and optionally FEC encoding as specified in TS 26.502 [18].

13.	The MBSTF delivers the packets to the MB-UPF to NG-RAN, and NG-RAN broadcast to the UE as specified in step 13 to step 15 in clause 7.3.1.

14.	Based on the service announcement information received in step 8 or step 10, the UE receives the packets, is required performs FEC decoding to restore the file, and gets the group message from the file, using the MBS Client as specified in TS 26.502 [18]. The MBS Client can expose the file towards the application in the UE using the MBS-7 API (see TS 26.502 [18]).

15.	The NEF sends a Group Message Delivery notify to the AF, containing Group Message Correlation ID, Delivery Status. The Delivery Status indicates whether delivery of Group Message Payload corresponding is successful or not. The delivery status information is received by the NEF via Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation (see TS 26.502 [18]).

* * * * End of Changes * * * *