3GPP TSG-SA WG2 Meeting #154AH-E		S2-2300589r01

Elbonia, Jan 16th – 20th, 2023	(revison of S2-221xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0164 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On Clarifying the Scenario Considering the Power Saving Mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | KI#5 had been concluded in SA2#154 meeting. It is concluded that Sol#14 is used as the basis for normative work with some clarifications as mentioned in TR 23.700-47 clause 8.5. Therefore, it is possible to specify the conclusion of KI#5 in TS 23.247. To be specific, a clause with including the general description can be added, it lists the general principles for 5GS providing MBS data to the UEs with requiring the power saving. Other potential enhancement shall be triggered by the RAN WGs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions for coexistence with existing power saving mechanisms for capability-limited UEs in TS 23.247. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the functionalities for KI#5, which has been concluded in TR 23.700-47. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of the change * * * *

## 6.X	Support of coexistence with existing power saving mechanisms for capability-limited UEs

MBS provide means to deliver data over MBS session to multiple UEs at the same time. However, those UEs may make use of power-saving mechanisms at the same time, e.g., MICO (Mobile Initiated Connection Only) mode with Active Time,  extended idle mode DRX (Extended Discontinuous Reception) as defined in TS 23.501 [xx].

If a UE becomes unreachable for unicast service due to either MICO mode with Active Time or extended idle mode DRX, the UE may still perform MBS specific procedures, e.g. activation/deactivation of the MBS service, MBS data transfer reception, reception of service announcement (if needed).

In addition to the functionalities defined in 5.3.2.8, the following applies for the case that UEs with existing power saving mechanisms and receiving MBS data:

-	When the UE needs to be awake due to MBS coinciding with the UE already being in CM-CONNECTED/RRC_CONNECTED mode due to other reasons, the UE follows normal connected mode procedures.

-	When the UE needs to be awake due to MBS coinciding with the UE already being in CM-IDLE mode and reachable (e.g. in active time in MICO or PTW for eDRX) the UE follows normal idle mode procedure.

-	When the UE needs to be awake due to MBS coinciding with the UE being in CM-IDLE mode and in deep sleep, i.e. unreachable for paging to the network, the UE leaves the deep sleep state only to perform procedures related to MBS service.

-	If the MBS service does not require the UE to transition to CM-CONNECTED/RRC_CONNECTED mode, i.e. the UE receives MBS service in broadcast mode and therefore can be in CM-IDLE mode, then the UE does not update the AMF to become reachable for paging. The UE would therefore still be considered unreachable for paging in the AMF. This minimizes the signalling between the UE and the network.

-	If the MBS service requires the UE to transition to CM-CONNECTED/RRC_CONNECTED mode (e.g. reception in MBS multicast mode) then the UE performs regular procedures for MBS multicast mode. This would therefore make the UE become reachable in the network for other unicast services.

-	When the UE is in the middle of an MBS data transfer, and the UE is scheduled to move to deep sleep due to power saving, e.g. end of PTW for extended idle mode DRX, expiration of active time for MICO or the UE transitioning from CM-CONNECTED to CM-IDLE in the case of MICO with no active time, then the UE does not go to deep sleep during the remainder of the current MBS data transfer.

-	At the session start time and the possible scheduled activation times, for an MBS multicast session, if the UE has already joined the MBS session as defined in clause 7.2.1.3, at the possible scheduled activation times, UEs in idle mode need to listen for paging requests and if paged by the network with group paging follow the existing procedures in clause 7.2.5.2. In this case, how long the UE need to listen to paging is left up to UE implementation. If the UE has not previously joined the MBS multicast session, at the possible scheduled activation time it performs MBS join procedure as currently defined in clause 7.2.1.3. Whether the UE performs MBS join procedure in advance and stays "joined" every time at activation time is left up to UE implementation.

Editor's Note: The procedure for MBS when the UE is in  CM-CONNECTED/ RRC_INACTIVE state with eDRX > 10.24s is FFS.

NOTE 1:	If at the end of the current MBS data transfer, the UE knows there is another MBS data transfer scheduled soon, in that case depending of the time between MBS data transfers, the UE can decide to go to sleep between MBS data transfers.

There are several possible ways the UE can be notified of an upcoming MBS session:

-	If MBS User Services defined in TS 26.502 [18] is used, the UE needs to receive MBS service announcement while awake (i.e. while in connected mode, or while idle mode during PTW for extended idle mode DRX, or active time for MICO). The UE wakes up if not already awake for MBS service reception based on the schedule received in the service announcement. For this option, the MBS service announcement may be provided via MBS broadcast service announcement or via any of the possible unicast service announcement delivery mechanisms.

NOTE 2:	In order to allow all UEs using power saving function to receive the service announcement in time to be able to receive the MBS broadcast data delivery, the application server needs to be aware of the maximum unreachable period of the UEs.

-	The UE may be configured by the AF with specific times (i.e., session start time) to perform MBS procedures and a possible sequence of scheduled activation times when the AF may activate the MBS session and transmit MBS data, and wakes up from deep-sleep if needed at those times. The UE may also receive MBS service announcements and/or MBS broadcast delivery at those times (if needed). At the session start time and the possible scheduled activation times, the UEs apply the above-mentioned procedures to receive MBS data.

NOTE 5:	This means that for an MBS multicast session, if the UE has already joined the MBS session as defined in clause 7.2.1.3, at the possible scheduled activation times, UEs in CM-IDLE mode need to listen for paging requests and if paged by the network with group paging follow the existing procedures in clause 7.2.5.2. In this case, how long the UE need to listen to paging is left up to UE implementation.

NOTE 6:	The configuration (e.g. TMGI, start time) is out of scope of 3GPP and assumed to be performed between AF and UE at application layer. The AF needs to initiate MBS bearer service procedures during those time intervals.

Editor's note:	How to support NR capability-limited (RedCap) UEs in MBS will be decided in normative phase considering possible related decisions of RAN WGs

# * * * * End of changes * * * *