3GPP TSG-SA WG2 Meeting #154AH-E		S2-2300589

Elbonia, Jan 16th – 20th, 2023	(revison of S2-221xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0164 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On Clarifying the Scenario Considering the Power Saving Mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | KI#5 had been concluded in SA2#154 meeting. It is concluded that Sol#14 is used as the basis for normative work with some clarifications as mentioned in TR 23.700-47 clause 8.5. Therefore, it is possible to specify the conclusion of KI#5 in TS 23.247. To be specific, a clause with including the general description can be added, it lists the general principles for 5GS providing MBS data to the UEs with requiring the power saving. Other potential enhancement shall be triggered by the RAN WGs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions for coexistence with existing power saving mechanisms for capability-limited UEs in TS 23.247. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the functionalities for KI#5, which has been concluded in TR 23.700-47. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of the change * * * *

## 6.X	Support of MBS reception by capability-limited UEs applying power saving mechanisms

MBS provide means to deliver data over MBS session to multiple UEs at the same time. However, those UEs may make use of power-saving mechanisms, e.g., MICO (Mobile Initiated Connection Only) mode with Active Time,, extended idle mode  eDRX (Extended Discontinuous Reception) at the same time ) as defined in TS 23.501 [5].

If a UE becomes unreachable for unicast service due to either MICO mode with Active Time or extended idle mode DRX, the UE may still perform MBS specific procedures, e.g. activation/deactivation of the MBS service, MBS data transfer reception, reception of service announcement (if needed).To receive MBS data, those UEs need to wake up at coordinated times when the MBS data are being transmitted

The UE is informed via the service announcement about a start time and/or a sequence of scheduled activation times (e.g. a first time and a periodicity) of the MBS session when the AF may activate the MBS session and transmit MBS data.

The AF may send data starting either at the start time or at any scheduled activation times. If the AF sends data using an MBS multicast session at a scheduled activation time, it shall previously activate the MBS session at that scheduled activation time.

For an MBS multicast session, a UE needs to join the multicast session as defined in clause 7.2.1.3 prior to MBS data reception. To join the multicast session, the UE needs to be in RRC_CONNECTED state. The UE may select any suitable time when it is in RRC_CONNECTED state or transition to RRC_CONNECTED state before joining the multicast session.

NOTE 1:	If the UE has not previously joined the MBS multicast session, at the possible scheduled activation time it performs MBS join procedure. Whether the UE performs MBS join procedure in advance and stays "joined" or every time at activation time is left up to UE implementation.

If a UE is in RRC_CONNECTED mode due to other reasons at the start time and the scheduled activation times, the UE follows normal connected mode procedures.

If a UE is in RRC_IDLE or RRC_INACTIVE mode and reachable (e.g. in active time in MICO or PTW for eDRX) at the start time and the scheduled activation times) the UE follows normal idle mode or inactive mode procedures.

If a UE is in RRC_IDLE mode and not reachable (i.e. in deep sleep) at the start time and the scheduled activation times,the UE leaves the deep sleep state only to perform procedures related to MBS service:

-	For an MBS multicast session, at the possible start time, an RRC_IDLE or RRC_INACTIVE UE needs to send a request to transition to RRC_CONNECTED state and join the MBS multicast session (if not done before).

-	At the possible scheduled activation times, an RRC IDLE UE that already joined the multicast session needs to listen for paging requests and if paged by the network follow the existing procedures in clause 7.2.5.2. How long the device listens to paging is left up to device implementation.

-	For an MBS broadcast session, at the possible start time and the possible scheduled activation times, an RRC_IDLE device listens for broadcasted MBS data, but should not update the AMF to become reachable for paging to minimize signaling load.

When the UE is in the middle of an MBS data transfer, and the UE is scheduled to move to deep sleep due to power saving, e.g. end of PTW for extended idle mode DRX, expiration of active time for MICO or the UE transitioning from CM-CONNECTED to CM-IDLE in the case of MICO with no active time, then the UE does not go to deep sleep during the remainder of the current MBS data transfer.

NOTE 1:	If the UE has not previously joined the MBS multicast session, at the possible scheduled activation time it performs MBS join procedure as currently defined in clause 7.2.1.3. Whether the UE performs MBS join procedure in advance and stays "joined" or every time at activation time is left up to UE implementation.

Editor's Note: The procedure for MBS when the UE is in  CM-CONNECTED/ RRC_INACTIVE state with eDRX > 10.24s is FFS.

NOTE 3:	If at the end of the current MBS data transfer the UE knows there is another MBS data transfer scheduled soon, depending on the time between MBS data transfers, the UE can decide to go to sleep between MBS data transfers.

Editor's note:	How to support NR capability-limited (RedCap) UEs in MBS is ffs. Possible related decisions of RAN WGs need to be considered

# * * * * End of changes * * * *