3GPP TSG-SA WG2 Meeting #154AH-e	S2-2300600r02

# Electronic meeting, 16 – 20 January 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0180 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on path switching between different U2N relays |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN has not discussed/introduced configured criteria for path switch between two indirect network communication paths. The SL-RSRP or SD-RSRP can still be for the relay reselection criterion. After the UE has evaluated the USRP, the UE does not require to remember the RSD which is has been matched and does not require the UE to the remember the order of the selection. And the UE will not use this selected RSD as the starting point for the next reevaluation of the whole URSP. So for the target path selection, it would be beneficial for the UE to just reevaluate the URSP or the 5G ProSe policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NG-RAN configured criteria is removed. Keeping evaluating the next RSD of the selected URSP for the target path selection is replaced by the reevaluating of URSP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear what the NG-RAN configured criteria is. Remembering the order of selected RSD adds the UE complexity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 5.15	Path switching between two UE-to-Network Relays

If relay reselection has been triggered, see clause 6.5.3, , the 5G ProSe Remote UE performs the path switching between two UE-to-Network Relays based on the URSP rules of the 5G ProSe Remote UE traffic handling described in clause 6.5.4, 5G ProSe Policy in clause 5.1.4, or information from application layer (if available) with following considerations:

-	The 5G ProSe Remote UE shall first select the target path which has same type with the original path (i.e. 5G ProSe Layer-2 UE-to-Network Relay, 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, and 5G ProSe Layer-3 UE-to-Network Relay without N3IWF).

-	If the same type target path is not available, then this 5G ProSe Remote UE re-evaluates the URSP as specified in clause 6.6.2.1 of TS 23.503 [9] or 5G ProSe Policy in clause 5.1.4 for the target path selection.

NOTE:	If there is the information from the application layer (e.g. provided by an application server), the 5G ProSe Remote UE may use this information to discover the target 5G ProSe UE-to-Network Relay UE. If there are multiple discovered 5G ProSe UE-to-Network Relay UEs, the 5G ProSe Remote UE selects the target 5G ProSe UE-to-Network Relay based on the priority, if any, in the information from the application layer.

The principles from the 5G ProSe Policy in clause 5.1.4 and URSP rules of the 5G ProSe Remote UE traffic handling described in clause 6.5.4 take the higher priority than the information from the application layer.

The path switching procedures between two indirect network communication paths are as described in clause 6.5.5.

* * * * End of Changes * * * *