3GPP TSG-SA WG2 Meeting #154-AH-E	S2-2300636r01

16 - 20 January 2023, Elbonia	(revision of S2-2211232, of S2-2211218 of S2-2211012 of S2-2210921)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0263 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of PRUs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The study on enhancement to the 5GC LoCation Services Phase 3 (FS_eLCS_Ph3) has reached conclusions on support of PRUs in TR 23.700-71. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedures for PRU Association and Target UE positioning assistance are added to support PRUs according to the conclusions in TR 23.700-71. In addition to the conclusions in TR 23.700-71, PRU Disassociation procedures are included since a serving LMF for a PRU may not always be available (e.g. may be removed, replaced, modified or temporarily withdrawn for maintenance) and a PRU, similarly, may not always be available . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to support PRUs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11, 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 6.11	Common Sub-Procedures

The procedures defined in this sub-clause are applicable to both a UE and PRU. Wherever, a UE is referenced in these procedures, a PRU may be used instead.

**** Next Change ****

## 6.X	Procedures applicable to a PRU

### 6.X.1	PRU Association Procedure

Figure 6.X.1-1 shows a procedure used by a PRU to associate as a PRU with a serving LMF. The procedure is used for initial PRU Association with the serving LMF which may occur when the PRU first starts to access the HPLMN. The procedure can also be used to perform a PRU Association update to inform the serving LMF of the continued availability of the PRU or to inform the serving LMF of some change to the PRU such as a change of location (e.g. a change of tracking area or change of serving AMF), a change of PRU state (e.g. on/off state), a change of PRU type (e.g. switch between mobile and static) or a change of the PRU positioning capabilities. The PRU shall only perform the Association procedure in the HPLMN.

Figure 6.X.1-1: PRU Association Procedure

Precondition:

The PRU is currently registered in the HPLMN. For initial PRU Association, a Routing Identifier has been configured in the PRU indicating the serving LMF. For subsequent PRU Association, a Routing ID indicating a serving LMF has been returned to the PRU at step 7a or 7b of a previous PRU Association procedure.

NOTE 1:	A Correlation ID and a Routing ID are different terms for the same identifier. The term "Correlation ID" is used for an identifier in service operations between an AMF and LMF while the term "Routing ID" is used for an identifier in a NAS message sent over the N1 reference point between a PRU and AMF.

1.	The PRU performs a UE Triggered Service Request if in CM IDLE state.

2.	The PRU sends a location supplementary services PRU Association Request to the serving AMF in an UL NAS TRANSPORT message and includes the preconfigured Routing ID for an initial Association or the Routing ID received at step 7a or step 7b for a previous PRU Association procedure. The PRU Association Request is included in the UL NAS TRANSPORT message as a "Location services message container" at the NAS level. The PRU Association Request includes a reason for the PRU Association (e.g. initial PRU Association, or PRU Association update), the PRU’s positioning capabilities, location information (if known), a type and state of the PRU (e.g. mobile/static type).

3.	[Optionally,] The AMF may verify that the sender of the PRU Association Request is a PRU using subscription information from the UDM. If step 3 is not performed, the AMF treats the PRU as a normal UE.

4.	The AMF selects the serving LMF based on the Routing ID and optionally the current TAI and transfers the PRU Association Request to the serving LMF using an Namf_Communication_N1MessageNotify service operation. If step 3 was performed, the AMF includes in the Namf_Communication_N1MessageNotify service operation an indication that the PRU subscription information was verified by the AMF. The AMF also includes the SUPI of the PRU.

5.	The serving LMF verifies the PRU if it is not verified by AMF in step 3. This can be based on matching the SUPI for the PRU received at step 4 with the list of authorized SUPI(s) configured in the LMF.

6a.	If the serving LMF received an indication that the PRU was verified by the AMF in step 4 or the serving LMF verified the PRU at step 5 and can accept the PRU Association, the serving LMF returns a PRU Association Accept, as a location services supplementary services message, using Namf_Communication_N1N2MessageTransfer service operation towards the AMF, and a Correlation ID. The Correlation ID is assigned by the serving LMF to identify the serving LMF and optionally the PRU. The PRU Association Accept indicates conditions for performing PRU Association updates with the serving LMF which may include a periodic PRU Association update timer and PRU Association update based on a change of PRU location, change of PRU TAI, change of serving AMF, change of PRU state, change of PRU type or change of PRU positioning capabilities.

NOTE 2:	A periodic PRU Association is independent of a periodic NAS Registration and may occur with greater, equal or lesser frequency. For example, a fixed (non-mobile) PRU may perform periodic PRU Association infrequently whereas a mobile PRU may perform periodic PRU Association at short intervals to enable the LMF to follow its current location.

7a.	The serving AMF forwards the PRU Association Accept and a Routing ID equal to the Correlation ID to the PRU in a DL NAS TRANSPORT message. The PRU stores the Routing ID which is used for any further PRU Association update with the serving LMF.

6b.	If the serving LMF is unable to verify the PRU at step 5 or cannot accept the PRU Association for some other reason (e.g. the serving LMF is not the correct serving LMF for the PRU), the serving LMF returns a PRU Association Reject message, using Namf_Communication_N1N2MessageTransfer service operation towards the AMF, and may include the Routing ID of a new serving LMF.

7b.	The serving AMF forwards the PRU Association Reject in a DL NAS TRANSPORT message to the PRU.

8.	If PRU Association is performed successfully as in steps 6a and 7a, the serving LMF may optionally verify any PRU location provided at step 4 or obtain a more accurate location of the PRU using the procedures defined in clause 6.11. The LMF also stores information received for the PRU.

9.	If PRU Association is performed successfully as in steps 6a and 7a and if this is an initial PRU Association or if this is a PRU Association update and information for the PRU has changed, the serving LMF may optionally instigate an Nnrf_NFManagement_NFUpdate Request service operation towards an NRF and includes an existence indication of a PRU associated with a TAI and further includes PRU information which may include state, type, positioning capabilities, a PRU identifier (which may be local or global) and the location of the PRU. If this is a PRU Association update, the LMF may only inform the NRF about the changed PRU information (e.g. PRU location or state). If the PRU identifier already exists in the NRF for the serving LMF (from a previous Association), the NRF overwrites the old PRU information with the new PRU information. Otherwise, the NRF adds the PRU information to the information stored in the NRF for the serving LMF.

For the case that LMF only sends TAI associated PRU existence indication to the NRF, the LMF indicates the existence of PRU(s) to NRF only when the PRU Association is the first one in this TAI. The LMF also indicates to the NRF to remove the TAI associated existence of PRU(s) when there are no longer any PRUs associated in the LMF for this TAI.

10.	The NRF returns a confirmation response to the serving LMF.

11.	If the PRU receives a Routing ID of a new serving LMF at step 7b, the PRU shall peform an initial PRU Association procedure with the new serving LMF.

NOTE 3:	The PRU may be configured with a limit on the number and/or duration of unsuccessful PRU Association attempts. When this limit is reached or if a PRU Association is rejected at step 7b without a new serving LMF being indicated, the PRU may send an indication to a controlling NF or AF. The controlling NF or AF could then reconfigure the PRU with a different serving LMF or could indicate that the PRU is temporarily deactivated. Such procedures are out of scope of this specification.

### 6.X.2	LMF Initiated PRU Disassociation Procedure

Figure 6.X.2-1 shows a procedure used by a serving LMF to disassociate an already associated PRU. The procedure may be used prior to the serving LMF becoming unavailable (e.g. for maintenance, removal or replacement) or to transfer the PRU to a different serving LMF for other reasons.

Figure 6.X.2-1: LMF Initiated PRU Disassociation Procedure

Precondition:

The PRU has previously associated with the serving LMF using the procedure in clause 6.X.1.

1.	The serving LMF sends a PRU Disassociation Request as a location services supplementary services message, using the Namf_Communication_N1N2MessageTransfer service operation, and a Correlation ID identifying the LMF. The PRU Disassociation Request may include a Routing ID for a new serving LMF and a time window when the PRU is allowed to associate with the new serving LMF.

2.	If the PRU is in CM IDLE state, the serving AMF performs a Network Triggered service request to place the PRU in CM CONNECTED state.

3.	The serving AMF forwards the PRU Disassociation Request and a Routing ID equal to the Correlation ID to the PRU using DL NAS TRANSPORT message.

4.	The PRU returns a supplementary services PRU Disassociation Accept to the serving AMF in an UL NAS TRANSPORT message and includes the Routing ID received in step 3.

5.	The serving AMF forwards the PRU Disassociation Accept to the serving LMF indicated by the Routing ID received at step 4 and includes a Correlation ID equal to the Routing ID.

6.	If the serving LMF has indicated the PRU to an NRF during PRU Association, the serving LMF instigates an Nnrf_NFManagement_NFUpdate Request service operation towards the NRF and includes an indication of PRU removal and the PRU identifier if PRU information was sent to the NRF. The NRF then removes the TAI associated PRU existence indication and further remove PRU information in the NRF for this PRU for the serving LMF.

For the case that the LMF only sends TAI associated PRU existence indication to the NRF, the LMF indicates to the NRF to remove the existence of PRU(s) when all PRUs in a TAI have been disassociated.

7.	The NRF returns a confirmation response to the serving LMF.

8.	If the PRU received a Routing ID for a new serving LMF at step 3, the PRU performs an initial PRU Association with the new serving LMF as described in clause 6.X.1. If a time window was received at step 3, the PRU Association with the new serving LMF is performed during the time window.

NOTE:	If a new serving LMF is not indicated at step 3 or if the PRU is unable to successfully associate with the new serving LMF at step 8 during the time window, the PRU may send an indication to a controlling NF or AF. The controlling NF or AF could then reconfigure the PRU with a different serving LMF or could indicate that the PRU is temporarily deactivated. Such procedures are out of scope of this specification.

### 6.X.3	PRU Initiated PRU Disassociation Procedure

Figure 6.X.3-1 shows a procedure used by a PRU to disassociate from a serving LMF. The procedure may be used prior to the PRU becoming unavailable (e.g. for a SW upgrade or power down) or when the PRU will be transferred to a different serving LMF.

Figure 6.X.3-1: PRU Initiated PRU Disassociation Procedure

Precondition:

The PRU has previously associated with the serving LMF using the procedure in clause 6.X.1 and is currently registered in the HPLMN.

1.	The PRU performs a UE Triggered Service Request if in CM IDLE state.

2.	The PRU sends a location supplementary services PRU Disassociation Request to the serving AMF in an UL NAS TRANSPORT message and includes the Routing ID received at step 7a for a previous PRU Association procedure. The PRU also indicates whether an acknowledgment is expected. The PRU Disassociation Request is included in the UL NAS TRANSPORT message as a "Location services message container" at the NAS level.

NOTE 1:	A PRU could indicate whether an acknowledgment is expected according to whether the PRU expects to be still able to receive the acknowledgment at a later time.

3.	[Optionally,] The AMF may verify that the sender of the PRU Disassociation Request is a PRU using subscription information from the UDM.

4.	The AMF selects the serving LMF based on the Routing ID and optionally the current TAI and transfers the PRU Disassociation Request to the serving LMF using an Namf_Communication_N1MessageNotify service operation. If step 3 was performed, the AMF includes in the Namf_Communication_N1MessageNotify service operation an indication that the PRU subscription information was verified by the AMF. The AMF also includes the SUPI of the PRU.

5.	The serving LMF verifies the PRU if it is not verified by the AMF in step 3. This can be based on matching the SUPI for the PRU received at step 4 with the list of authorized SUPI(s) configured in the LMF. The serving LMF also verifies that the PRU is currently associated in the serving LMF. If the PRU is not currently associated in the serving LMF, the serving LMF performs steps 6 and 7 but not steps 8 and 9.

NOTE 2:	Inconsistency between Association in a PRU versus a serving LMF might arise if a PRU is powered off or loses network coverage and if the serving LMF then performs an LMF initiated PRU Disassociation.

6.	If the PRU has indicated that an acknowledgment is expected, the serving LMF returns a PRU Disassociation Accept, as a location services supplementary services message, using an Namf_Communication_N1N2MessageTransfer service operation towards the AMF, and a Correlation ID.

7.	The serving AMF forwards the PRU Disassociation Accept and a Routing ID equal to the Correlation ID to the PRU in a DL NAS TRANSPORT message.

8.	If the serving LMF has indicated the PRU to an NRF during a previous PRU Association, the serving LMF instigates an Nnrf_NFManagement_NFUpdate Request service operation towards the NRF and includes an indication of PRU removal and the PRU identifier if PRU information was sent to the NRF. The NRF then removes the TAI associated PRU existence indication and further removes PRU information in the NRF for this PRU for the serving LMF.

For the case that the LMF only sends TAI associated PRU existence indication to the NRF, the LMF indicates to the NRF to remove the existence of PRU(s) when all PRUs in a TAI have been disassociated.

9.	The NRF returns a confirmation response to the serving LMF.

### 6.X.4	Positioning of a target UE

Figure 6.X.4-1 shows a procedure used by a serving LMF for a target UE to obtain a location of the target UE using location information provided by one or more PRUs.

Figure 6.X.4-1: Location of a target UE using PRUs

1.	The serving LMF for the target UE and other PRU serving LMFs may use the procedures defined in clause 6.11 to obtain location information from one or more PRUs associated in the serving LMF and in the other PRU serving LMFs that is not related to the target UE. For example, the location information may include location information for the PRU(s) or for the NG-RAN or both.

2.	The serving LMF for the target UE receives a location request from the serving AMF for the target UE. The location request may be included in an Nlmf_Location_DetermineLocation Request service operation for a 5GC-MO-LR, 5GC-MT-LR or 5GC-NI-LR for the target UE. Alternatively, the location request may be implied by receipt of an Namf_Communication_N1MessageNotify service operation carrying a supplementary services event report from the target UE for a periodic or triggered 5GC-MT-LR.

3.	The serving LMF uses the procedures defined in clause 6.11 to obtain location information for the target UE from the target UE and/or from the NG-RAN.

4.	The serving LMF selects one or more PRUs associated with the serving LMF to assist in locating the target UE. The selected PRU(s) may be nearby to an initial location estimate for the target UE obtained at step 3 or indicated by a serving cell identifier for the target UE received at step 2. The serving LMF also considers the state of PRU(s), e.g. PRU(s) with off state should not be selected.

5.	The serving LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF. The service operation includes a PRU indication and an area which could be a list of cells or TAs decided by the serving LMF of the target UE based on the serving cell of the target UE.

6.	If step 5 is performed, the NRF selects one or more other PRU serving LMFs based on the PRU indication and the area received in step 5 and sends an Nnrf_NFDiscovery Response to the serving LMF of target UE. The service operation includes the profiles of the other PRU serving LMFs selected by the NRF. Each profile may include PRU information, e.g. a PRU identifier, type of PRU (mobile or fixed), PRU location, state of PRU (on or off).

7.	If steps 5 and 6 are performed, the serving LMF of the target UE may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs indicated at step 6. The service operation for each of the other PRU serving LMFs includes target UE cell ID or one or more PRU identifiers received at step 6 for this LMF, and an indication of location measurements requested from each PRU.

8.	The serving LMF uses the procedure defined in clause 6.11.1 to obtain location information related to the target UE from the PRU(s) selected at step 4.

Editor's note:	Location information for a target UE obtained from a PRU needs to be verified by RAN.

9.	If step 7 is performed and if PRU information is included in the PRU serving LMF profile, each of the other PRU serving LMFs for step 7 uses the procedure defined in clause 6.11.1 to obtain the location measurements requested at step 7 from each of the PRUs identified at step 7 for this LMF.

If step 7 is performed and if PRU information is not included in the PRU serving LMF profile, each of the other PRU serving LMFs for step 7 selects one or more PRUs based on the locally associated PRU information (e.g., the target UE cell ID, the PRU state (on/off)), and uses the procedure defined in clause 6.11.1 to obtain the location measurements requested at step 7 from each of the selected PRUs.

10.	If step 9 is performed, each of the other PRU serving LMFs for step 9 returns the location measurements obtained from PRUs at step 9 to the serving LMF for the target UE.

11.	The serving LMF for the target UE determines the location of the target UE based on the location information obtained at step 1 (if step 1 is performed), step 3, step 8 and step 10.

12a.	If an Nlmf_Location_DetermineLocation Request service operation for a 5GC-MO-LR, 5GC-MT-LR or 5GC-NI-LR was received at step 2, the serving LMF returns the location estimate of the target UE to the serving AMF.

12b.	If an Namf_Communication_N1MessageNotify service operation carrying a supplementary services event report from the target UE for a periodic or triggered 5GC-MT-LR was received at step 2, the serving LMF sends an event report for the target UE to a GMLC with the location estimate obtained at step 11 as described in clause 6.3.1.

**** End of Changes ****