3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300646r01

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3763 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of N3GPP access path switching on MA PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The new WID of ATSSS_Ph3 has include objectives about N3GPP path switching. In TR 23.700-53 conclusion part, the solution for N3GPP path switching on MA PDU session has been consolidated. Therefore, this CR aims to update TS 23.502 to capture the consolidated solution for N3GPP path switching on MA PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new clause to describe the procedure of N3GPP path swiching on MA PDU session. The procedure includes the capability negotiation between UE and network during Registration procedure, using Mobility Registration Update procedure to perform path switching and two different methods of path switching (i.e. with AN resource releasing or without AN resource releasing). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not capture the N3GPP path switching on MA PDU session included in the WID of ATSSS_Ph3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.9.1, 4.22.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.22.9.1	Registration procedures

The signalling flow for a Registration is based on the signalling flow in Figure 4.2.2.2.2-1 with the following differences and clarifications:

-	In step 1, if the UE wants to re-activate the user plane of the MA PDU Session(s) over the access the Registration message is sent to, the UE indicates PDU Session ID(s) of the MA PDU Session(s) in the List Of PDU Sessions To Be Activated.

If the UE locally releases the MA PDU Session(s) in both accesses, the UE indicates it in the PDU Session Status. If the AMF receives the PDU Session Status and finds mismatch, regardless of roaming mode of the MA PDU Session(s) (i.e. non-roaming, local breakout roaming, home routed roaming in the same PLMN or home routed roaming in different PLMNs), the AMF invokes Nsmf_PDUSession_ReleaseSMContext service towards the SMF(s) in order to release any network resources related to the MA PDU Session(s).

-	In step 4, the old AMF determines whether the new AMF support ATSSS or not based on the supported features provided by the new AMF.

-	In step 5, if the old AMF determined in step 4 that the new AMF does not support ATSSS, the old AMF does not include the PDU Session context of the MA PDU Session(s) in the UE context transferred to the new AMF.

-	If the old AMF has not included MA PDU Session(s) in the UE context in step 5, the old AMF informs the corresponding SMF(s) to release the MA PDU Session(s) by invoking the Nsmf_PDUSession_ReleaseSMContext service operation as described in clause 4.22.10.

-	In step 21, the AMF provides an MA PDU Session Support indicator in Registration Accept message to inform the UE whether ATSSS is supported or not. The UE uses this indicator to determine whether an MA PDU session related procedure can be initiated or not, as described in clause 4.22.1.

In step 17, if the UE indicated to re-activate MA PDU Session(s) in the List Of PDU Sessions To Be Activated the AMF includes access type which the Registration Request message is received on when the AMF triggers Nsmf_PDUSession_UpdateSMContext service operation. The SMF only re-activates user plane resources of the access type the Registration Request message is received on.

-	In step 21, the AMF indicates to the UE whether it supports non-3GPP access path switching. The UE uses this indication to determine whether non-3GPP access path switching procedure can be initiated or not, as described in clause 4.22.9.X.

-	In step 22, if the AMF indicates that the PDU Session(s) has been released in the PDU Session Status to the UE in Registration Accept message, the UE removes locally any internal resources related to the MA PDU Session(s) that are not marked as established.

# * * * * Second change * * * *

4.22.9.X	UE Triggered Non-3GPP Access Path Switching

The path switching between non-3GPP accesses is performed during the Registration procedure as described in Figure 4.22.9.X-1. There are two options for non-3GPP path switching as follow:

-	Non-3GPP path switching without releasing AN resources over old non-3GPP access during switching procedure. In this option, user plane resources of old non-3GPP access is kept until user plane resources are established over the new non-3GPP access.

-	Non-3GPP path switching with releasing AN resources over old non-3GPP access during switching procedure. In this option, user plane resources of the old non-3GPP access is released first and then user plane resources are established over the new non-3GPP access.

-	The UE does not perform non-3GPP path switching if the PLMN of the selected target non-3GPP AN is different from the PLMN of the source non-3GPP AN.

Figure 4.22.9.X-1. Mobility Registration procedure to support non-3GPP access switching

Figure 4.22.9.X-1 takes switching from untrusted non-3GPP access path to trusted non-3GPP access path as example.

1.	UE registers over untrusted non-3GPP access and optionally registers over 3GPP access as specified in clause 4.22.9.1 and establishes MA PDU sessions as specified in clause 4.22.2.1 with following addition:

-	In step 1, the UE indicates to AMF whether it supports non-3GPP access path switching.

-	In step 2, if the AMF supports non-3GPP access path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF selects a SMF that supports non-3GPP access path switching.

-	In step 3, if the AMF supports non-3GPP access path switching and the UE indicated in step 1 that the UE supports non-3GPP access path switching, the AMF indicates to the SMF that the UE supports non-3GPP access path switching.

-	In step 11, if the AMF indicated in step 3 that non-3GPP access path switching is supported, and the SMF supports non-3GPP access path switching, the SMF indicates support of non-3GPP access path switching in the PDU Session Establishment Accept message.

2.	When UE wants to perform non-3GPP access path switching (e.g. switching the untrusted non-3GPP access path to trusted non-3GPP access path), UE performs Registration procedure over trusted non-	3GPP access path with the Registration type value set to "Mobility Registration Update" and optionally a Non-3GPP switching indication. Additionally, UE also requests user-plane establishment using the "PDU Sessions to be activated" parameter.

If the non-3GPP switching indication is included in the Registration Request, steps 4 is omitted.

If the non-3GPP switching indication is not included in the Registration Request, steps 9 is omitted.

3.	Authentication or security procedure may be performed over trusted non-3GPP access.

NOTE:	Detials will be specified by SA WG3.

4.	AMF triggers the procedure of releasing AN resources over untrusted non-3GPP access.

5.	AMF sends Nsmf_PDUSession_UpdateSMContext Request to SMF associated with the PDU Sessions in order to perform non-3GPP path switching. If the AMF receives Registration Request message with "Non-3GPP path switching" indication, and the AMF supports to maintain the old non-3GPP N2 connection while establishing the new non-3GPP N2 connection, the AMF does not trigger AN release procedure over old non-3GPP access (i.e. step 4 is skipped) and includes "Non-3GPP path switching" indication in Nsmf_PDUSession_UpdateSMContext request. If the SMF receives this indication, the SMF does not release the existing user-plane resources of old non-3GPP access. SMF performs N4 update and sends Nsmf_PDUSession_UpdateSMContext Response to AMF including N2 information which includes the CN tunnel information.

Editor’s note:	whether a new CN tunnel information is allocated or reusing the CN tunnel information over old non-3GPP access path is FFS.

6.	The AMF sends an N2 message to TNGF and includes the CN tunnel information received from the SMF. The N2 message is an Initial Context Setup Request (if not sent in step 3) or a PDU Session Resource Setup request.

The TNGF completes the IPSec SA establishment as described in clause 4.12a.2.2 of TS 23.502 [3] and triggers establishment of UP resources towards the UE, e.g. to create a first Child SA for the PDU Session.

After this step, the UE may start to send UL traffic via the new non-3GPP access.

7.	The TNGF sends Initial Context Setup Response to the AMF including AN tunnel information.

8.	The AMF forwards the information to the SMF and the SMF provides the AN tunnel information to the UPF(s) indicating that the following DL data over non-3GPP access will be sent via trusted non-3GPP access. For MA PDU Sessions, the UPF starts to use the trusted non-3GPP access as the non-3GPP access for the MA PDU Session, i.e. untrusted non-3GPP access is not used for downlink traffic anymore. For SA PDU Sessions, the downlink tunnel is switched to the trusted non-3GPP access. The UPF still accepts uplink packets from the untrusted non-3GPP access.

9.	If step 4 is skipped, the AMF triggers an AN release procedure of the untrusted non-3GPP access.

10.	The AMF sends a NAS Registration Accept message to the UE, via the TNGF.

11.	The AMF may update the registration for non-3GPP access in UDM, by providing the new RAT type.

12.	The UE may send a NAS Registration Complete message, as described in TS 23.502 [3].

# * * * * End of changes * * * *