3GPP TSG-SA WG2 Meeting #154-AH-E	S2-2300647r02

16 - 20 January 2023, Elbonia	(revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3956 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer of Satellite Coverage Data to a UE and AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions for FS_5GSAT_Ph2 in TR 23.700-28, include supporting the the transfer of satellite coverage information to a UE and AMF. This CR introduces the support as proposed in a comparison of alternative solutions in S2-2300668. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for transfer of coverage data to a UE and to an AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transfer of enhanced satellite coverage data to a UE compared to support for EPS in Release 17 and to an AMF would not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X.Y (new), 5.4.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change (all new) ****

5.4.X Support of discontinuous network coverage

#### 5.4.X.Y UE coverage information provisioning to the UE

A UE may use satellite coverage information for satellite access to support discontinuous coverage operations. In addition to satellite assistance information provided by E-UTRA, coverage information can be provided to a UE by an external server or AF by user plane means using protocols and procedures defined in TS 24.xxx. In the case of an external server, the UE obtains the server address and an authorization token from the serving AMF using NAS and then sends a request containing the authorization token to the external server using a PDU session or SMS to request and receive coverage information. In the case of an AF, the UE uses NIDD as described in clause 5.31.5 to request and receive coverage data from an AF associated with the HPLMN.

Editor’s Note:	It needs to be determined whether both external server and AF provision will be supported or just one of these and whether, in the case of an AF, NIDD will be extended to support provision of coverage data from an AF associated with a VPLMN for a roaming UE.

Editor's Note:	Whether RAN agrees to provide satellite assistance information over NR is FFS.

Editor's Note:	It is expected that CT1 will define the protocol and procedures for providing coverage information over user plane. TS 24.xxx will be replaced later by the appropriate CT1 TS number.

#### 5.4.X.Z UE Coverage information provisioning to the AMF

The AMF may use satellite coverage information to support satellite access by UEs with discontinuous coverage operation. Coverage information may be obtained by the AMF using O&M, from an AF or from an external server

Editor’s Note: 	It needs to be determined whether all three sources of coverage data will be supported or just one or two of these and the procedures to be used for an AF and external server.

**** End of Changes ****