3GPP TSG-SA WG2 Meeting #154AH-e 		S2-2300656r03

Elbonia, Jan 16 – 20, 2023	(revison of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0168 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RRC Inactive state reception for MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Per TR conclusion of KI#1, to support the UE receive the MBS session data in RRC Inactive state, 5GC provides the MBS session assistance information to NG-RAN node. The assistance information includes two parts: - MBS session level assistance information: existing MBS session QoS parameters (e.g. ARP, 5QI) can be used. - UE level MBS assistance information. Three option are suggested, i.e. flag, multiple choices, multiple integer values for the assistance information. For UE level assistance information, the intention is to give NG-RAN UE level additional information to let NG-RAN be aware which UE is preferred to be kept in RRC Connected state. Three candidate options are suggested: - Option 1: Using a flag to indicate that the UE is preferred to be in RRC connected state, i.e., Y/N value. From the AF view, it is easy for AF to differentiate which UE is preferred to be in RRC connected state. From NG-RAN view it is also a clear indication to let NG-RAN do the decision. - Option 2: Using the high/low/medium value to indicate which group member is preferred to be in RRC connected state. Comparing to the flag mechanism, it is ambiguity of the meaning of medium value. From the NG-RAN node view, it is also unclear how to handle the group member with the medium value setting. It seems give some flexibility to the NG-RAN setting, e.g. per different network condition. But it can not avoid that different NG-RAN node handling can be different per different algorithm. So it this what the expectation from AF? - Option 3: Using the expected traffic pattern, such as, provide some IEs of expected UE behaviour (e.g., the Communication Duration Time and Periodic Time) to NG-RAN. For this it is assume similar like the existing traffic pattern parameter is given to the NG-RAN. To support that, the AF need have a clear view on each 5MBS service traffic pattern. However it is unclear how the AF can derive the related 5MBS service quickly, considering the related 5MBS service can be dynamical, e.g. different communication service for Public Communication service group. Also the intention of the UE level parameter is only for the usage of suggestion to keep the UE in RRC connected state, it is not related to any other parameter setting. It seems other additional detail parameter is not required. Per above consideration, it is suggested to adopt option 1, i.e. using a flag. For the MBS session activation and mobility, it is also suggested to add the related TR conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions for multicast MBS data reception in RRC Inactive state in TS 23.247. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the features of multicast MBS data reception in RRC Inactive state, which has been concluded in TR 23.700-47. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 6.X	Support of Multicast MBS session data reception in RRC_INACTIVE state

### 6.X.1	General

A multicast MBS session data, is possible to be configured to be received by some UE(s) in RRC_CONNECTED and other UE(s) in RRC:INACTIVE state

NG-RAN nodes determine (e.g. at congestion time) whether to apply delivery enabling reception by UEs in RRC_INACTIVE state for an MBS session and which UE(s) are moved from CM-CONNECTED with RRC_CONNECTED to CM-CONNECTED with RRC_INACTIVE state and still receive MBS session data. The NG-RAN nodes further take other on-going services into account, e.g., the data over the PDU sessions or other MBS sessions to determine whether a certain UE can be switch to RRC Inactive state.

NOTE 1:	 How NG-RAN nodes perform those decisions is up to NG-RAN implementation.

Based on input from the AF, configuration and/or policies the 5GC provides information about the MBS session that assists NG-RAN in those decisions:

-	The MBS session QoS parameters, e.g. the most demanding ARP and 5QI

-	UE level MBS assistance information: The UE level MBS assistance information is an optional parameter and is set per MBS session. It indicates whether a UE is preferred to be kept in the RRC Connected state (as the UE is expected to send frequent uplink data) while the MBS session is active .

Editor's Note:	The UE level MBS assistance information protocol details may be updated based on RAN WG feedback.

NOTE 2:	The "RRC Inactive Assistance Information" in clause 5.3.3.2.5 of TS 23.501 [5] is sent by AMF to NG-RAN and may be used by NG-RAN together with MBS session QoS parameters and UE level MBS assistance information for deciding whether to send a UE to RRC Inactive state.

The AF determines the UE level MBS assistance information, e.g., based on the expected traffic pattern of the UE (e.g., frequent talker) and provides it to 5GC via the External Parameter Provisioning procedures as specified in clause 4.15.6.2 of TS 23.502 [6].

The SMF obtains the UE level assistance information from the UDM as part of the MBS subscription data, and provides to NG-RAN node as part of the PDU session information in an N2 SM Information Container.

### 6.X.2	Multicast MBS Session Activation

When an MBS session is to be activated and the UE(s) that joined the MBS session are in RRC_INACTIVE Inactive state, the 5GC notifies the related NG-RAN nodes of this event as specified in Clause 7.2.5.1. UEs in RRC Inactive state in cells where the MBS session is delivered allowing RRC-inactive reception should remain in RRC Inactive state during the MBS session activation while starting to receive the MBS session data.

Editor's Note:	How NG-RAN notifies the UE that the MBS session is re-activated and whether the MBS session is allowed to be received in RRC-inactive state will be decided by RAN WGs.

If the UE level MBS assistance information is available at the SMF, the SMF provides it to NG-RAN node it in response to the PDU session update request during the MBS session activation procedure.

### 6.X.3	Mobility for UE supporting RRC Inactive MBS data reception

When an RRC Inactive UE receives ongoing MBS session data and moves to a new cell within the RNA and if the multicast session is available, the UE continues receiving the MBS session data.

Editor´s Note: NG-RAN nodes procedures to send MBS data in the new cell within the RNA where the multicast session is unavailable will be decided by RAN WGs.

When an RRC Inactive UE moves outside the RNA but within the same RA, the UE performs the "UE Triggered Connection Resume in RRC Inactive state procedure" to the NG-RAN node. When an RRC Inactive UE moves outside the RA, the UE performs mobility registration procedures and resume the RRC Connection.If the RRC Connection resume fails, the UE transition to CM-IDLEand may then initiate the mobility registration update or Service Request procedure and activate the associated PDU session to enable reception of MBS data. Hence the shared tunnel (if not already established) or the individual delivery can be established towards the NG-RAN node to enable reception of MBS data.

When an RRC Inactive UE moves outside the RA, the UE performs mobility registration procedures and resume the RRC Connection.

The SMF provides the UE level MBS assistance information in N2 SM information to target NG-RAN node, if it is available at SMF and UE moves to that target NG-RAN node via Xn or N2 based handover.

# * * * * End of changes * * * *