3GPP TSG-SA WG2 Meeting #154AH-e 		S2-2300656r06

Elbonia, Jan 16 – 20, 2023	(revison of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0168 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RRC Inactive state reception for MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Per TR conclusion of KI#1, to support the UE receive the MBS session data in RRC Inactive state, 5GC provides the MBS session assistance information to NG-RAN node. The assistance information includes two parts: - MBS session level assistance information: existing MBS session QoS parameters (e.g. ARP, 5QI) can be used. - UE level MBS assistance information. Three option are suggested, i.e. flag, multiple choices, multiple integer values for the assistance information. For UE level assistance information, the intention is to give NG-RAN UE level additional information to let NG-RAN be aware which UE is preferred to be kept in RRC Connected state. Three candidate options are suggested: - Option 1: Using a flag to indicate that the UE is preferred to be in RRC connected state, i.e., Y/N value. From the AF view, it is easy for AF to differentiate which UE is preferred to be in RRC connected state. From NG-RAN view it is also a clear indication to let NG-RAN do the decision. - Option 2: Using the high/low/medium value to indicate which group member is preferred to be in RRC connected state. Comparing to the flag mechanism, it is ambiguity of the meaning of medium value. From the NG-RAN node view, it is also unclear how to handle the group member with the medium value setting. It seems give some flexibility to the NG-RAN setting, e.g. per different network condition. But it can not avoid that different NG-RAN node handling can be different per different algorithm. So it this what the expectation from AF? - Option 3: Using the expected traffic pattern, such as, provide some IEs of expected UE behaviour (e.g., the Communication Duration Time and Periodic Time) to NG-RAN. For this it is assume similar like the existing traffic pattern parameter is given to the NG-RAN. To support that, the AF need have a clear view on each 5MBS service traffic pattern. However it is unclear how the AF can derive the related 5MBS service quickly, considering the related 5MBS service can be dynamical, e.g. different communication service for Public Communication service group. Also the intention of the UE level parameter is only for the usage of suggestion to keep the UE in RRC connected state, it is not related to any other parameter setting. It seems other additional detail parameter is not required. Per above consideration, it is suggested to adopt option 1, i.e. using a flag. For the MBS session activation and mobility, it is also suggested to add the related TR conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions for multicast MBS data reception in RRC Inactive state in TS 23.247. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the features of multicast MBS data reception in RRC Inactive state, which has been concluded in TR 23.700-47. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause 7.2.9a is specified in the CR#159 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 6.X	Support of Multicast MBS session data reception i RRC_INACTIVE state

To provide multicast MBS service to more UEs in a cell, the NG-RAN may decide to move some UE(s) receiving multicast MBS data from m RRC_CONNECTED to RRC_INACTIVE state if the UE(s) is capable to receiving MBS data in RRC_INACTIVE state

.

NOTE 1:	 How NG-RAN nodes perform those decisions is up to NG-RAN implementation.

The decision in NG-RAN may be based on the following assistance information:

-	The MBS session QoS parameters

-	UE level MBS assistance information

Editor's Note:	The UE level MBS assistance information protocol details require  RAN WG feedback.

Editor's Note:	Whether the UE level MBS assistance information is the final naming is FFS.

NOTE 1:	 How the NG-RAN nodes perform those decisions is up to NG-RAN implementation.

NOTE 2:	If the "RRC Inactive Assistance Information" in clause 5.3.3.2.5 of TS 23.501 [5] is sent by AMF to NG-RAN, whether and how it is used by NG-RAN for deciding whether to send a UE to RRC_INACTIVE state is to be decided by RAN WG(s).

The MBS session QoS parameters (e.g. ARP and 5QI) are provided to the NG-RAN by the MB-SMF during user plane establishment for shared delivery.

The UE level MBS assistance information is provided to NG-RAN by the SMF if the UE level MBS assistance information is available in the SMF. The SMF gets from the UDM the UE level MBS assistance information which is provisioned by the AF via the NEF to the UDM as part of the MBS subscription data (as specified in clause 6.4). The SMF provides the UE level MBS assistance information to the NG-RAN in procedures such as PDU Session modification and handover.

When an RRC_INACTIVE UE is receiving ongoing MBS session data, if the UE moves to a new cell within the RNA, or if the UE moves outside the current RNA but within the current Registration Area, or if the UE moves out of the current Registration Area, the UE shall be able to receive the MBS session data if applicable in the new area.

NOTE: The scenario of UE moving to a new cell within the RAN Notification Area is specified in RAN specifications.

# * * * * End of changes * * * *