3GPP TSG-SA2 Meeting #154AH-e	S2-2300663R02

# Elbonia, January 16 – 20, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3962 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Direct Exposure from UPF and UPF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For the UPF data exposure it is described in clause 5.8.2.17. As this is related to event exposure service, other event exposure service is described in section 5.20. Also the section 5.8.2 may be interpreted as only related to the interaction between SMF and UPF even this section is related to all UPF function. To make it more explicit, it is suggested to add a new clause 5.20b on this UPF event exposure function and refer to the clause 5.8.2.17. - For the UPF selection it should be in clause 6.3.3.1. However it is implemented at the clause 6.3.3.2 at TS23.502 v18.0.0. It is suggested to move it back to clause 6.3.3.1 as it is not only related to provision suitable UPF to SMF. Also to dfifferentiate this UPF selection is for data collection and different comparing to the UPF selection for the PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause 5.20b. - Move the UPF discovery and selection for data collection back to clause 6.3.3.1. And add some clarification on the existing UPF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UPF direct event exposure may be neglected. - Unsuitable to restrict the UPF discovery and selction for data collection only for SMF provision. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20.b(new), 6.3.3.1, 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Management |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | Whether we need move the whole 5.8.2.17 clause to 5.20.b? |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.20b Direct Exposure of Network Infomation from UPF

It is possible to directly expose the UPF information to NF consumer via the service based interface as described in clause 5.8.2.17.

# * * * * Next change * * * *

#### 6.3.3.1	Overview

The selection and reselection of the UPF for PDU session establishment are performed by the SMF by considering UPF deployment scenarios such as centrally located UPF and distributed UPF located close to or at the Access Network site. The selection of the UPF shall also enable deployment of UPF with different capabilities, e.g. UPFs supporting no or a subset of optional functionalities.

The UPF selection for PDU session establishment in home routed roaming case, the UPF(s) in home PLMN is selected by SMF(s) in HPLMN, and the UPF(s) in the VPLMN is selected by SMF(s) in VPLMN. The exact set of parameters used for the selection mechanism is deployment specific and controlled by the operator configuration.

The UPF selection for PDU session establishment involves:

-	a step of SMF Provisioning of available UPF(s). This step may take place while there is no PDU Session to establish and may be followed by N4 Node Level procedures defined in clause 4.4.3 of TS 23.502 [3] where the UPF and the SMF may exchange information such as the support of optional functionalities and capabilities.

-	A step of selection of an UPF for a particular PDU Session; it is followed by N4 session management procedures defined in clause 4.4.1 of TS 23.502 [3].

To collect the data from the UPF as defined in clause 5.8.2.17, the related dedicated UPF is discovered and selected as following:

-	When the NF consumer or SCP directly subscribes to the UPF, the NF consumer or SCP queries the NRF including the related discovery parameter. The NRF returns the UPF(s) which meet(s) the discovery request.

-	When the NF consumer or SCP shall subscribe via the SMF, the NF consumer gets the serving SMF information from the UDM per SUPI, DNN and S-NSSAI. After that, the NF consumer sends a subscription to the indicated SMF and the SMF identifies the related UPF(s) using the parameters of the subscription (e.g. target flow description, AoI, etc.) and transfers the related event subscription information to the identified UPF(s). If the NF consumer does not know the SUPI but only the UE IP address, it may need to invoke the BSF to get the SUPI corresponding to the triplet (IP address, DNN and S-NSSAI).

#### 6.3.3.2	SMF Provisioning of available UPF(s)

SMF may be locally configured with the information about the available UPFs, e.g. by OA&M system when UPF is instantiated or removed.

NOTE 1:	UPF information can be updated e.g. by OA&M system any time after the initial provisioning, or UPF itself updates its information to the SMF any time after the node level interaction is established.

The UPF selection functionality in the SMF may optionally utilize the NRF to discover UPF instance(s). In this case, the SMF issues a request to the NRF that may include following parameters: DNN, S-NSSAI, SMF Area Identity, ATSSS steering capabilities. In its answer, the NRF provides the NF profile(s) that include(s) the IP address(es) or the FQDN of the N4 interface of corresponding UPF instance(s) to the SMF.

UPFs may be associated with an SMF Area Identity in the NRF. This allows limiting the SMF provisioning of UPF(s) using NRF to those UPF(s) associated with a certain SMF Area Identity. This can e.g. be used in the case that an SMF is only allowed to control UPF(s) configured in NRF as belonging to a certain SMF Area Identity.

The NRF may be configured by OAM with information on the available UPF(s) or the UPF instance(s) may register its/their NF profile(s) in the NRF. This is further defined in clause 4.17 of TS 23.502 [3].

To collect the data from the UPF as defined in clause 5.8.2.17, the related UPF is discovered and selected as following:

-	When the NF consumer may directly subscribe to the UPF, the NF consumer queries the NRF including the related discovery parameter. The NRF returns the UPF(s) which meet(s) the discovery request.

-	When the NF consumer shall subscribe via the SMF, the NF consumer gets the serving SMF information from the UDM per SUPI, or IP address, DNN and S-NSSAI. After that, the NF consumer sends a subscription to the indicated SMF and the SMF identities the related UPF(s) using the parameters of the subscription (e.g. target flow description, AoI, etc.) and transfers the related event subscription information to the identified UPF(s).

# * * * * End of change * * * *