SA WG2 Meeting #S2-154AH-e	S2-2300680

16 - 20 January 2023, Electronic, Elbonia	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3964 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3, NEF exposure for handling PDU Session Type change and managing temporal invalidity/validity condition for a group of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion for the KI#3 in the 23.700-074: - The TSCTSF or PCF (in non-TSCTSF case) is responsible to manage the temporal invalidity/validity condition (start-time, end-time). - PDU Session Release COMMAND for re-establishment of the PDU Session and URSP rules for the highest priority PDU Session Type of a group are used to change PDU Session Type of the PDU Session targeting the group for each group member within the group. To release the PDU sssion |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clasue 5.29a.X for “provisioning of Group traffic characteristics and monitoring of performance characteristics”. Adding one sub-clasues on the AF-requested PDU session type change . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for AF-requested PDU session type change is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29a.X.A(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.29a.X	provisioning of Group traffic characteristics and monitoring of performance characteristics

5.29a.X.A	Change of PDU Session Type for a group UEs

The Service specific parameters provisioning procedure as defined in clause 4.15.6.7 and 4.15.6.7a of TS 23.502 [3] are applicable for provisioning of PDU session type used for a group of UEs with the following enhancements:

-	The Service specific parameter may containe the requested PDU session type for the group of UEs.

-	The NEF sends the requested PDU session type to UDM for authorization.

-	The AF request information stored in the UDR ("Application Data", Data Subset setting to "Service specific information") contains the request PDU session type.

-	The PCF adjusts the PDU session type precedence in the URSP and send to UE according to the request PDU session type.

When the UE receives the URSP, the UE evaluates the URSP and releases the PDU session and re-establishes the PDU session with the high precedence PDU session type in the URSP.

Next CHANGE

END OF CHANGES