SA WG2 Meeting #S2-154AH-e	S2-2300682r01

16 - 20 January 2023, Electronic, Elbonia	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0171 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1, adding RRC inactive network functionality of NF description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Accoring to the the KI#1 conclusion of TR 23.700-047, the RRC_Inactive reception for multoicast MBS session is adopt for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes to add the NF functionality for the RRC inactive reception for the multicast MBS session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion for the FS_5MBS_Ph2 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.3, 5.3.2.7, 5.3.2.8, 5.3.2.9, 5.3.2.10, 5.3.2.3, 5.3.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.3.2.3	SMF

In addition to the functions defined in TS 23.501 [5], the SMF performs the following functions to support MBS:

-	Discovering MB-SMF for a Multicast MBS session.

-	Authorizing Multicast MBS session join operation for served UEs if needed.

-	Interacting with MB-SMF to obtain multicast Session Context information used as input to modify the PDU Session associated with MBS session.

-	Interacting with RAN to provide information about a Multicast MBS session that a UE is participating in.

-	Interacting with RAN and UPF for multicast data transport using 5GC Individual MBS traffic delivery method.

-	Interacting with RAN to provide UE level MBS Assitance information.

NOTE:	SMF and MB-SMF may be co-located or deployed separately.

Next CHANGE

#### 5.3.2.7	NG-RAN

In addition to the functions defined in TS 23.501 [5], the NG-RAN performs the following functions to support MBS:

-	Management of MBS QoS flows via N2.

-	Delivery of MBS data packets for multiple UEs over radio using PTM or PTP.

-	Configuration of UE for MBS QoS flow reception at AS layer.

-	Control switching between PTM and PTP delivery per UE.

-	Support for multicast session service continuity during Xn Handover and N2 Handover.

-	Support group paging at multicast session activation over radio toward UEs in CM-IDLE state and CM-CONNECTED with RRC INACTIVE state.

-	Reception of MBS data packets from 5GC via shared MBS traffic delivery.

-	Determine to move UE(s) receiving multicast MBS data from RRC_CONNECTED state tp CM-CONNECTED with RRC INACTIVE state if the UE(s) are capable of receiving MBS data in RRC_INACTIVE state.

Next CHANGE

#### 5.3.2.8	UE

In addition to the functions defined in TS 23.501 [5], the UE may perform the following functions to support MBS:

-	Reception of multicast data using PTM/PTP.

-	Reception of broadcast data using PTM.

-	Handling of incoming MBS QoS flows.

-	Support of signalling for joining and leaving a Multicast MBS session.

-	MBS resource management support at AS layer.

-	Reception of notification in CM-IDLE state and CM-CONNECTED with RRC _INACTIVE state for multicast data transmission.

-	Support reception of multicast MBS data in RRC_INACTIVE state.

NOTE :	UE functionality for MBS security is provided in TS 33.501 [20].

Next CHANGE

#### 5.3.2.9	AF

The AF performs the following functions to support MBS:

-	Requesting multicast or broadcast service from the 5GC by providing service information including QoS requirement to 5GC.

-	Instructing MBS session operation towards 5GC if needed.

-	Interacting with NEF for MBS related service exposure.

-	Provisioning the optional UE level MBS assistance information, which includes parameter of the expected behavior for a UE involved in an MBS Session .

Next CHANGE

#### 5.3.2.10	NEF

In addition to the functions defined in TS 23.501 [5], the NEF performs the following functions to support MBS:

-	Providing an interface to AFs for MBS procedures including service provisioning, MBS session and QoS management.

-	Interacting with AF and NFs in 5GC, e.g. MB-SMF for MBS session operations, determination of transport parameters.

-	Selection of MB-SMF to serve an MBS Session.

-	Support of provisioning the optional UE level MBS assistance information, which includes parameter of the expected behavior for a UE involved in an MBS Session

Next CHANGE

#### 5.3.2.13	UDM

In addition to the functions defined in TS 23.501 [5], the UDM performs the following functions to support MBS:

-	Support management of subscription for authorization for multicast MBS sessions.

-	Support management of subscription UE level MBS assistance information, which includes parameter of the expected behavior for a UE involved in an MBS Session.

#### 5.3.2.14	UDR

In addition to the functions defined in TS 23.501 [5], the UDR performs the following functions to support MBS if deployed:

-	Support storage of retrieval of MBS subscription data by the UDM for UE authorization information for multicast MBS sessions and UE level MBS assistance information.

-	Support storage and retrieval of policy data by the PCF for Multicast or Broadcast MBS sessions.

END OF CHANGES