SA WG2 Meeting #S2-154AH-e	S2-2300687r03

16 - 20 January 2023, Electronic, Elbonia	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0176 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2, adding MOCN network sharing for the same content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Accoring to the the KI#1 conclusion of TR 23.700-047, the MOCN network sharing for the same content for Broadcast MBS session is adopt for the normative work. This CR focus on high level function description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause on MOCN sharing optimization desciption |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion for the FS_5MBS_Ph2 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS23.247 CR 0160, CR165 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The referenced clause 6.5.X (for Associated Session ID) is introduced in TS 23.247 CR 0160 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.X	Shared broadcast services during Network Sharing

For network sharing as specified in clause 5.18 of TS 23.501 [2],

multiple PLMNs participating in the network sharing may deliver the same MBS broadcast service, and the shared NG-RAN nodes may broadcast the MBS data of the shared service only a single time for resource efficiency. The AF may create multiple broadcast MBS sessions via multiple CNs to deliver the same content. The NG-RAN allocates radio resource for one of broadcast MBS Sessions instead of allocating radio resource for all the broadcast MBS Sessions.

The NG-RAN may identify MBS sessions deliverying the same content in one of the following ways:

-	The AF may provide Associated Session ID (see clause 6.5.X) to the NG-RAN via 5GCs when creating a  broadcast MBS sessions.

-

The association of TMGIs may be configured in NG-RAN, for instance via configuring a range of TMGIs for shared MBS services for each PLMN participating in RAN sharing, and associating TMGIs with the same position in the ranges, so that the shared NG-RAN nodes can determine that the multiple broadcast MBS sessions are transmitting the same content for the same shared MBS service.

NOTE 1: When the association of MBS session identifiers is configured in NG-RAN, there is no requirement on the AF to provide an Associated Session ID.

Editor´s notes: How to support shared broadcast services for location-dependent MBS sessions is FFS.

Illustrated in Figure 6.X-1 is an example that the AF creates broadcast MBS Sessions via 5GC Operators A, B and C respectively to deliver the same content and N3mb unicast transport is used from 5GC to the NG-RAN. Based on operator policy in the NG-RAN node, the N3mb tunnel may be established from the 5GC of only one operator (i.e., Operator A in Figure 6.X-1) to the shared NG-RAN, or the N3mb tunnels may be established from the 5GCs of all the operators to the shared NG-RAN. Over the Uu interface, the NG-RAN allocates radio resource for only one of the established broadcast MBS Sessions regardless of the number of N3mb tunnels established to deliver the MBS packets.

Figure 6.X-1: Example of Shared broadcast services induring Network Sharing

END OF CHANGES