3GPP TSG SA WG2 Meeting #154ad	S2-2300693r01

# Elbania, eMeeting, Jan 16 - 20, 2023											(revision of S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3966 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Paging enhancement during satellite discontinuous coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR23.700-28 has conclusion about paging enhancement of UE considering satellite discontinuous coverage: - AMF/MME takes the Tracking Area reported by RAN before AN release as the paging area. Therefore, it is proposed to add such enhancement in TS23.501. To enable this, it is necessary to let AMF notify NG-RAN to send UE location before AN release. - Based on the satellite unreachability period (discontinuous coverage), AMF will request UE location from RAN. - When there is paging requirement, AMF will utilize the UE location that reported by RAN as paging area first. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case of NR satellite access, AMF will page the UE firstly with the UE location as received before AN release. AMF requests RAN to report UE location based on unreachability period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification based on Rel-18 5GSAT_Ph2 conclusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3, 5.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change  * * * *

### 5.4.x.y	Paging optimization

In case of UE is not aware of satellite discontinuous coverage information and is in CM-CONNECTED state, the AMF may request UE location based on the UE unreachability period as received in clause 5.4.x.3 to enhance the paging strategy handling as specified in clause 5.4.3, i.e. when paging the UE that is using discontinuous satellite access, the AMF may use location information provided by NG-RAN when the AN resource is released as the location information. The location request may be sent either separately via NG-RAN location reporting procedures reporting as specified in clause 4.10 of TS 23.502 [3], or AMF-initiated AN release as specified in clause 4.2.6 of TS 23.502 [3].

.

# * * * * Second change  * * * *

### 5.4.7	NG-RAN location reporting

NG-RAN supports the NG-RAN location reporting for the services that require accurate cell identification (e.g. emergency services, lawful intercept, charging) or for the UE mobility event notification service subscribed to the AMF by other NFs. The NG-RAN location reporting may be used by the AMF when the target UE is in CM-CONNECTED state. The NG-RAN location reporting may be used by the AMF to determine the geographically located TAI in the case of NR satellite access.

The AMF may request the NG-RAN location reporting with event reporting type (e.g. UE location or UE presence in Area of Interest), reporting mode and its related parameters (e.g. number of reporting).

If the AMF requests UE location, the NG-RAN reports the current UE location (or last known UE location with time stamp if the UE is in RRC Inactive state) based on the requested reporting parameter (e.g. one-time reporting or continuous reporting).

If the AMF requests UE location, in the case of NR satellite access, the NG-RAN provides all broadcast TAIs to the AMF as part of the ULI. The NG-RAN also reports the TAI where the UE is geographically located if this TAI can be determined.

If the AMF requests UE presence in the Area Of Interest, the NG-RAN reports the UE location and the indication (i.e. IN, OUT or UNKNOWN) when the NG-RAN determines the change of UE presence in Area Of Interest.

After N2 based Handover, if the NG-RAN location reporting information is required, the AMF shall re-request the NG-RAN location reporting to the target NG-RAN node. For Xn based Handover, the source NG-RAN shall transfer the requested NG-RAN location reporting information to target NG-RAN node.

The AMF requests the location information of the UE either through independent N2 procedure (i.e. NG-RAN location reporting as specified in clause 4.10 of TS 23.502 [3]), or by including the request in some specific N2 messages as specified in TS 38.413 [34]. .

The AMF receives location information of the UE either through independent N2 procedure (i.e. NG-RAN location reporting as specified in clause 4.10 of TS 23.502 [3]), or by including the response in N2 UE Context Release Complete message as specified in TS 38.413 [34].

# * * * * End of change  * * * *